3GPP TSG-RAN WG4 Meeting #94-e	R4-2000987

# Electronic Meeting, 24 Feb. – 6 Mar., 2020

Source:	OPPO

Title:	On RLM for NR-U

Agenda Item:	8.1.4.9

Document for:	Discussion and Decision

Introduction

In last meeting, WF on RRM requirements for NR-U [1] was agreed in RAN4, which summarized the new RRM requirements agreements for NR-U especially for RLM.

For RLM: in-sync

- FFS whether and how to take into account COT and LBT failures in the RLM requirements

- Lin,max

  - Option 1:

    - Lin,max = [7] for Max(TDRX,TSSB)≤40 where TDRX=0 for non-DRX

    - Lin,max = [5] for 40<Max(TDRX,TSSB)≤320

    - Lin,max = [3] for TDRX>320

- FFS whether for longer DRX cycles > 640, an additional criterion applies, to limit the acceptable period of consecutive LBT failures to [X] seconds

For RLM: out-of-sync

- Extend the out-of-sync evaluation period

  - Option 1: Out-of-sync evaluation period is scaled by a fixed factor of N (N > 1)

  - Option 2: Same as for other measurements (out-of-sync evaluation is based on Lout, where Lout ≤Lout,max is the number of SSBs not available at the UE during TEvaluate_out_SSB)

- FFS if UE can distinguish whether signal is available for RLM out-of-sync

For RLM: QCL-ed SSBs

Option 1:

UE is required to monitor at least one SSB from the set of SSBs that are QCLed with each other

Option 2:

UE is required to monitor all SSBs from the set of SSBs that are QCLed with each other

Option 3:

UE is required to monitor all SSBs regardless of QCL assumptions

For CSI-RS based RLM:

- RLM requirements based on periodic CSI-RS (TCSI-RS) are to be defined in Rel-16

  - Background: RAN1 agreed that CSI-RS-based RLM-RS both within and outside the SSB-based RLM measurement window (i.e., DRS transmission window) can be used for in-sync and out-of-sync evaluations (resolves FFS from previous agreement)

In this document, we discuss the remaining issues for requirements of RLM for NR-U.

Discussion

As background, RAN1 has achieved the following agreements on RLM for NR-U.

- An RLM measurement window for serving cell RLM measurements based on SSBs in the DRS is supported for in-sync and out-of-sync evaluations.

- For SSB-based RLM, UE may assume the RLM measurement window to be the same as DRS transmission window. Note this implies that the SSB-based RLM-RS cannot fall outside the measurement window, FFS whether and how DRS transmission window is configured to the UE.

- CSI-RS based RLM-RS both within and outside the SSB-based RLM measurement window (i.e., DRS transmission window) can be used for in-sync and out-of-sync evaluations.

From RAN1’s agreements, there is no consensus in RAN1 to introduce a mechanism to handle missing RLM-RS due to LBT failure.

From RAN4’s perspective, longer evaluation period has be expected for TEvaluate_out_SSB than that of TEvaluate_in_SSB, which is applicable to NR FR1 and FR2, as shown in Table 8.1.2.2-1/2 and 8.1.3.2-1/2 for SSB and CSI-RS based RLM. And these can be as a baseline for extending evaluation duration of RLM for NR-U

Table 8.1.2.2-1: Evaluation period TEvaluate_out_SSB and TEvaluate_in_SSB for FR1

| Configuration | TEvaluate_out_SSB (ms) | TEvaluate_in_SSB (ms) |
| --- | --- | --- |
| no DRX | max(200,ceil(10*P)*TSSB) | max(100,ceil(5*P)*TSSB) |
| DRX cycle≤320 | max(200,ceil(15*P)*max(TDRX,TSSB)) | max(100,ceil(7.5*P)*max(TDRX,TSSB)) |
| DRX cycle>320 | ceil(10*P)*TDRX | ceil(5*P)*TDRX |
| NOTE: TSSB is the periodicity of SSB configured for RLM. TDRX is the DRX cycle length. |  |  |

Table 8.1.2.2-2: Evaluation period TEvaluate_out_SSB and TEvaluate_in_SSB for FR2

| Configuration | TEvaluate_out_SSB (ms) | TEvaluate_in_SSB (ms) |
| --- | --- | --- |
| no DRX | max(200,ceil(10*P*N)*TSSB) | max(100,ceil(5*P*N)*TSSB) |
| DRX cycle≤320 | max(200,ceil(15*P*N)*max(TDRX,TSSB)) | max(100,ceil(7.5*P*N)*max(TDRX,TSSB)) |
| DRX cycle>320 | ceil(10*P*N)*TDRX | ceil(5*P*N)*TDRX |
| NOTE: TSSB is the periodicity of SSB configured for RLM. TDRX is the DRX cycle length. |  |  |

Table 8.1.3.2-1: Evaluation period TEvaluate_out_CSI-RS and TEvaluate_in_CSI-RS for FR1

| Configuration | TEvaluate_out_CSI-RS (ms) | TEvaluate_in_CSI-RS (ms) |
| --- | --- | --- |
| no DRX | max(200, ceil(Mout×P)×TCSI-RS) | max(100, ceil(Min×P) × TCSI-RS) |
| DRX ≤ 320ms | max(200, ceil(1.5×Mout×P)× max(TDRX, TCSI-RS)) | max(100, ceil(1.5×Min×P)× max(TDRX, TCSI-RS)) |
| DRX > 320ms | ceil(Mout×P) × TDRX | ceil(Min×P) × TDRX |
| NOTE: TCSI-RS is the periodicity of CSI-RS resource configured for RLM. The requirements in this table apply for TCSI-RS equal to 5 ms, 10ms, 20 ms or 40 ms. TDRX is the DRX cycle length. |  |  |

Table 8.1.3.2-2: Evaluation period TEvaluate_out_CSI-RS and TEvaluate_in_CSI-RS for FR2

| Configuration | TEvaluate_out_CSI-RS (ms) | TEvaluate_in_CSI-RS (ms) |
| --- | --- | --- |
| no DRX | max(200, ceil(Mout×P×N)×TCSI-RS) | max(100, ceil(Min×P×N) × TCSI-RS) |
| DRX ≤ 320ms | max(200, ceil(1.5×Mout×P×N)× max(TDRX, TCSI-RS)) | max(100, ceil(1.5×Min×P×N)× max(TDRX, TCSI-RS)) |
| DRX > 320ms | ceil(Mout×P×N) × TDRX | ceil(Min×P×N) × TDRX |
| NOTE: TCSI-RS is the periodicity of CSI-RS resource configured for RLM. The requirements in this table apply for TCSI-RS equal to 5 ms, 10 ms, 20 ms or 40 ms. TDRX is the DRX cycle length. |  |  |

#Issue 1: RLM In-Sync

RAN4 has decided to extend evaluation duration of RLM in-sync with Lin,max  RS samples. And we also support the values of Option 1 for CSI-RS with Lin,max periodic CSI-RS TCSI-RS as DRS transmission window TDRS.

  - Lin,max = [7] for Max(TDRX,TDRS)≤40 where TDRX=0 for non-DRX

  - Lin,max = [5] for 40<Max(TDRX,TDRS)≤320

  - Lin,max = [3] for TDRX>320

Proposal 1: For SSB based and CSI-RS based RLM-RS, extend evaluation duration of RLM in-sync with the following Lin,max  RS samples:

  - Lin,max = [7] for Max(TDRX,TDRS)≤40 where TDRX=0 for non-DRX

  - Lin,max = [5] for 40<Max(TDRX,TDRS)≤320

  - Lin,max = [3] for TDRX>320

Currently, RAN2 has agreed that the MAC CE can report multiple failed cells which have already declared consistent UL LBT failure. There is no indication for consecutive DL LBT failures. So for longer DRX cycles > 640, there is no need to define additional restriction due to consecutive DL LBT failures.

Proposal 2: For RLM in-sync, there is no need to define additional restriction due to consecutive DL LBT failures when longer DRX cycles > 640.

#Issue 2: RLM Out-of-Sync

Two options are to be down-selected to extend the out-of-sync evaluation period.

- Option 1: Out-of-sync evaluation period is scaled by a fixed factor of N (N > 1)

- Option 2: Same as for other measurements (out-of-sync evaluation is based on Lout, where Lout ≤Lout,max is the number of SSBs not available at the UE during TEvaluate_out_SSB)

RAN4 does not decide on whether UE can distinguish whether signal is available for RLM out-of-sync. From the majority's view, it is difficult to distinguish OOS comes from poor channel conditions or missing RLM-RS due to LBT failures. Some companies proposed to define max of OOS evaluation period only when the RLM-RS fall within the channel occupancy time duration and when UE can decode the DCI containing COT duration. However, based on RAN1’s feedback, DCI may not be always available. This cannot always help distinguish such scenario.

Thus, we support extend the out-of-sync evaluation period in an easier way defining fixed scaling factor for TEvaluate_out_DRS. As for FR2, the scaling factor N is already defined in Rel-15, N =8 for SSB RLM, N=1 for CSI-RS RLM. For FR1, in our view, for different configuration of DRX, the scaling factor can be different. For example, evaluation period TEvaluate_out_DRS for SSB based RLM in FR1 can be defined as Table 1.

Table 1: Evaluation period TEvaluate_out_DRS for NR-U in FR1

| Configuration | TEvaluate_out_SSB (ms) |
| --- | --- |
| no DRX | max(200,ceil(10*P*N1)*TDRS) |
| DRX cycle≤320 | max(200,ceil(15*P*N2)*max(TDRX,TDRS)) |
| DRX cycle>320 | ceil(10*P*N3)*TDRX |
| Note 1: TDRS is the periodicity of DRS configured for RLM. TDRX is the DRX cycle length. Note 2: N1, N2, N3 is fixed scaling factor due to LBT. |  |

For CSI-RS based RLM, the scaling factor can be reused.

Proposal 3: Define TEvaluate_out_DRS for NR-U with fixed scaling factor for different configuration:

  - For no DRX, N1=2

  - For DRX cycle≤320, N2 =2

  - For DRX cycle>320, N3 =1.5.

Conclusion

In this contribution, we propose our view as followings:

Proposal 1: For SSB based and CSI-RS based RLM, extend evaluation duration of RLM in-sync with the following Lin,max  RS samples:

  - Lin,max = [7] for Max(TDRX,TDRS)≤40 where TDRX=0 for non-DRX

  - Lin,max = [5] for 40<Max(TDRX,TDRS)≤320

  - Lin,max = [3] for TDRX>320

Proposal 2: For RLM in-sync, there is no need to define additional restriction due to consecutive DL LBT failures when longer DRX cycles > 640.

Proposal 3: Define TEvaluate_out_DRS for NR-U with fixed scaling factor for different configuration:

  - For no DRX, N1=2

  - For DRX cycle≤320, N2 =2

  - For DRX cycle>320, N3 =1.5.

References

- R4-1915777, WF on RRM requirements for NR-U, Ericsson