3GPP TSG-RAN WG4 Meeting #94-e                       	 R4- 2001027

# Electronic Meeting, 24 Feb. – 6 Mar., 2020

Agenda Item:	8.4.5.2

Source:	MediaTek Inc.

Title:	Discussion on NR V2X synchronization related requirement

Document for:		Discussion

- Introduction

In last RAN4 meeting, a WF about NR V2X RRM requirement was approved. The open issues about synchronization are listed below.

RAN4 #93 - Study Ping-Pong effect when gNB is timing reference source - RX Dropping Rate for selection/reselection of NR V2X synchronization reference source - UE is allowed to drop up to X slots of its V2X data reception per PSBCH monitoring occasion and overall drop rate shall not exceed [0.3%] of its V2X data reception during Tdetect,SyncRef UE_V2X for the purpose of selection / reselection to the SyncRef UE - Define X value in next meeting - Initiation/cease SLSS Tx with SyncRef UE as synchronization - Side condition - Option 1 : -6dB same as NR Uu measurement(LTE-V2X measurement) - Option 2 : 0dB - Other options are not precluded

In this paper, we will discuss on the above open issues about synchronization.

- Terminology

In legacy LTE, there are several requirements defined with different terminology, such as V2V, V2X etc. However, in NR R16, the unified terminology sidelink was used in RAN1 spec. Thus, RAN4 shall follow the same terminology to use sidelink to replace V2X in RAN4’s requirement.

Proposal 1: Use terminology ‘sidelink’ to replace ‘V2X’ in all RAN4’s sidelink requirement.

- Ping-Pong effect when gNB is timing reference source

When the measured RSRP of the sync source for transmission of sidelink communication is below the level indicated by RRC signalling, the UE may transmit SLSS. Although the evaluation requirement is extended to obtain better measurement robustness, however, due to measurement uncertainty, the averaged RSRP value may be sometimes better than the threshold and sometimes worse than the threshold. Therefore, sidelink UE may suffer Ping-Pong effect between transmitting and not transmitting SLSS Tx.

Observation 1: From RAN4’s view, the measurement uncertainty will result in ping-pong initiation of SLSS transmission.

In NR IDLE mode, although the evaluation period is even 16 DRX periods, the measurement margin is still added to avoid the ping-pong selection between serving cell and target best cell because of measurement uncertainty. For example, in Figure 1, when the cell’s RSRP value is less than measurement uncertainty plus serving cell’s RSRP threshold, such as cell 1, the UE won’t reselect to this target cell.

Figure 1. Cell reselection when rangeToBestCell is not configured

Observation 2: In NR IDLE mode, a measurement margin is added to the RSRP velue to avoid the ping-pong selection between serving cell and target best cell.

Similarly as the NR Uu IDLE mode requirement, a robust solution is to add a measurement margin to this RSRP results in NR V2X. Three different status can be defined based on the RSRP threshold and the 3dB margin.

- Not transmit SLSS(UE is in the center of a cell)

- Transmit SLSS(UE is in the edge of a cell)

- Hysteresis Zone

When UE is moving from state 1 to state 2, the UE doesn’t need to transmit SLSS in state 3. The UE only need to initiate the SLSS transmission when RSRP is less than the configured RSRP threshold. On the other side, when UE is moving from state 2 to state 1, the UE still need to transmit SLSS in state 3. The UE ceases the SLSS transmission if RSRP becomes larger than the configured RSRP threshold plus the 3dB margin.

Figure 2. Initiation/Cease SLSS based on RSRP margin

Proposal 2: Similar to NR IDLE mode, the measurement margin should be introduced when reselecting the sync. reference source and initiation of SLSS transmission.

- Reselection the Synchronization Reference Source

In LTE V2X, to avoid UEs to support multiple V2X TX/RX timings, it was recommended to modify the RRM requirements in order to allow UE to prioritize the PSBCH monitoring over PSSCH/PSCCH reception. During the SyncRef source detection, UE firstly searches the potential SyncRef UE SLSS. Once UE identified the reasonable SynRef source, UE knows the timing configuration of the subsequent PSBCH and SLSS transmissions from the particular SyncRef UE. Therefore, UE is permitted to drop its V2X reception procedures up to 2 subframes based on current synchronization source reference timing (e.g. PSCCH/PSSCH reception). Considering async scenarios, the data dropping rate in LTE V2X is

4(evaluation time 640ms/160ms) × 3(candidate sync. offset indication) × 2(async. subframes) × 1ms(unit of a subframe) / 8s = 0.3%

Observation 3: In LTE V2X, 2 subframes data reception dropping is permitted when UE is monitoring the PSBCH of potential SyncRef source.

In NR sidelink, RAN1 had already agreed to use the same numerology for S-SSB as reception signals. The dropping rate for different SCS within the SLSS periodicity of 160ms is shown below. Thus, to dealing with 30KHz and 60KHz SCS, we can simply extend the LTE V2X requirement (which is based on 15KHz SCS) with the same dropping rate but shorten the overall evaluation time.

| SCS(KHz) | # of slots in 160ms | Number of slots that can be dropped | Dropping rate |
| --- | --- | --- | --- |
| 15 | 160 | 6 | 6/160 |
| 30 | 320 | 6 | 6/320 |
| 60 | 640 | 6 | 6/640 |

RAN1#94bis Agreements: - UE assumes a single numerology in using a resource pool. RAN1#95 Agreements: - Each resource pool is (pre)configured within a SL BWP. - Only one SL BWP is configured in a carrier for a NR V2X UE. Numerology is a part of SL BWP configuration. - S-SSB has the same numerology, which includes SCS and CP length, as that of control and data channels for a given carrier. RAN1 #AH1901 Agreements: - Only one SL BWP is configured in a carrier for a NR V2X UE. - Configuration for SL BWP is separated from Uu BWP configuration signalling. UE is not expected to use different numerology in the configured SL BWP and active UL BWP in the same carrier at a given time.

Proposal 3: UE is allowed to drop up to 2 slots of its V2X data reception per PSBCH monitoring occasion and overall drop rate shall not exceed [0.3%] of its V2X data reception during Tdetect,SyncRef UE_V2X for the purpose of selection / reselection to the SyncRef UE. Tdetect,SyncRef UE_V2X is defined as follow.

| SCS(KHz) | Tdetect,SyncRef UE_V2X(s) |
| --- | --- |
| 15 | 8 |
| 30 | 4 |
| 60 | 2 |
| 120 | 1 |

- Initiation/Cease of SLSS transmission

The remaining issue is the side condition for initiation/cease of SLSS transmission when SyncRef UE is the SyncRef source. The intention of sidelink UE initiates the SLSS transmission is that this sidelink UE is in the coverage edge of its SyncRef source. This sidelink UE shall transmit the SLSS to maintain the timing of other sidelink UEs which is directly or in-directly synced to this UE. Thus, if the side condition for initiation/cese of SLSS transmission is at a high SNR level, the system performance will be dramatically decreased because too many UEs transmit the SLSS. The same side condition -6dB as NR Uu and LTE V2X is a reasonable value for NR sidelink UEs.

Proposal 4: The same side conditions -6dB as NR Uu and LTE V2X measurements shall be used in NR sidelink.

- Summary

In this paper, we have some discussion on synchronization related requirement.

Observation 1: From RAN4’s view, the measurement uncertainty will result in ping-pong initiation of SLSS transmission.

Observation 2: In NR IDLE mode, a measurement margin is added to the RSRP velue to avoid the ping-pong selection between serving cell and target best cell.

Observation 3: In LTE V2X, 2 subframes data reception dropping is permitted when UE is monitoring the PSBCH of potential SyncRef source.

Proposal 1: Use terminology ‘sidelink’ to replace ‘V2X’ in all RAN4’s sidelink requirement.

Proposal 2: Similar to NR IDLE mode, the measurement margin should be introduced when reselecting the sync. reference source and initiation of SLSS transmission.

Proposal 3: UE is allowed to drop up to 2 slots of its V2X data reception per PSBCH monitoring occasion and overall drop rate shall not exceed [0.3%] of its V2X data reception during Tdetect,SyncRef UE_V2X for the purpose of selection / reselection to the SyncRef UE. Tdetect,SyncRef UE_V2X is defined as follow.

| SCS(KHz) | Tdetect,SyncRef UE_V2X(s) |
| --- | --- |
| 15 | 8 |
| 30 | 4 |
| 60 | 2 |
| 120 | 1 |

Proposal 4: The same side conditions -6dB as NR Uu and LTE V2X measurements shall be used in NR sidelink.

- Reference

- R4-1915783, “WF on NR V2X RRM requirement”, LG Electronics