3GPP TSG-RAN WG4 Meeting #94-e                       	 R4- 2001028

# Electronic Meeting, 24 Feb. – 6 Mar., 2020

Agenda Item:	8.4.5.3

Source:	MediaTek Inc.

Title:	Discussion on NR V2X measurements requirement

Document for:		Discussion

- Introduction

In last RAN4 meeting, a WF about NR V2X RRM requirement was approved. The open issues about measurement are listed below.

- Autonomous Resource Reselection measurement - Define requirement for resource selection/reselection measurement based on single-shot L1 SL-RSRP - Whether to define other measurement requirements needs RAN1’s further inputs - RSSI measurement for congestion control - Base line is that the S-RSSI measurement accuracy requirements in LTE V2X can be reused for defining SL-RSSI measurement accuracy requirements in NR V2X - After RAN1 measurement procedure is completed, it should be reconsidered - Resource pre-emption mechanism - Study whether define or not requirement to verify resource pre-emption mechanism

In RAN1 #98bis meeting, some agreements for resource reselection are listed as follow.

Agreements: - Resource (re-)selection procedure supports re-evaluation of Step 1 and Step 2 before transmission of SCI with reservation - The re-evaluation of the (re-)selection procedure for a resource reservation signalled in a moment ‘m’ is not required to be triggered at moment > ‘m – T3’ (i.e. resource reselection processing time needs to be ensured) - FFS condition to change resource(s) from previous iteration to resource(s) from current iteration - FFS relationship of T1 and T3, if any - FFS whether to handle it differently for blind and feedback-based retransmission resources Agreements: - Support a resource pre-emption mechanism for Mode-2 - A UE triggers reselection of already signaled resource(s) as a resource reservation in case of overlap with resource(s) of a higher priority reservation from a different UE and, SL-RSRP measurement associated with the resource reserved by that different UE is larger than an associated SL-RSRP threshold - Only the overlapped resource(s) is/are reselected - FFS - the timeline for reselection - other details - FFS whether or not to support other potential UE behaviour (e.g, power boosting/reduction) - This mechanism can be enabled or disabled, per resource pool - FFS details

In RAN1 #99 meeting, some RSRP measurements agreements for resource reselection are listed as follow.

Agreements: … - Procedure of using sidelink RSSI for ranking of resources is not applied - Support (pre)-configuration per resource pool between: - L1 SL-RSRP measured on DMRS of PSSCH after decoding of associated 1st stage SCI, or - L1 SL-RSRP measured on DMRS of PSCCH for 1st SCI after decoding of associated 1st stage SCI - Note: L1 SL-RSRP is measured only based on one of the above, but not both

In this paper, we will discuss on the RAN4 measurements’ open issues based on RAN1’s newest agreements.

- Resource reselection procedure for sidelink - Mode 2

S-RSSI

In legacy LTE sidelink, the S-RSSI measurement is used to sort the reservation resources after candidate resource selection by SL RSRP measurement, but this procedure will be not applied in NR based on the latest RAN1 agreement. Thus, RAN4 doesn’t need to define any requirement for S-RSSI measurement for autonomous resource reselection for NR sidelink.

Proposal 1: Do not define S-RSSI measurement requirement for resource reselection in NR sidelink.

Resource Reselection Procedure

Different to LTE sidelink, two new procedures are introduced in NR sidelink autonomous resource reselection.

- Re-evaluation

After resource (re-)selection procedure, the UE will determine a subset of resources for PSSCH/PSCCH transmission. The resource reservation period and time-frequency resource assignment information will be transmitted by UE. In NR sidelink, the same UE has to re-evaluate the reserved resources before transmission of SCI with reservation.

- Pre-emption

In NR sidelink, the UE also supports to trigger reselection of already signaled resource(s) in case that a resource reservation is overlapped with resource(s) of a higher priority reservation from a different UE.

In legacy LTE sidelink RRM requirement, RAN4 didn’t capture any resource reselection procedure in core requirement because this procedure had already explicitly captured in RAN1 requirement. Similarly as LTE sidelink, NR sidelink RRM requirements for resource reselection procedures are not needed. Only the L1 SL-RSRP measurements requirement needs to be captured in NR sidelink RRM requirement.

Proposal 2: Do not explicitly define SL resource reselection, re-evaluation, and pre-emption procedures in RAN4 core requirement. RAN4 only needs to define the L1 SL-RSRP measurement requirement.

In latest RAN1 agreement, L1 SL-RSRP supports both PSSCH and PSCCH DMRS RSRP measurement. The higher layer parameter RSforSensing will be used to indicate UE use PSSCH-RSRP or PSCCH-RSRP to reselect resources.

Proposal 3: Introduce L1 SL-RSRP measurement requirements for both PSSCH-RSRP and PSCCH-RSRP.

- L1 SL-RSRP

Time-Frequency Resource for L1 SL-RSRP

The less DMRS resources used, the worse RSRP performance can achieved. RAN4 should consider the worst case on evaluating the L1 SL-RSRP measurement accuracy for the minimum requirement definition.

In frequency domain, RAN1 had agreed the minimum sub-channel PRB size is 10 PRBs for PSSCH and PSCCH. Thus, RAN4 defines the PSCCH and PSSCH measurements requirements base on 10 PRBs.

RAN1 #98bis meeting Agreements: - Support {10, 15, 20, 25, 50, 75, 100} PRBs for possible sub-channel size. - FFS other values (e.g., 4, 5, 6, etc.) - One value of the above set is (pre)configured for the sub-channel size for the resource pool. RAN1 #99 meeting Working assumption: - Candidate numbers of PRBs for 2-symbol and 3-symbol PSCCH are - {10, 12 15, 20, 25}

Proposal 4: Define PSCCH measurements requirement base on 10 PRBs.

Proposal 5: Define PSSCH measurement requirement base on 10 PRBs.

In time domain, RAN1 had agreed PSCCH pattern can be 2-symbol or 3-symbol. PSCCH-DMRS will occupied in all the PSCCH symbols. Thus, RAN4 should define PSCCH measurements requirement based on 2-symbol pattern.

For PSSCH DMRS pattern, RAN1 had agreed 2, 3, and 4 symbols for PSSCH DMRS. Thus, RAN4 should also define PSSCH measurements requirement based on 2-symbol pattern.

RAN1 #98bis meeting Agreements: - Rel-15 NR PDCCH DMRS pattern is reused for PSCCH DMRS pattern. - For frequency-domain pattern for PSCCH DMRS, reuse Rel-15 NR PDCCH DMRS, i.e., comb-4 fixed RE mapping for PSCCH DMRS. - (Working assumption) For time-domain pattern for PSCCH DMRS, every symbol of PSCCH has PSCCH DMRS REs. RAN1 #99 meeting Agreements: - For Rel-16, (normal CP) - Support 7, 8, 9,…, 14 symbols in a slot without SL-SSB for SL operation - Target reusing Uu DM-RS patterns for each of the symbol-length, with modifications as necessary - No other additional spec impact is expected for supporting 7, 8, …, 13 - # of DM-RS symbols - 2, 3, 4 - For a dedicated carrier, only 14-symbol is mandatory

Proposal 6: Define PSCCH RSRP measurement requirement base on 2 symbols.

Proposal 7: Define PSSCH RSRP measurement requirement base on 2 DMRS symbol pattern.

However, when 3 or 4 PSSCH DRMS symbols are configured, collision between PSSCH DMRS and PSCCH happens on the same OFDM symbol, as shown in below figure. Furthermore, if PSSCH sub-channel size equals PSCCH, the 1st symbol’s DMRS in the pattern is completely missing. In this case, UE behavior seems undefined. Some clarification from RAN1 is required before RAN4 discusses the corresponding measurement requirement.

| - PSCCH occupies only a subset of the PRBs of PSSCH | - PSCCH occupies whole PRBs of PSSCH |
| --- | --- |

Proposal 8: Send LS to RAN1 to clarify the PSSCH DMRS pattern when PSSCH sub-channel size equals PSCCH.

Measurement on multiple DMRS antenna ports

RAN1 #96bis meeting Working assumption: - Transmission of 1 TB with up to 2 layers in a PSSCH is supported. RAN1 #97 meeting Working assumption: - Rel-15 PDSCH DMRS Configuration type 1 and/or type 2 are reused for frequency-domain pattern of PSSCH DMRS. - FFS whether to support either one or both types - FFS details on multiplexing of different ports for PSSCH DMRS

In NR sidelink, the PSSCH supports at most 2 layers MIMO transmission. When UE is configured to use PSSCH-RSRP in resource reselection, UE may have to compare the RSRP measured from single antenna port and RSRP measured from two antenna ports. PSSCH-RSRP is used to evaluate whether a certain resource could be used to transmission signals. Thus, UE should know how to calculate the PSSCH-RSRP of single or two antenna ports.

RAN1 decided to re-use NR Uu PDSCH DMRS configuration design in NR sidelink, but didn’t clearly indicate the detail design. In currently NR Uu PDSCH DMRS design, there are following 5 scenarios for PDSCH DMRS configuration.

  - Scenario 1: CDM group number = 1, DMRS port 0 or 1 (single port)

  - Scenario 2: CDM group number = 1, DMRS port 0 and 1 (dual ports)

  - Scenario 3: CDM group number = 2, DMRS port 0 or 1 (single port)

  - Scenario 4: CDM group number = 2, DMRS port 0 and 1 (dual ports)

  - Scenario 5: CDM group number = 2, DMRS port 0 and 2 (dual ports)

The ratio of PDSCH EPRE to DMRS EPRE is defined in TS38.214 shown as follow. There are different PDSCH EPRE ratio to PDSCH for different CDM group number. Thus, the way to calculate the RSRP for different scenarios shall also be different.

Table 1. The ratio of PDSCH EPRE to DM-RS EPRE

| Number of DM-RS CDM groups without data | DM-RS configuration type 1 | DM-RS configuration type 2 |
| --- | --- | --- |
| 1 | 0 dB | 0 dB |
| 2 | -3 dB | -3 dB |
| 3 | - | -4.77 dB |

Figure 1. DMRS pattern for different number of CDM group

For example, there are two transmission UEs, each UE’s transmit power per RE is P. Assuming UE1 uses single antenna port with CDM group=1, it means that the calculated DMRS RSRP for UE1 will be power P (not consider the estimation bias and path loss). Assuming UE2 uses two antenna ports transmission with CDM group=1, it means each antenna port’s DMRS RSRP power will be P/2. Therefore, UE shall know the clear definition for calculating the PSSCH-RSRP.

In NR sidelink, the precoding is also used in MIMO transmission of PSSCH channel and the associated DMRS. PSSCH-RSRP is used to evaluate the data traffic busy ratio in subframes to decide whether exclude the slots in candidate resources. With precoding, UE#1 may observe low RSRP from UE#2 (and vice versa) and then both UE may decide to transmit signals to UE#3 on the same radio resource. Furthermore, the UE cannot to know the reason of higher PSSCH-RSRP is from busy traffic or precoding gain. Thus, it’s better to RAN4 to define the PSSCH-RSRP performance simulation based on single antenna port at first.

Figure 2. Precoding Impact for PSSCH RSRP Calculation

Proposal 9: RAN4 should ask RAN1 to clarify the PSSCH-RSRP definition regarding different number of CDM groups and different number of antenna ports.

Proposal 10: RAN4 shall evaluate the PSSCH-RSRP performance based on single antenna port as baseline.

Side condition

The agreed L1 SL-RSRP measurements should be conducted after decoding of associated 1st stage SCI. It means at least the side condition of L1 SL-RSRP measurements should equal with the SNR for correctly decoding the PSCCH of 1st stage SCI. Currently, RAN1 had already agreed some NR PSCCH design. The PSCCH DMRS pattern can reuse NR PDCCH DMRS pattern.

RAN1 #98bis meeting Agreements: - Rel-15 NR PDCCH DMRS pattern is reused for PSCCH DMRS pattern. - For frequency-domain pattern for PSCCH DMRS, reuse Rel-15 NR PDCCH DMRS, i.e., comb-4 fixed RE mapping for PSCCH DMRS. - (Working assumption) For time-domain pattern for PSCCH DMRS, every symbol of PSCCH has PSCCH DMRS REs. - FFS: how to initialize DMRS sequence generator

Based on the discussion above, a reasonable PSCCH configuration should be 10PRBs and 2 symbols. The typical PSCCH simulation configuration is shown as follow.

Table 2. NR sidelink PSCCH Parameters

| Parameter | Unit | Value |
| --- | --- | --- |
| at antenna port | dBm/15kHz | -98 |
| Active cell(s) |  | Cell 1(Serving Cell) |
| SCS | KHz | 15 |
| Allocated resource blocks(see Note 1) |  | 10 |
| OFDM Symbols per slot (see Note 2) |  | 2 |
| Modulation |  | QPSK |
| Payload (without CRC) | Bits | 41(see Note 3) |
| CRC | Bits | 24 |
| SCI Format |  | 0-1 |
| Number of PSCCH transmissions |  | 1 |
| Timing offset (Note 4) |  | CP/2-12Ts |
| Frequency offset (Note 5) | Hz | +1200 |
| Synchronization |  | gNB or gNB-equivalent |
| Antenna configuration |  | 1x2 Low |
| Propagation conditions | - | AWGN, TDL- C with 30ns, 1400Hz, TDL- C with 100ns, 300Hz |
| Note 1: PSCCH candidate numbers of PRBs are {10, 12 15, 20, 25}. Note 2: PSCCH for 1st stage SCI with 2 and 3 symbols is supported. Note 3: One possible PSCCH payload size with Nmax=3. The detail payload size depends on RAN1 design. Note 3: Time offset of Sidelink UE receive signal with respect to gNB reference timing. Note 4: Frequency offset of Sidelink UE with respect to gNB reference frequency. |  |  |

Proposal 11: The side condition for L1 SL-RSRP measurements shall guarantee successful decoding of 1st stage SCI.

Proposal 12: RAN4 shall define the simulation assumption of 1st stage SCI to evaluate the side condition for L1 SL-RSRP measurements.

- Summary

In this paper, we have some discussion on the SL L1-RSRP measurements requirement.

Proposal 1: Do not define S-RSSI measurement requirement for resource reselection in NR sidelink.

Proposal 2: Do not explicitly define SL resource reselection, re-evaluation, and pre-emption procedures in RAN4 core requirement. RAN4 only needs to define the L1 SL-RSRP measurement requirement.

Proposal 3: Introduce L1 SL-RSRP measurement requirements for both PSSCH-RSRP and PSCCH-RSRP.

Proposal 4: Define PSCCH measurements requirement base on 10 PRBs.

Proposal 5: Define PSSCH measurement requirement base on 10 PRBs.

Proposal 6: Define PSCCH RSRP measurement requirement base on 2 symbols.

Proposal 7: Define PSSCH RSRP measurement requirement base on 2 DMRS symbol pattern.

Proposal 8: Send LS to RAN1 to clarify the PSSCH DMRS pattern when PSSCH sub-channel size equals PSCCH.

Proposal 9: RAN4 should ask RAN1 to clarify the PSSCH-RSRP definition regarding different number of CDM groups and different number of antenna ports.

Proposal 10: RAN4 shall evaluate the PSSCH-RSRP performance based on single antenna port as baseline.

Proposal 11: The side condition for L1 SL-RSRP measurements shall guarantee successful decoding of 1st stage SCI.

Proposal 12: RAN4 shall define the simulation assumption of 1st stage SCI to evaluate the side condition for L1 SL-RSRP measurements.

- Reference

- R4-1915783, “WF on NR V2X RRM requirement”, LG Electronics