3GPP TSG-RAN WG4 Meeting #94-e                       	 R4- 2001035

# Electronic Meeting, 24 Feb. – 6 Mar., 2020

Agenda Item:	8.15.1.3

Source:	MediaTek Inc.

Title:	Discussion on CGI reading requirement for NR

Document for:		Discussion

- Introduction

In last meeting, a WF of CGI reading has been agreed [1]. The remaining issues are captured as follow.

- Do not define requirements for unknown cells - Known cell condition for FR1 - It has been meeting the relevant cell identification requirement during the last 5 seconds. - UE sends at least a valid L3-RSRP reporting. - FFS: The SSB with the same index as the one with best RSRP measurement remains detectable. - Depending on if the interruption is defined in symbol level or SMTC level. - During CGI reading, the SSB [with the same index (in the L3-RSRP reporting)] remains detectable - Known cell conditions for FR2 - Option 1: reuse known cell condition for NR FR2 handover - Other options are not precluded - Interruptions for each autonomous gap during MIB decoding - Option 1: SMTC duration + 2*RF tuning time + 1 slot (victim cell SCS) - Option 2: 4 symbols (target cell SCS) + 2*RF tuning time + 1 slot (victim cell SCS) - Interruptions for each autonomous gap during SIB1 decoding - Option 1: - 2*RF tuning time + 2 slots (target cell SCS) + 1 slot (victim cell SCS) for multiplexing pattern 1 - 2*RF tuning time + 1 slot (target cell SCS) + 1 slot (victim cell SCS) for multiplexing pattern 2/3 - Option 2: other options are not precluded

In this paper, we will continue to discuss the requirements for CGI reading for NR cell.

- Known Cell Definition

In RAN4 previous discussion, it considers different scenarios for Known or Unknown target Cell. In the requirements, additional delay for AGC retuning, Rx beam switch (in FR2) and cell search is allowed for Unknown Cell. In CGI reading, the procedure is a little different, the UE had already searched the cell and reported the measurements to the network. Thus, the additional delay for cell search does not need in unknown cell. Whether to allow additional time for AGC and Rx beam switching (in FR2) still depends on the duration between the moment UE sending the last measurement report to the network and the moment UE receiving the CGI request from the network.

Observation 1: Whether to allow additional time for AGC and Rx beam switching (in FR2) still depends on the duration between the moment UE sending the last measurement report to the network and the moment UE receiving the CGI request from the network.

In last meeting, one of the open issue is whether to define the known condition based on the reported SSB or overall SMTC duration. The reason why we define the known condition is that the UE can reduce the additional delay for AGC retuning, Rx beam switching (in FR2) and cell search in known cell. When UE had sent a L3-RSRP measurement report with SSB index, it means that the UE had finished the Rx beam sweeping and knows the Tx-Rx beam pair for this SSB. However, if the known condition is defined based on overall SMTC duration, it implicitly indicates that the reported SSB may be undetectable and other SSB(s) can be detected during the CGI reading procedure. Thus, UE may still need Rx beam sweeping to find a suitable Tx-Rx beam pair for other SSB(s). Considering CGI reading is a best effort procedure, it’s better to define the known condition to accelerate the overall procedure other than increasing the chance to fulfill the condition in real field. At the same time, the similar known condition definition had already applied only on the measured SSB in other procedure’s requirement, such as SCell activation. Therefore, it’s reasonable to only use measured SSB to define the known condition in CGI reading.

Proposal 1: Define the known condition base on reported SSB other than overall SMTC duration in CGI reading.

In SCell activation, the known condition is defined as after UE has sent the L3-RSRP measurement reporting with SSB index, the SCell activation command shall be configured during the period equal to 3s. From our view, the known condition definition for CGI reading which is also based on L3-RSRP reporting is similar as SCell activation as follow.

Proposal 2: For the target CGI reading in FR2 bands, the CGI reading is under the known condition if it has been meeting the following conditions:

-	During the period equals to [3s] from the last transmission of the SSB used for L3-RSRP report to UE receives the target CGI reading command,

-	the UE has sent a valid L3-RSRP measurement report

-	During the period from UE sends a valid L3-RSRP reporting to UE repots a valid CGI,

-	the SSBs used for L3-RSRP report remain detectable according to the cell identification conditions specified in clauses 9.2 and 9.3

-	the MIB information contained in the SSB used for L3-RSRP report remains decodable with the SNR ≥ [-3]dB

-	the RMSI CORSETs associated with the SSB used for L3-RSRP report remain detectable with the SNR ≥ [-3]dB

- CGI reading procedure

In the general requirement framework, what UE needs to do in NR is not much different with that in LTE, e.g., MIB and SIB decoding. But details could be different due to the new design in NR, such as Rx beam sweeping in FR2. In general, the overall CGI reading includes the following steps (as shown in Figure 1):

- AGC/AFC tuning before the first time receiving MIB

- Rx beam sweeping

- MIB decoding

- RMSI decoding

Figure 1. The overall CGI reading flow in the time domain

Proposal 3: The Delay of autonomous gap based CGI reading can be divided into the following time units

Tbasic_identify_CGI,NR=TAGC+ TMIB, decoding+ TRMSI, decoding

AGC/AFC tuning before the first time receiving MIB (TAGC)

For FR1 intra-frequency, no additional AGC retuning time is needed because the UE has already retuning the AGC for serving frequency.

For FR1 inter-frequency known CGI reading, UE could have some prior knowledge for AGC because UE has measured the SSBs and reported its RSRP. But if the time period for known definition between the last L3-RSRP report and CGI reading command is larger than the time duration required for maintaining the effective AGC information by UE, an additional 1 shot AGC retuning time is needed.

Proposal 4: In FR1,

- TAGC=0 for intra-frequency known CGI reading,

- TAGC=TSSB for inter-frequency known CGI reading.

Where, TSSB is the SMTC periodicity of the carrier with target cell.

For FR2 intra-frequency and inter-frequency known cell, if the time period for known definition between the last L3-RSRP report and CGI reading command is larger than the time duration required for maintaining the effective Rx beam information by UE, the UE may lost the prior knowledge to choose which Rx beam to receive the neighbor cell signal. Additional Rx beam sweeping with AGC retuning shall be considered. If the time period for known definition is less the duration for maintaining the effective Rx beam information by UE, the UE may only need 1 shot additional AGC retuning time for inter-frequency known CGI reading.

Proposal 5: In FR2, when the UE can be guaranteed to use the same Rx spatial filter during AGC retuing,

- TAGC=0 for intra-frequency known CGI reading,

- TAGC=TSSB for inter-frequency known CGI reading.

Where, TSSB is the SMTC periodicity of the carrier with target cell.

MIB decoding (TMIB, decoding)

For FR1 MIB decoding, it has already agreed to re-use FR2 MIB decoding performance requirement as 5TSSB. The Rx beam sweeping can be captured in AGC retuning. After UE finished the Rx beam sweeping and chose the same Rx beam to decode the MIB and SIB information, the MIB decoding delay for FR2 can be use the same value as FR1 when the UE can be guaranteed to use the same Rx spatial filter to finish MIB decoding in FR2.

Proposal 6: Assuming UE uses the same Rx spatial filter to finish MIB decoding in FR2, the MIB decoding delay are [5] ×TSSB for both FR1 and FR2, where TSSB is the SMTC periodicity of the carrier with target cell.

RMSI decoding(TRMSI, decoding)

The RAN1’s agreement for RMSI is shown as follow.

RAN1 #90bis Agreements: - RMSI TTI is 160ms from RAN1 perspective RAN1 #91 Agreements: - When the SS/PBCH blocks and corresponding RMSI CORESETs occur in different time instances, - The UE assumes that the RMSI CORESET monitoring window corresponding to an SS/PBCH block in the radio frame satisfies the condition mod(SFN,2)=0 - Note: RMSI scheduling periodicity is up to gNB implementation - In Rel-15, NR does not support RMSI search space sets configuration where the SS/PBCH blocks and corresponding RMSI search space sets occur in the same time instances if the carrier frequency is lower than 6GHz.

Considering the agreement from RAN1, the RMSI TTI is 160ms. The challenge for RMSI decoding compared to MIB decoding is that RMSI scheduling periodicity is up to gNB implementation based on the RAN1 agreement. The periodicity and offset are not known to UE. Note that this also has the impact to the time needed in AGC for RMSI.

Once the UE has no prior information about the RMSI scheduling periodicity and offset, UE may failed to decode RMSI because the network actually doesn’t send RMSI in the slot that UE tried to decode. To overcome this problem, the UE had to interrupt the serving cell receiving to try the RMSI receiving at all the possible RMSI positions based on the minimum RMSI scheduling periodicity. For example, we assume UE needs to try 3 attempts in order to successfully decode the RMSI. The network schedules the RMSI based on 40ms periodicity, but UE has no prior information. UE had to interrupt the serving cell by the autonomous gap with a 20ms periodicity. In this case, 6 autonomous gap interruption are required even if UE really needs only 3 samples.

Figure 2. Uncertainty RMSI Reception

Proposal 7: The UE’s interruption for RMSI decoding shall base on minimum RMSI scheduling periodicity 20ms. It means 8 interruption occasions shall be counted in each 160ms RMSI periodicity.

When the interruption duration is defined as the duration of PDCCH and PDSCH for RMSI + 2*RF tuning time during each autonomous gap, the key point is that the side condition for RMSI detection should be guaranteed so that the RMSI associated with the same SSB reporting index can be used by UE for the whole CGI reading procedure. Otherwise, UE has to try a different SSB and RMSI. The potential SNR side condition for RMSI detection could be -3dB.

Proposal 8: To guarantee the RMSI with the same SSB reporting index can be detected during the whole CGI reading, the potential SNR side condition could be [-3dB].

In last meeting, a SIB1 decoding simulation assumption was agreed [2]. The one shot simulation results for SIB1 decoding are attached below. Considering one shot detection BLER equals 0.7 for SNR=-3dB, 7 times RMSI decoding can fulfill the 90% target success rate for SIB1 decoding.

Figure 3. One-shot simulation results for SIB1 decoding

Proposal 9: The RMSI decoding delay is [7] ×TRMSI, where TRMSI is the RMSI scheduling periodicity based on gNB implementation.

- Summary

In this paper, we propose the CGI reading requirement for NR SA toward NR cell. In our opinion, the CGI reading requirement will only depend on the target cell’s delay requirement.

Observation 1: Whether to allow additional time for AGC and Rx beam switching (in FR2) still depends on the duration between the moment UE sending the last measurement report to the network and the moment UE receiving the CGI request from the network.

Proposal 1: Define the known condition base on reported SSB other than overall SMTC duration in CGI reading.

Proposal 2: For the target CGI reading in FR2 bands, the CGI reading is under the known condition if it has been meeting the following conditions:

-	During the period equals to [X ms] from the last transmission of the SSB used for L3-RSRP report to UE receives the target CGI reading command,

-	the UE has sent a valid L3-RSRP measurement report with SSB index

-	During the period from UE sends a valid L3-RSRP reporting to UE repots a valid CGI,

-	the SSBs used for L3-RSRP report remain detectable according to the cell identification conditions specified in clauses 9.2 and 9.3

-	the MIB information contained in the SSB used for L3-RSRP report remains decodable with the SNR ≥ [-3]dB

-	the RMSI CORSETs associated with the SSB used for L3-RSRP report remain detectable with the SNR ≥ [-3]dB Proposal 3: The Delay of autonomous gap based CGI reading can be divided into the following time units

Tbasic_identify_CGI,NR=TAGC+ TMIB, decoding+ TRMSI, decoding

Proposal 4: In FR1,

- TAGC=0 for intra-frequency known CGI reading,

- TAGC=TSSB for inter-frequency known CGI reading.

Where, TSSB is the SMTC periodicity of the carrier with target cell.

Proposal 5: In FR2,

- TAGC=0 for intra-frequency known CGI reading,

- TAGC=TSSB for inter-frequency known CGI reading.

Where, TSSB is the SMTC periodicity of the carrier with target cell.

Proposal 6: When the UE can be guaranteed to use the same Rx spatial filter to finish MIB decoding in FR2, the MIB decoding delay are [5] ×TSSB for both FR1 and FR2, where TSSB is the SMTC periodicity of the carrier with target cell.

Proposal 7: The UE’s interruption for RMSI decoding shall base on minimum RMSI scheduling periodicity 20ms. It means 8 interruption occasions shall be counted in each 160ms RMSI periodicity.

Proposal 8: To guarantee the RMSI with the same SSB reporting index can be detected during the whole CGI reading, the potential SNR side condition could be [-3dB].

Proposal 9: The RMSI decoding delay is [7] ×TRMSI, where TRMSI is the RMSI scheduling periodicity based on gNB implementation.

- References

- R4-1915932, “Way forward on CGI reading for RRM enh”, ZTE

- R4-1915851, “Simulation assumption on SIB1 decoding for NR CGI reading”, ZTE