3GPP TSG-RAN WG4 Meeting #94-e                                                               R4-2001061

Electronic Meeting, 24th February – 6th March, 2020

Source:	Huawei, HiSilicon

Title:	TP to TR 38.716.02-00 for CA_n20A-n75A

Agenda item:	9.2.2

Document for:	Approval

Background

This contribution provides text proposal on the NR CA band combination CA_n20A-n75A as defined in Revised WID on NR inter-band Carrier Aggregation/Dual connectivity for 2 bands DL with x bands UL (x=1, 2) RP-192615 [1].

Text Proposal

---Start of changes---

6.x	CA_n20-n75

6.x.1	Common for 1 band UL and 2 bands UL CA

6.x.1.1 Operating bands for CA

Table 6.x.1.1-1: CA band combination CA_n20A-n75A

| NR CA Band Combination | NR Band | Uplink (UL) band |  |  | Downlink (DL) band |  |  | Duplex mode |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | BS receive / UE transmit |  |  | BS transmit / UE receive |  |  |  |
|  |  | FUL_low – FUL_high |  |  | FDL_low – FDL_high |  |  |  |
| CA_n20-n75 | n20 | 832 MHz | – | 862 MHz | 791 MHz | – | 821 MHz | FDD |
|  | n75 | N/A |  |  | 1432 MHz | – | 1517 MHz | SDL |

6.x.1.2	Channel bandwidths per operating band for CA

Table 6.x.1.2-1: Supported bandwidths per CA band combination CA_n20A-n75A

| CA operating / channel bandwidth [MHz] |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| NR CA Configuration | UL Configuration | NR Band | SCS [kHz] | 5 | 10 | 15 | 20 | 25 | 30 | 40 | 50 | 60 | 80 | 90 | 100 | Bandwidth combination set |
| CA_n20A-n75A | - | n20 | 15 | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  |  | 0 |
|  |  |  | 30 |  | Yes | Yes | Yes |  |  |  |  |  |  |  |  |  |
|  |  |  | 60 |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  | n75 | 15 | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  |  |  |
|  |  |  | 30 |  | Yes | Yes | Yes |  |  |  |  |  |  |  |  |  |
|  |  |  | 60 |  | Yes | Yes | Yes |  |  |  |  |  |  |  |  |  |

6.x.1.3	Co-existence studies

Table 6.x.1.3-1/2 summarizes frequency ranges where harmonics and/or harmonics mixing occur for CA_n20-n75.

Table 6.x.1.3-1: Impact of UL/DL Harmonic

|  |  |  |  |  | 2nd Harmonic |  | 3rd Harmonic |  | 4th Harmonic |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Band | UL Low Band Edge | UL High Band Edge | DL Low Band Edge | DL High Band Edge | UL Low Band Edge | UL High Band Edge | UL Low Band Edge | UL High Band Edge | UL Low Band Edge | UL High Band Edge |
| n20 | 832 | 862 | 791 | 821 | 1664 | 1724 | 2496 | 2586 | 3328 | 3448 |
| n75 | - | - | 1432 | 1571 |  |  |  |  |  |  |

Based on above table, the 4th harmonics of band n20 UL will fall into the band n75 Rx.

Table 6.x.1.3-2: Impact of UL/DL Harmonic mixing

|  |  |  |  |  | 2nd Harmonic |  | 3rd Harmonic |  | mth Harmonic |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Band | UL Low Band Edge | UL High Band Edge | DL Low Band Edge | DL High Band Edge | DL Low Band Edge | DL High Band Edge | DL Low Band Edge | DL High Band Edge | DL Low Band Edge | DL High Band Edge |
| n20 | 832 | 862 | 791 | 821 | 1582 | 1642 | 2373 | 2463 |  |  |
| n75 |  |  | 1432 | 1571 | 2864 | 3142 | 4296 | 4713 |  |  |

Based on above table, there is no harmonics mixing issue for the band combination of n20 and n75.

6.x.1.4	∆TIB and ∆RIB values

For CA_n20A-n75A, the TIB,c and RIB can follow the values of DC_20_n75. The values are given in the tables below.

Table 6.x.1.4-1: ΔTIB,c

| Inter-band CA Configuration | NR Band | ΔTIB,c [dB] |
| --- | --- | --- |
| CA_n20-n75 | n20 | 0.3 |

Table 6.x.1.4-2: ΔRIB

| Inter-band CA Configuration | NR Band | ΔRIB [dB] |
| --- | --- | --- |
| CA_n20-n75 | n20 | 0 |
|  | n75 | 0 |

6.x.1.5	REFSENs requirements

There is no additional MSD requirement for CA_n20-n75. But the Reference sensitivity for CA_n20-n75 which includes SDL band is listed below.

Table 6.x.1.5-1: Reference sensitivity for SDL bands

| NR Band/Channel bandwidth |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| NR CA Configuration | NR band | SCS (kHz) | 5 MHz | 10 MHz | 15 MHz | 20 MHz | 25 MHz | 30 MHz | 40 MHz | 50 MHz | 60 MHz | 80 MHz | 90 MHz | 100 MHz |
|  |  |  | dB | dB | dB | dB | dB | dB | dB | dB | dB | dB | dB | dB |
| CA_n20A-n75A | n20 | 15 | -97.0 | -93.8 | -91.0 | -89.8 |  |  |  |  |  |  |  |  |
|  |  | 30 |  | -94.1 | -91.1 | -90.0 |  |  |  |  |  |  |  |  |
|  |  | 60 |  |  |  |  |  |  |  |  |  |  |  |  |
|  | n75 | 15 | -100 | -96.8 | -95.0 | -93.8 |  |  |  |  |  |  |  |  |
|  |  | 30 |  | -97.1 | -95.1 | -94.0 |  |  |  |  |  |  |  |  |
|  |  | 60 |  | -97.5 | -95.4 | -94.2 |  |  |  |  |  |  |  |  |
| NOTE 1: The transmitter shall be set to PUMAX, as defined in subclause 6.2.4. NOTE 2: Four Rx antenna ports shall be the baseline for this operating band, except for two Rx vehicular UE. |  |  |  |  |  |  |  |  |  |  |  |  |  |  |

---End of changes---

Reference

[1]	RP-192615, “Revised WID on Rel-16 NR Inter-band Carrier Aggregation/Dual Connectivity  for 2 bands DL with x bands UL (x=1,2)”, ZTE Corporation