3GPP TSG-RAN WG4 Meeting #94-e	R4-2001194

Electronic Meeting, February 24 – March 6, 2020

Agenda item:	8.18.2.1	30% TP test point	[NR_perf_enh-Perf]

Source: 		NTT DOCOMO, INC.

Title: 	Views on 30% TP test point for BS demodulation

Document for:	Approval

- Introduction

In RAN4 #93, some parameters and configurations for 30% TP test point PUSCH requirements were agreed [1]. However, no consensus has been reached on the FR2 additional DM-RS configurations and PT-RS configurations. In this contribution, we provide our views to resolve these remaining open issues.

2.	Discussion

2.1	Previous agreement

In the last meeting, the following agreements were made in the chairman’s note [1].

RAN #93

- 30% TP test point

  - Antenna for FR1: 1T2R

  - Use MCS 16 instead of MCS 2 for full PRB allocation.

  - Waveform: only CP-OFDM for MCS 16

  - FR2 DM-RS configuration

    - Option 1: 1+1 and 1+0

    - Option 2: 1+1

    - Option 3: 1+0

  - FR2 PT-RS configuration

    - Option 1: with and without PT-RS

    - Option 2: with PT-RS

    - Option 3: without PT-RS

  - Channel Model:

    - FR1: TDLC300-100 (MCS 16)

    - FR2:TDLA30-300 (MCS 16)

  - Aim to finalize all the test requirements in the next meeting.

2.2	FR2 DM-RS configuration

The applicability rules for FR2 additional DM-RS configuration and the description of the manufacture’s declaration are specified as below.

8.1.2.1.3, TS 38.141-2 [2]

Unless otherwise stated, for BS type 2-O, PUSCH requirement tests shall apply only for the additional DM-RS position declared to be supported (see D.101 in table 4.6-1). If both options (i.e., pos0 and pos1) are declared to be supported, the tests shall be done for pos1.

4.6, TS 38.141-2 [2]

Table 4.6-1 Manufacturers declarations for BS type 1-H, BS type 1-O and BS type 2-O radiated test requirements

| Declaration identifier | Declaration | Description | Applicability (Note 1) |  |  |
| --- | --- | --- | --- | --- | --- |
|  |  |  | BS type 1-H (Note 2) | BS type 1-O | BS type 2-O |
| --- Omitted --- |  |  |  |  |  |
| D.101 | PUSCH additional DM-RS positions | Declaration of the supported additional DM-RS position(s) for FR2, i.e., pos0, pos1, or both. | n/a | n/a | x |
| --- Omitted --- |  |  |  |  |  |

According to the above applicability rule, only one additional DM-RS position from declared supported positions by manufacture will be used for the test. For additional DM-RS positions, “pos0”, “pos1” or “both (i.e., “pos0” and “pos1”)” can be declared. Table 1 shows additional DM-RS positions that are tested in each option according to the above rules and declarations.

Table 1: PUSCH additional DM-RS position tested for each option

| Declared supported PUSCH DM-RS position | Tested PUSCH DM-RS position |  |  |
| --- | --- | --- | --- |
|  | Option 1 (Specify requirements with both pos0 and pos1) | Option 2 (Specify requirements with pos1) | Option 3 (Specify requirements with pos0) |
| “pos0” | “pos0” | No test | “pos0” |
| “pos0” | “pos1” | “pos1” | No test |
| “Both” (i.e., pos0 and pos1) | “pos1” | “pos1” | No test |

According to the applicability rule, if the BS supports the PUSCH DM-RS position only with "pos0”, the test shall be performed with "pos0”. However, if option 2 is adopted, the test case may be skipped since "pos0” test case is not specified. Similarly, if the BS supports the PUSCH additional DM-RS position with “pos1”' or "both”, the test shall be performed with “pos1”. However, if option 3 is adopted, the test case may be skipped since “pos1” test case is not specified.

From the above, if Option 2 or Option 3 is adopted, the PUSCH tests at 30% throughput test point may be skipped in some cases. To avoid skipping the test, RAN4 needs both “pos0” and “pos1” test case. Therefore, we would like to propose to adopt option 1 (i.e., 1+1 and 1+0).

Proposal 1: For FR2 additional DM-RS configuration, RAN4 adopts option 1 (i.e., 1+1 and 1+0).

2.3	FR2 PT-RS configuration

The applicability rules for FR2 PT-RS configuration and the description of the manufacture’s declaration are specified as below.

8.1.2.1.3, TS 38.141-2 [2]

Unless otherwise stated, for BS type 2-O, PUSCH requirement tests with transform precoding disabled shall apply for the PT-RS option declared to be supported (see D.106 in table 4.6-1). If both PT-RS options (without and with PT-RS) are declared to be supported, the tests shall be done for either without or with PT-RS only; the same chosen option shall then be used for all tests.

4.6, TS 38.141-2 [2]

Table 4.6-1 Manufacturers declarations for BS type 1-H, BS type 1-O and BS type 2-O radiated test requirements

| Declaration identifier | Declaration | Description | Applicability (Note 1) |  |  |
| --- | --- | --- | --- | --- | --- |
|  |  |  | BS type 1-H (Note 2) | BS type 1-O | BS type 2-O |
| --- Omitted --- |  |  |  |  |  |
| D.106 | PUSCH PT-RS | Declaration of PT-RS in PUSCH support: without PT-RS, with PT-RS or both. | n/a | n/a | x |
| --- Omitted --- |  |  |  |  |  |

According to the above applicability rule, only one PT-RS position from declared positions by manufacture will be used for the test. For candidate support options for PT-RS positions, “without PT-RS”, “with PT-RS” or “both” can be declared. Table 2 shows PT-RS positions that are tested in each option according to the above rules and declarations.

Table 2: PUSCH PT-RS configuration tested for each option

| Declared supported PUSCH PT-RS position | Tested PUSCH PT-RS position |  |  |
| --- | --- | --- | --- |
|  | Option 1 (Specify requirements with “without PT-RS” and “with PT-RS”) | Option 2 (Specify requirements with “with PT-RS”) | Option 3 (Specify requirements with “without PT-RS”) |
| “without PT-RS” | “without PT-RS” | No test | “without PT-RS” |
| “with PT-RS” | “with PT-RS” | “with PT-RS” | No test |
| “Both” (i.e., “without PT-RS” and “with PT-RS”) | Either “without PT-RS” or “with PT-RS” | “with PT-RS” | “without PT-RS” |

According to the applicability rule, if the BS supports the PUSCH PT-RS position only with "without PT-RS", the test shall be performed with "without PT-RS". However, if option 2 is adopted, the test case may be skipped since "without PT-RS" test case is not specified. Similarly, if the BS supports the PUSCH PT-RS position only with "with PT-RS", the test shall be performed with "with PT-RS". However, if option 3 is adopted, the test case may be skipped since "with PT-RS" test case is not specified.

From the above, if Option 2 or Option 3 is adopted, the PUSCH tests at 30% throughput test point may be skipped in some cases. To avoid skipping the test, RAN4 needs both “without PT-RS” and “with PT-RS” test cases. Therefore, we would like to propose to adopt option 1 (i.e., with and without PT-RS).

Proposal 2: For FR2 PT-RS configuration, RAN4 adopts option 1 (i.e., with and without PT-RS).

2.4	How to incorporate the requirements

In RAN4 #92bis, for PUSCH requirements with 30% TP test point, it was agreed to introduce requirements with only minimum channel bandwidth for each SCS as below [3].

Bandwidth/SCS:

- the minimal channel bandwidth per SCS (5MHz CBW/15kHz SCS, 10MHz CBW/30kHz SCS, 50MHz CBW/60kHz SCS, 50MHz CBW/120kHz SCS)

Regarding how to incorporate the requirements, the following candidate approaches can be considered:

- Option 1: Add the 30% TP test case with minimum channel bandwidth to tables for

      - 5MHz for 15kHz SCS

      - 10MHz for 30kHz SCS

      - 50MHz for 60kHz SCS

      - 50MHz for 120kHz SCS

- Option 2: Add the 30% TP test case with minimum channel bandwidth to tables for

      - 5/10/20MHz for 15kHz SCS

      - 10/20/40/100MHz for 30kHz SCS

      - 50/100MHz for 60kHz SCS

      - 50/100/200MHz for 120kHz

According to the applicability rule specified in 8.1.2.1.2, TS38.141-2 [2], “Unless otherwise stated, for each subcarrier spacing declared to be supported, the tests shall be done only for the widest supported channel bandwidth. If performance requirement is not specified for this widest supported channel bandwidth, the tests shall be done by using performance requirement for the closest channel bandwidth lower than this widest supported bandwidth; the tested PRBs shall then be centered in this widest supported channel bandwidth.”, in other words, if there is no table for widest supported channel bandwidth, refer to the table for the closest channel bandwidth lower than this widest supported channel bandwidth.

Taking into account of the above applicability rule, if option 1 is adopted, there is a risk to miss the new test case for 30% TP test point since the table for minimum channel bandwidth may not be referred in some cases according to the description of the existing applicability rule. E.g., when the BS supports only 80MHz CBW and 30kHz SCS, the requirement of the table for 40MHz CBW and 30kHz SCS is referred, and there is no test case for 30% TP test point in the table.

In order to follow the existing applicability rule for CBW, it is suitable to adopt option 2. Therefore, RAN4 should add the test cases to all general PUSCH requirement tables.

Proposal 3: Add the 30% TP test case with RBs for minimum channel bandwidth to the table for

      - 5/10/20MHz for 15kHz SCS

      - 10/20/40/100MHz for 30kHz SCS

      - 50/100MHz for 60kHz SCS

      - 50/100/200MHz for 120kHz

Text examples to incorporate previous agreements and the above proposals are shown below. NOTE: Only examples for “100MHz CBW and 30kHz SCS” and “100MHz CBW and 120kHz SCS” tables in TS 38.104. The same approach can be used for other general PUSCH requirement tables.

TS 38.104:

Table 8.2.1.2-7: Minimum requirements for PUSCH, Type A, 100 MHz channel bandwidth, 30 kHz SCS

| Number of TX antennas | Number of RX antennas | Cyclic prefix | Propagation conditions and correlation matrix (Annex G) | Fraction of maximum throughput | FRC (Annex A) | Additional DM-RS position | SNR (dB) |
| --- | --- | --- | --- | --- | --- | --- | --- |
| 1 | 2 | Normal | TDLB100-400 Low | 70 % | G-FR1-A4-11 | pos1 | TBD |
|  |  | Normal | TDLB100-400 Low | 70 % | G-FR1-A3-14 | pos1 | -2.8 |
|  |  | Normal | TDLC300-100 Low | 70 % | G-FR1-A4-14 | pos1 | 10.2 |
|  |  | Normal | TDLA30-10 Low | 70 % | G-FR1-A5-14 | pos1 | 13.0 |
|  | 4 | Normal | TDLB100-400 Low | 70 % | G-FR1-A3-14 | pos1 | -5.8 |
|  |  | Normal | TDLC300-100 Low | 70 % | G-FR1-A4-14 | pos1 | 6.5 |
|  |  | Normal | TDLA30-10 Low | 70 % | G-FR1-A5-14 | pos1 | 9.0 |
|  | 8 | Normal | TDLB100-400 Low | 70 % | G-FR1-A3-14 | pos1 | -8.7 |
|  |  | Normal | TDLC300-100 Low | 70 % | G-FR1-A4-14 | pos1 | 3.2 |
|  |  | Normal | TDLA30-10 Low | 70 % | G-FR1-A5-14 | pos1 | 5.8 |
| 2 | 2 | Normal | TDLB100-400 Low | 70 % | G-FR1-A3-28 | pos1 | 1.4 |
|  |  | Normal | TDLC300-100 Low | 70 % | G-FR1-A4-28 | pos1 | 19.2 |
|  | 4 | Normal | TDLB100-400 Low | 70 % | G-FR1-A3-28 | pos1 | -2.2 |
|  |  | Normal | TDLC300-100 Low | 70 % | G-FR1-A4-28 | pos1 | 11.6 |
|  | 8 | Normal | TDLB100-400 Low | 70 % | G-FR1-A3-28 | pos1 | -5.2 |
|  |  | Normal | TDLC300-100 Low | 70 % | G-FR1-A4-28 | pos1 | 7.1 |

Table 11.2.2.1.2-4: Minimum requirements for PUSCH, 100 MHz channel bandwidth, 120 kHz SCS

| Number of TX antennas | Number of demodulation branches | Cyclic prefix | Propagation conditions and correlation matrix (Annex G) | Fraction of maximum throughput | FRC (Annex A) | Additional DM-RS position | PT-RS | SNR (dB) |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 1 | 2 | Normal | TDLA30-300 Low | 70 % | G-FR2-A3-4 | pos0 | No | -2.4 |
|  |  |  |  |  | G-FR2-A3-16 | pos1 | No | -2.5 |
|  |  | Normal | TDLA30-300 Low | 70 % | G-FR2-A4-3 | pos0 | Yes | TBD |
|  |  |  |  |  |  |  | No | TBD |
|  |  |  |  |  | G-FR2-A4-13 | pos1 | Yes | TBD |
|  |  |  |  |  |  |  | No | TBD |
|  |  | Normal | TDLA30-300 Low | 70 % | G-FR2-A4-4 | pos0 | Yes | 11.9 |
|  |  |  |  |  |  |  | No | 10.5 |
|  |  |  |  |  | G-FR2-A4-14 | pos1 | Yes | 11.1 |
|  |  |  |  |  |  |  | No | 10.5 |
|  |  | Normal | TDLA30-75 Low | 70 % | G-FR2-A5-4 | pos0 | Yes | 13.5 |
|  |  |  |  |  |  |  | No | 12.9 |
|  |  |  |  |  | G-FR2-A5-9 | pos1 | Yes | 13.4 |
|  |  |  |  |  |  |  | No | 12.8 |
| 2 |  | Normal | TDLA30-300 Low | 70 % | G-FR2-A3-9 | pos0 | No | 1.4 |
|  |  |  |  |  | G-FR21-A3-21 | pos1 | No | 1.2 |
|  |  | Normal | TDLA30-300 Low | 70 % | G-FR2-A4-9 | pos0 | Yes | 20.8 |
|  |  |  |  |  |  |  | No | 19.4 |
|  |  |  |  |  | G-FR2-A4-19 | pos1 | Yes | 18.5 |
|  |  |  |  |  |  |  | No | 18.0 |

3.	Conclusion

In this contribution, we discuss on PUSCH 30% TP test point. The following proposals are obtained.

Proposal 1: For FR2 additional DM-RS configuration, RAN4 adopts option 1 (i.e., 1+1 and 1+0).

Proposal 2: For FR2 PT-RS configuration, RAN4 adopts option 1 (i.e., with and without PT-RS).

Proposal 3: Add the 30% TP test case with RBs for minimum channel bandwidth to the table for

      - 5/10/20MHz for 15kHz SCS

      - 10/20/40/100MHz for 30kHz SCS

      - 50/100MHz for 60kHz SCS

      - 50/100/200MHz for 120kHz

References

- RAN4 #93 Meeting report

- TS 38.141-2 v16.2.0: NR; Base Station (BS) conformance testing Part 2: Radiated conformance testing, MCC, January 2020

- R4-1912669, “Ad Hoc minutes for NR Rel-15 and Rel-16 BS demodulation”, Huawei, HiSilicon, RAN #92bis, October 2019