3GPP TSG-RAN WG4 Meeting #94-e	R4-2001215

Online, 24th Feb. – 06th March, 2020

Source:	LG Electronics

Agenda Item:	8.4.1

Title: 	Summary of e-mail discussion on NR V2X UE basic RF parameters and MPR simulation assumptions

Document for:	Approval

Attendances: LGE, Huawei, Intel, Ericsson, Qualcomm, MediaTek, CATT, CMCC, Samsung, Futurewei, Vodafoen, AT&T, DISH …

1	Agenda of NR V2X service in rel-16

1) NR V2X UE basic RF parameters

2) MPR detail simulation assumptions for PSSCH/PSCCH transmission

3) MPR detail simulation assumptions for simultaneous PSFCH transmission

4) Work split for Draft CRs for NR V2X UE

- 38.101-1

- CR on UE system parameters for NR V2X UE

- CR on UE RF core requirements for n38 or n47 SL operation

- CR on UE RF core requirements TDM operation between LTE SL and NR SL at n47

- 38.101-3

- CR on UE RF core requirements for con-current operation for NR V2X UE

- NR V2X UE basic RF parameters for NR V2X

- NR V2X UE RF parameters

  - Baseline antenna configuration

- Option1: 1Tx/2Rx in both licensed band and n47

  - 2Rx antenna and 2 recevier chain each LTE V2X and NR V2X

- Option2: 2Tx/2Rx in both licensed band and n47

- Option3: 2Tx/4Rx in both licensed band and n47

  - Supporting UL-MIMO or Tx diversity

- Option1 : Support from rel-16

- Option2 : Support from rel-17

  - UE capability on the same/different RF and modem Tx chain at n47

- Option1 : Do not specify the UE capability.

  - The default is different RF Tx chain for inter-band con-current operation

  - LTE SL and NR SL at n47 has depend on UE implementation whether or share/different the RF Tx chain

- Option2 : Specify the UE capability on the same/different RF and modem Tx cahin

Based on agreements , WF on NR V2X UE baseline RF parameters will be requested

2-1 Company preference for Baseline antenna configuration

[Editor Note] each company can provide their view in here

- LG preperence

  - Baseline antenna configuration

- Option1: 1Tx/2Rx in both licensed band and n47 ( and licensed bands)

- Huawei preperence

- Option1: 1Tx/2Rx in both licensed band and n47 for NR-V2X to derive the baseline requirements

- CATT preference

  - Baseline antenna configuration

- Option1: 1Tx/2Rx in both licensed band and n47 (and licensed bands)

- QCOM preference

  - Baseline antenna configuration

- Option1: 1Tx/2Rx in both licensed band and n47

2.2 Company preference for supporting UL-MIMO or Tx diversity in rel-16

[Editor Note] each company can provide their view in here

- LG preperence

  - Supporting UL-MIMO or Tx diversity

- Option2: Support from rel-17

- Huawei preperence

  - Supporting UL-MIMO or Tx diversity

- Option1: Support from Rel-16 (contributions will be prepared for RAN4#94)

- CATT preference

  - Supporting UL-MIMO or Tx diversity

- Option2: Support from rel-17

- QCOM preference

  - Supporting UL-MIMO or Tx diversity

- Option1 : Support of TX diversity and UL-MIMO from Rel-16 is optional

2.3 Company preference for UE capability for UE RF and modem Tx chain at n47

[Editor Note] each company can provide their view in here

- LG preperence

  - UE capability on the same/different RF and modem Tx chain at n47

- Option1 : Do not specify the UE capability.

- Huawei preperence

  - UE capability on the same/different RF and modem Tx chain at n47

- Option1 : Do not specify the UE capability.

- CATT preference

  - UE capability on the same/different RF and modem Tx chain at n47

- Option1 : Do not specify the UE capability.

- QCOM preference

- Option1 : Do not specify the UE capability

2.4 Other issues for baseline UE RF parameters

[Editor note] each company can provide their view in here

Agreement

2-1 Company preference for Baseline antenna configuration

- All interested companies prefer the baseline antenna configuration as follow

- Baseline antenna configuration

  - 1Tx/2Rx in both licensed band and n47 (and licensed bands)

2.2 Company preference for supporting UL-MIMO or Tx diversity in rel-16

- LG, CATT: Support from rel-17

- Huawei: Support from rel-16

- QC : 	Support from rel-16 as optional feature

- RAN4 need further input from Operators and Vendors and it will be decided in next RAN4 meeting (RAN4 #94 meeting) based on standard progress.

2.3 Company preference for UE capability for UE RF and modem Tx chain at n47

- All interested companies prefer not to specify UE capability according to UE RF and modem Tx chain

- MPR detail simulation assumptions for PSSCH/PSCCH transmission

Table 3.1: MPR simulation assumption for NR V2X

| parameter | Assumption |
| --- | --- |
| center frequency | 2.7GHz/5.9GHz |
| Bandwidth | 10/20/30/40MHz |
| Maximum output power | 23dBm |
| numerology | 15 kHz/30kHz/60kHz |
| Modulation | QPSK/16QAM/64QAM/256QAM |
| Waveform | CP-OFDM |
| Carrier leakage | 25dBc |
| IQ image | 25dBc |
| CIM3 | 45 or 60 dBc |
| PA calibration | PA calibrated to deliver -30dBc ACLR for a fully allocated RB in 20MHz QPSK DFT-S-OFDM waveform at 1 dB MPR. This is based on assumption to share PA between LTE V2X and NR V2X at 5.9GHz as worst case. |

For NR V2X, simultaneous transmission of PSCCH and PSSCH in the same subframe is supported. The following constraints in table 8.1.2-2 can be assumed based on current RAN1’s agreement.

Table 3.2: MPR simulation assumption based on RAN1’s agreement

| Items | Assumption |
| --- | --- |
| Allowed sub-channel sizes | • Support {10, 15, 20, 25, 50, 75, 100} PRBs for possible sub-channel size. |
| Allowed LCRB allocation | 10,15,20,25,30,40,45,50,60,70,75,80,90,100,105,110,120,130,135,140,150,160,165,170,175,180,190,195,200,210 |
| Regarding PSCCH / PSSCH multiplexing |  |
| PSCCH size | 10RB*3symbol |
| PSD offset of X dB between PSCCH and PSSCH | 0dB |

Discussion:

[Editor Note]: Each company can propose the detail MPR simulation assumptions based on RAN1 agreements

- LG proposal

- Change CIM3 with from -45dBc to -60dBc

- Only consider multiplexing PSCCH/PSSCH in subslot with 10RBs

- PSD offset 0dB between PSCCH and PSSCH to prevent transient period

- For PA calibration, propose to meet -30dBc ACLR for 20MHz QPSK DFT-S-OFDM waveform with full RBs allocation at 1dB MPR

- Huawei proposal

- Keep CIM3 of -45dBc

- LCRB as integer multiple of sub-channel size based on RAN1’s agreement.

- 0dB PSD offset.

- Propose to use 10RB*3symbol as PSCCH size.

- For PA calibration, propose to meet -30dBc ACLR for 20MHz QPSK CP-OFDM waveform with 105RB allocation at 3dB MPR.

- QCOM proposal

- Change CIM3 with from -45dBc to -60dBc

- LCRB selected as integer multiple of the sub-channel size

- PSD offset 0dB between PSCCH, PSSCH and DMRS

- For PA calibration, propose waveform with SCS=15kHz, BW=20MHz, full RB, QPSK DFTS waveform at 1dB MPR to meet all specs.

Agreement

MPR for PSCCH/PSSCH

- Above revised Table 3.1 and 3.2 will be used to evaluate MPR simulation.

- More detail simulation parameters will be further discussed in next RAN4 meeting

- MPR detail simulation assumptions for simultaneous PSFCH transmission

In last RAN4 #93 meeting, we discussed the basic simulation assumption of the required MPR levels to decide the number of simultaneous PSFCH transmission user.

Firstly, RAN4 make common assumptions for PSFCH transmission which also allow non-contiguous RB allocation or just allow contiguous RB transmision.

Second is that number of PSFCH RB size for single user.

Third is detail parameters which could reuse the common simulation assumption for PSSCH/PSCCH in Table 8.1.2-1 in TR38.886 v0.5.0.

And if needed, RAN4 decide the power difference between power of #1 PSFCH and power of #2 PSFCH and others.

Discussion:

[Editor Note]: Each company can propose the detail MPR simulation assumptions based on RAN1 agreements

- QC proposals

- MPR Simulations to be done as follows:

  - 1 RB per user

  - Up to five (N≤5) simultaneous users placed randomly over a 40MHz spectrum

  - Total power of all users equals 23dBm

  - All users have the same power per RB

  - Simulation parameters as per table 8.1.2-1 in TR 38.886

  - Feedback sequences are per Section 5.2 TS 38.211

  - The MPR will be specified as a formula based on number of allocated RBs.

- LGE proposals

  - 1 RB per user

  - Assume up to five (N≤5) simultaneous users (The number of simultaneous transmission user shall be decided by MPR simulation results)

  - Both Non-contiguous PSFCH RB allocation and contiguous PSFCH allocation are allowed

- MPR will be derived by non-contiguous PSFCH RB allocation

  - Channel BW: 10/20/30/40MHz

  - Total power of all users equals 23dBm

  - All users have the same power per RB

  - Simulation parameters as per table 8.1.2-1 in TR 38.886

  - Feedback sequences are per Section 5.2 TS 38.211

  - The MPR will be specified as a formula or table based on number of allocated RBs.

- HW proposals

  - Both non-contiguous and contiguous PSFCH RB allocation are considered

  - PSFCH generation shall be based on existing RAN1 agreements

  - Assumption of N in RAN4 is only for MPR simulation purpose, the final number is up to RAN1 decision

Agreement

MPR for multiple PSFCH

- Baseline MPR simualtion parameters

  - 1 RB per user

  - Both Non-contiguous PSFCH RB allocation and contiguous PSFCH allocation are allowed

    - MPR will be derived by non-contiguous PSFCH RB allocation

  - Total power of all users equals 23dBm

  - All users have the same power per RB

  - Simulation parameters as per Table 3.1 and 3.2 in this document

  - Feedback sequences are per Section 5.2 TS 38.211

  - The MPR will be specified as a formula or table based on number of allocated RBs.

  - PSFCH generation shall be based on existing RAN1 agreements

  - Assumption of N in RAN4 is only for MPR simulation purpose, the final number is up to RAN1 decision

- Work split on Draft CRs for NR V2X UE

Required CRs to complete NR V2X Core WI in rel-16.

Candidate Company request to CR work and it will be shared in e-mail reflectors until 10th/Feb./2020

- 38.101-1

- CR on UE system parameters for NR V2X UE (Vivo/CATT)

- CR on UE RF core requirements for n38 or n47 SL operation

  - Tx Core requirements (LGE)

  - Rx Core requirements (CATT)

- CR on UE RF core requirements TDM operation between LTE SL and NR SL at n47 (LGE)

- 38.101-3

- CR on UE RF core requirements for con-current operation for NR V2X UE

  - Tx core requirements (Huawei)

  - Rx core requirements (CATT)

- 38.104

- CR on adding frequency bands for NR V2X (CATT)

LGE view:

- Co-rapporteur can be split the CR works for NR V2X in rel-16.

- Also candidate company also join to make draft CR work

- Initially LG propose split CR work as above

HW view:

- OK with the work split.

- We will also prepare CR for NR V2X supporting 2Tx as well as PC2

CATT view:

- Prefer to split UE RF core requirements into Tx requirements and Rx requirements

Agreement

Work split for CRs: Draft CRs will be shared in RAN4 email thread until 10th/ Feb. 2020

- 38.101-1

- CR on UE system parameters for NR V2X UE (Vivo)

- CR on UE RF core requirements for n38 or n47 SL operation

  - Tx Core requirements (LGE)

  - Rx Core requirements (CATT)

- CR on UE RF core requirements TDM operation between LTE SL and NR SL at n47 (LGE)

- 38.101-3

- CR on UE RF core requirements for con-current operation for NR V2X UE

  - Tx core requirements (Huawei)

  - Rx core requirements (CATT)

- 38.104

- CR on adding frequency bands for NR V2X (CATT)

Draft CR for 2Tx and HPUE could be shared in the reflector for review by interested companies. Whether or not to specify RF core requirements of 2Tx and HPUE will be decided in next RAN4 meeting.