3GPP TSG-RAN WG4 Meeting #94-e	R4-2001226

Online, 24th February – 6th March 2020

Agenda Item:	9.18.1

Source: 	MediaTek Inc.

Title: 	n28 supporting 30MHz REFSENS evaluation

Document for:	Approval

- 1. Introduction

A new WID in [1] has been approved in RAN_P#85. UE is going to support n28 with 30MHz channel bandwidth. Based on the previous discussion and WF[2][3], the Tdoc in RAN4#93[4] characterize REFSENS and UL configuration. Unfortunately REFSENS was not concluded in last meeting. The contribution is resubmission for continue discussion in this meeting.

- 2. Discussion

According to WF[3], REFSENS are characterized with same configuration and two cases below:

- Evaluate two cases in band n28 with same UL allocation (assumption:  25RB for 15KHz SCS, 10RB for 30KHz SCS).

  - Case 1: asymmetric channel bandwidth (UL 20MHz/DL 30MHz)

  - Case 2: symmetric 30 MHz channel bandwidth

- For uplink configuration and assumptions, we follow those for MPR/AMPR characterization:

  - PA calibrated to deliver -30dBc ACLR for a fully allocated QPSK DFT-S-OFDM waveform at 1 dB MPR,

  - -28 dBc image and carrier rejection,

  - 30 MHz channel bandwidth located at the lowermost channel (703~733) (Center frequency is 718 MHz.)

  - CIM3: 60 dBc

- As for CIM5 assumption we use 70dB in our calculation

Case 1 REFSENS

The case 1 REFSENS had been characterized in [2] and results are copied below, same calculation assumptions for case 2 are listed in table 2.1-1 and REFSENS analysis is in table

| Parameter | Value | Unit |
| --- | --- | --- |
| Antenna isolation | 10 | dB |
| n28 filter rejection from TX to RX | 55 | dB |
| n28 diversity receiver filter TX rejection | 45 | dB |
| Front-end loss | 4 | dB |
| Thermal noise at n28 RX ANT port | -165 | dBm/Hz |
| Transceiver effective phase noise due to wider BW | -144 | dBc/Hz |
| SNR requirement for QPSK | -1 | dB |

Table 2.1-1 Typical receiver performance parameters for MSD analysis

Proposal 1: Case 1 REFSENS for n28, asymmetric channel bandwidth (UL 20MHz/DL 30MHz) is proposed highlighted as below:

Table 7.3.2-1: Two antenna port reference sensitivity QPSK PREFSENS for asymmetric channel bandwidth

| Operating band / SCS / Channel bandwidth / Duplex-mode |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Operating Band | SCS kHz | 5 MHz (dBm) | 10 MHz (dBm) | 15 MHz (dBm) | 20 MHz (dBm) | 25 MHz (dBm) | 30 MHz (dBm) | 40 MHz (dBm) | 50 MHz (dBm) | 60 MHz (dBm) | 80 MHz (dBm) | 90 MHz (dBm) | 100 MHz (dBm) | Duplex Mode |
| n28 | 15 | -98.5 | -95.5 | -93.5 | -90.8 |  | -88.2 |  |  |  |  |  |  | FDD |
|  | 30 |  | -95.6 | -93.6 | -91.0 |  | -88.3 |  |  |  |  |  |  |  |

Table 7.3.2-3: Uplink configuration for reference sensitivity

| Operating band / SCS / Channel bandwidth / Duplex mode |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Operating Band | SCS kHz | 5 MHz | 10 MHz | 15 MHz | 20 MHz | 25 MHz | 30 MHz | 40 MHz | 50 MHz | 60 MHz | 80 MHz | 90 MHz | 100 MHz | Duplex Mode |
| n28 | 15 | 25 | 251 | 251 | 251 |  | 251 |  |  |  |  |  |  | FDD |
|  | 30 |  | 101 | 101 | 101 |  | 101 |  |  |  |  |  |  |  |

Case 2 REFSENS

Then we characterize case 2 REFSENS when symmetric uplink/downlink bandwidth equals 30MHz with same assumptions. First, we do the frequency analysis for IM3 of wanted UL and its image, with 25RB, 15KHz SCS. If we set UL RB to uppermost edge, IM3 will fall about 748~761.5MHz while DL is 758~788MHz. CIM5 will fall in own RX range at the same time while CIM3 does not. When we set UL RB to lowermost edge, CIM3 will fall in own RX range while IM3 of wanted UL and its image, and CIM5 are not. Then we do the calculation for 30MHz REFSENS following same procedure as case 1 did. With mesurement’s help, total level of “IM3 of wanted and its image” from PA output port is -17.5dBm and overlapping BW correction ratio is -5.9dB. Calculated REFSENS for case 2 is below:

Proposal 2: Case 2 REFSENS for n28, symmetric channel bandwidth (UL 30MHz/DL 30MHz) is proposed highlighted as below:

Table 7.3.2-1: Two antenna port reference sensitivity QPSK PREFSENS for symmetric channel bandwidth

| Operating band / SCS / Channel bandwidth / Duplex-mode |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Operating Band | SCS kHz | 5 MHz (dBm) | 10 MHz (dBm) | 15 MHz (dBm) | 20 MHz (dBm) | 25 MHz (dBm) | 30 MHz (dBm) | 40 MHz (dBm) | 50 MHz (dBm) | 60 MHz (dBm) | 80 MHz (dBm) | 90 MHz (dBm) | 100 MHz (dBm) | Duplex Mode |
| n28 | 15 | -98.5 | -95.5 | -93.5 | -90.8 |  | -79.9 |  |  |  |  |  |  | FDD |
|  | 30 |  | -95.6 | -93.6 | -91.0 |  | -80 |  |  |  |  |  |  |  |

Table 7.3.2-3: Uplink configuration for reference sensitivity

| Operating band / SCS / Channel bandwidth / Duplex mode |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Operating Band | SCS kHz | 5 MHz | 10 MHz | 15 MHz | 20 MHz | 25 MHz | 30 MHz | 40 MHz | 50 MHz | 60 MHz | 80 MHz | 90 MHz | 100 MHz | Duplex Mode |
| n28 | 15 | 25 | 251 | 251 | 251 |  | 251 |  |  |  |  |  |  | FDD |
|  | 30 |  | 101 | 101 | 101 |  | 101 |  |  |  |  |  |  |  |

Reference:

- RP-192261 New WI proposal: Addition of wider channel bandwidth in NR band n28, TSG-RAN Meeting #85

- R4-1911613 n28 supporting 30MHz, RAN4#92-bis

- R4-1912979 WF for 30MHz channel bandwidth for n28, RAN4#92-bis

- R4-1914190 n28 supporting 30MHz REFSENS evaluation