3GPP TSG-RAN WG4 Meeting #94-e	R4-2001255

Electronic Meeting, 24 Feb. – 6 Mar., 2020

Source: 	ZTE

Title:	TP to TR on IAB EMC emission requirement

Agenda Item:	8.5.6

Document for:	Approval

Introduction

The TR skeleton of IAB back ground has been approved in [1]. The IAB EMC emission requirements are discussed in [2]. This TP to TR is to capture the emission requirements part of the IAB EMC discussion.

Discussion

.

Conclusion

The TP to TR of IAB EMC emission requirements discussion has been provided. It is proposed to approve the following text proposal.

References

R4-1912889 TR skeleton, Samsung

R4-2001253 further discussion on IAB EMC emission requirements, ZTE

--------------Start of text proposal-------------

# 12	IAB EMC requirements

## 12.2	IAB EMC emission requirements.

12.2.1 Radiated emission

The radiated emission is the requirement that is to test the spurious emission radiating out of the base station besides the antenna connecotr, especially from the gap of the enclosure. Hence the requirement should apply per enclosure. In this case, an EUT with different enclosure should be tested separately. For IAB-DU function, the radiated emission limit should follow the base station requirement and for IAB-MT function, the radiated emission limit should follow the UE requirement.

However, if the DU and MT function are within one enclosure, as the enclosure as the reference point to define the requirement in this case, it is needed to separate the IAB-node into TDM and FDM/SDM as two different categories..

For a TDM IAB-node, the DU and MT functions separately in time domain so only one requirement should be met. Hence resue the TS 38.113[] requirement is quite reasonable.

However, when considering the specific requirement and the on-going RF architecture of IAB-node, it is still proposed the similar principle of NR BS radiated emission requirement that:

For type 1-C and 1-H TDM IAB-node, the radiated emission is the same as TS 38.113 radiated emission requirement.

For type 1-O and 2-O TDM IAB-node, the radiated emission will be covered by the OTA radiated spurious emission.

For FDM and SDM IAB-node, the DU and MT function may transmit at the same time. In this case, it is assumed two times radiated emission will come out of the IAB-node enclosure. While considering the spurious emission is always the regulatory requirement, it is hard to relax the requirement by 3dB to the regulators. Hence, it is proposed to keep the same radiated emission requirement for FDM and SDM IAB-node.

12.2.2 Conducted emission

The condcuted emission will apply based on different ports as table 12.2.2-1 below:

Table 12.2.2-1, conducted emission requirement

| Phenomenon | Application | Equipment test requirement |  | Reference subclause in the present document | Reference standard |
| --- | --- | --- | --- | --- | --- |
|  |  | BS equipment | Ancillary equipment |  |  |
| Conducted emission | DC power input/output port | applicable | applicable | 8.3 | CISPR 32 [11] |
| Conducted emission | AC mains input/output port | applicable | applicable | 8.4 | CISPR 32 [11] |
| Conducted emission | Telecommunication port | applicable | applicable | 8.5 | CISPR 32 [11] |

As shown in table 12.2.2-11 above, the conducted emission requirement are referred to CISPR 32[]. The CISPR has classified the equipment under test as two classes: Class A and Class B :

Equipment intended primarily for use in a residential environment shall meet the class B limits. All other equipment shall comply with class A limit.

In this case, as categorized in previous chapter, if the IAB-DU and IAB-MT function are implemented in different enclosure, then class A and class B limit will apply in case any of the enclosure are put into the residential environment.

12.2.3 Harmonic current emission and voltage fluctuation and flicker

These two requirements are based on IEC 61000 series and are all applicable to AC conducted power port. Hence for an IAB-node, the requirements are function independent. In this case, the requirement for base station shall apply.

--------------End of text proposal-------------