3GPP TSG-RAN WG4 Meeting #94-e	R4-2001263

# Online, 24th February – 6th March 2020

| CR-Form-v12.0 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.133 | CR | 0480 | rev |  | Current version: | 16.2.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | X | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | CR to 38.133 on CGI reading of NR cell |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | ZTE |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R4 |  |  |  |  |  |  |  |  |  |
| Work item code: | NR_RRM_Enh-Core |  |  |  |  |  | Date: |  |  | 2020-01-15 |
| Category: | B |  |  |  |  |  | Release: |  |  | Rel-16 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) Rel-12 (Release 12) Rel-13 (Release 13) Rel-14 (Release 14) Rel-15 (Release 15) Rel-16 (Release 16) |  |
| Reason for change: |  | In WID RP-191601, CGI reading requirements with autonomous gap for NR capable UE should be specified for various scenarios. - CGI reading for NR cell (NR capable UE in LTE SA, NR SA UE, EN-DC UE, NE-DC UE, and NR-DC) |  |  |  |  |  |  |  |  |
| Summary of change: |  | - Requirements for CGI identification of intra frequency NR cell are introduced. - Requirements for CGI identification of inter frequency NR cell are introduced. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | No requirments for CGI identification of an NR cell. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 9.2.7, 9,3.9 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | x | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | x | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

< Start of change #1 >

### 9.2.7	NR Intra frequency measurements with autonomous gaps

#### 9.2.7.1	Introduction

The requirements in this clause are applicable for CGI identification of an intra frequency NR target cell.

The requirements in this clause are specified for CGI identification of an NR target cell and are applicable for a UE:

-	in RRC_CONNECTED state, and

-	configured with SA or NR-DC or NE-DC operation mode.

The overall CGI reporting delay includes RRC procedure delay defined in clause 12 in TS 38.331 [2] and CGI reporting delay in clause 9.2.7.3.

#### 9.2.7.2	CGI identification of an NR cell with autonomous gaps

The UE shall identify and report the CGI of an NR target cell when requested by the network for the purpose of [‘reportCGI’]. Only one cell is provided to the UE with [cellForWhichToReportCGI] for identifying the CGI.The UE may make autonomous gaps in both downlink reception and uplink transmission for receiving MIB and SIB1 message according to clause 5.5.3 of TS 38.331 [2]. Note that a UE is not required to use autonomous gap if [high layer parameter] is set to false. If autonomous gaps are used for measurement with the purpose of [‘reportCGI’], regardless of whether DRX or eDRX_CONN is used or not, or whether SCell(s) are configured or not, the UE shall be able to identify a new CGI of NR cell within:

Tidentify_CGI_intra = (TMIB_intra + T SIB1_intra) ms

Where:

TMIB_intra is the time period used to acquire MIB message. TMIB_intra = [5 * TSMTC] ms.

TSIB1_intra is the time period used to acquire SIB1 message. TSIB1_intra = [TBD * TSMTC] ms.

An intra-frequency cell shall be considered detectable when following conditions for relevant SSB are fulfilled:

SS-RSRP related side conditions given in clauses 10.1.2 and 10.1.3 for FR1 and FR2, respectively, for a corresponding Band,

-	SSB_RP and SSB Ês/Iot according to Annex B.2.2 for a corresponding Band.

The requirement for identifying the CGI of an NR cell within Tidentify_CGI_intra is applicable when no DRX is used as well as when any of the DRX or eDRX_CONN cycles specified in TS 38.331 [2] is used.

Within the time Tidentify_CGI_intra, over which the UE identifies the CGI of an NR cell, the UE shall fulfil interruption requirements specified in,

-	Clause 8.2.1.2.8 and Clause [TBD] in TS36.133 [15] if the UE is configured with EN-DC operation mode,

-	Clause 8.2.2.2.7 if the UE is configured with SA operation mode,

-	Clause 8.2.3.2.8 and Clause [TBD] in TS36.133 [15] if the UE is configured with NE-DC operation mode,

-	Clause 8.2.4.2.8 if the UE is configured with NR-DC operation mode.

In the requirement a cell is known if it has been meeting the relevant intra frequency cell identification requirement during the last 5 seconds otherwise it is unknown. Relevant cell identification requirements are described in Clause 9.2.5.

#### 9.2.7.3	CGI reporting delay

The CGI reporting delay is defined as the time between a command that will trigger a CGI report and the point when the UE starts to transmit the measurement report over the air interface. This requirement assumes that the measurement report is not delayed by other RRC signalling on the DCCH. This measurement reporting delay excludes a delay uncertainty of 2 x TTIDCCH resulting when inserting the measurement report to the TTI of the uplink DCCH. This measurement reporting delay excludes any delay caused by lack of UL resources for UE to send the measurement report.

The CGI reporting delay shall be less than Tidentify_CGI_intra defined in clause 9.2.7.2.

< End of change #1 >

< Start of change #2 >

### 9.3.9	NR Inter frequency measurements with autonomous gaps

#### 9.3.9.1	Introduction

The requirements in this clause are specified for CGI identification of an inter frequency NR target cell and are applicable for a UE:

-	in RRC_CONNECTED state, and

-	configured with SA or NR-DC or NE-DC operation mode.

The overall CGI reporting delay includes RRC procedure delay defined in clause 12 in TS 38.331 [2] and CGI reporting delay in clause 9.3.9.3.

#### 9.3.9.2	CGI identification of an NR cell with autonomous gaps

The UE shall identify and report the CGI of an NR target cell when requested by the network for the purpose of [‘reportCGI’]. Only one cell is provided to the UE with [cellForWhichToReportCGI] for identifying the CGI.The UE may make autonomous gaps in both downlink reception and uplink transmission for receiving MIB and SIB1 message according to clause 5.5.3 of TS 38.331 [2]. Note that a UE is not required to use autonomous gap if [high layer parameter] is set to false. If autonomous gaps are used for measurement with the purpose of [‘reportCGI’], regardless of whether DRX or eDRX_CONN is used or not, or whether SCell(s) are configured or not, the UE shall be able to identify a new CGI of NR cell within:

Tidentify_CGI_inter = (TMIB_inter + T SIB1_inter) ms

Where:

TMIB_inter is the time period used to acquire MIB message. TMIB_inter = [5 * TSMTC] ms.

TSIB1_inter is the time period used to acquire SIB1 message. TSIB1_inter = [TBD * TSMTC] ms.

An inter-frequency cell shall be considered detectable when following conditions for relevant SSB are fulfilled:

SS-RSRP related side conditions given in clauses 10.1.4 and 10.1.5 for FR1 and FR2, respectively, for a corresponding Band,

-	SSB_RP and SSB Ês/Iot according to Annex B.2.3 for a corresponding Band.

The requirement for identifying the CGI of an NR cell within Tidentify_CGI_inter is applicable when no DRX is used as well as when any of the DRX or eDRX_CONN cycles specified in TS 38.331 [2] is used.

Within the time Tidentify_CGI_inter, over which the UE identifies the CGI of an NR cell, the UE shall fulfil interruption requirements specified in,

-	Clause 8.2.1.2.8 and Clause [TBD] in TS36.133 [15] if the UE is configured with EN-DC operation mode,

-	Clause 8.2.2.2.7 if the UE is configured with SA operation mode,

-	Clause 8.2.3.2.8 and Clause [TBD] in TS36.133 [15] if the UE is configured with NE-DC operation mode,

-	Clause 8.2.4.2.8 if the UE is configured with NR-DC operation mode.

In the requirement a cell is known if it has been meeting the relevant inter frequency cell identification requirement during the last 5 seconds otherwise it is unknown. Relevant cell identification requirements are described in Clause 9.3.4.

#### 9.3.9.3	CGI reporting delay

The CGI reporting delay is defined as the time between a command that will trigger a CGI report and the point when the UE starts to transmit the measurement report over the air interface. This requirement assumes that the measurement report is not delayed by other RRC signalling on the DCCH. This measurement reporting delay excludes a delay uncertainty of 2 x TTIDCCH resulting when inserting the measurement report to the TTI of the uplink DCCH. This measurement reporting delay excludes any delay caused by lack of UL resources for UE to send the measurement report.

The CGI reporting delay shall be less than Tidentify_CGI_inter defined in clause 9.3.9.2.

< End of change #2 >