3GPP TSG-RAN WG4 Meeting #94-e	R4-2001264

# Online, 24th February – 6th March 2020

| CR-Form-v12.0 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.133 | CR | 0481 | rev |  | Current version: | 16.2.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | X | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | CR to 38.133 on interruption requirements for CGI reading |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | ZTE |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R4 |  |  |  |  |  |  |  |  |  |
| Work item code: | NR_RRM_Enh-Core |  |  |  |  |  | Date: |  |  | 2020-01-15 |
| Category: | B |  |  |  |  |  | Release: |  |  | Rel-16 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) Rel-12 (Release 12) Rel-13 (Release 13) Rel-14 (Release 14) Rel-15 (Release 15) Rel-16 (Release 16) |  |
| Reason for change: |  | In WID RP-191601, CGI reading requirements with autonomous gap for NR capable UE should be specified for various scenarios. - CGI reading for NR cell (NR capable UE in LTE SA, NR SA UE, EN-DC UE, NE-DC UE, and NR-DC) |  |  |  |  |  |  |  |  |
| Summary of change: |  | - Interruption requirements for CGI identification of NR cell are introduced for EN-DC, NR SA, NE-DC and NR-DC. - Interruption requirements for CGI identification of LTE cell are introduced for EN-DC and NE-DC. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | No interruption requirments for CGI identification of an NR cell. No interruption requirments for CGI identification of an LTE cell. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 8.2.1.2.8, 8.2.2.2.7, 8.2.3.2.8, 8.2.4.2.8 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | x | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | x | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

< Start of change #1 >

### 8.2.1.2.8	 Interruptions during measurements with autonomous gaps

8.2.1.2.8.1    Interruptions when identifying CGI of an NR cell

When a UE is identifying CGI of an NR cell with autonomous gaps, the UE is allowed interruptions on PCell, PSCell or any activated SCell:

-	with interrupted slots up to ratio of interruption length X1 slot specified in Table 8.2.1.2.8.1-1 over SMTC period of target cell during MIB decoding time period TMIB_intra specified in clause [9.2.7],  or TMIB_inter specified in clause [9.3.9].

-	the length of each interruption during MIB decoding time period shall not exceed the number of X1 slots specified in Table  8.2.1.2.8.1-1.

-	with interrupted slots up to ratio of interruption length Y1 slot specified in Table 8.2.1.2.8-1 over 20 ms during SIB1 decoding time period TSIB1_intra specified in clause [9.2.7] or TSIB1_inter specified in clause [9.3.9], for SSB and CORESET for RMSI scheduling multiplexing patterns 1.

-	the length of each interruption during SIB1 decoding time period shall not exceed the number of Y1 slots specified in Table  8.2.1.2.8.1-1.

-	with interrupted slots up to ratio of interruption length Y2 slot specified in Table 8.2.1.2.8-1 over SMTC period of victim serving cell during SIB1 decoding time period TSIB1_intra specified in clause [9.2.7] or TSIB1_inter specified in clause [9.3.9], for SSB and CORESET for RMSI scheduling multiplexing patterns 2 and 3.

-	the length of each interruption during SIB1 decoding time period shall not exceed the number of Y2 slots specified in Table  8.2.1.2.8.1-1.

Table 8.2.1.2.8.1-1: Interruption length X1, Y1 and Y2 during measurements with autonomous gaps

|  | NR Slot length (ms) of victim cell | Interruption length X1 (slots) | Interruption length Y1 (slots) | Interruption length Y2 (slots) |
| --- | --- | --- | --- | --- |
| 0 | 1 | 2 + TSMTC_duration * Nslotsubframe,μ | 4 | 3 |
| 1 | 0.5 | 3 + TSMTC_duration * Nslotsubframe,μ | 5 | 4 |
| 2 | 0.25 Note1 | 5 + TSMTC_duration * Nslotsubframe,μ | 7 | 6 |
|  | 0.25 Note2 | 3 + TSMTC_duration * Nslotsubframe,μ | 5 | 4 |
| 3 | 0.125 | 5 + TSMTC_duration * Nslotsubframe,μ | 7 | 6 |
| NOTE 1: Victim cell is on FR1. NOTE 2: Victim cell is on FR2. NOTE 3: TSMTC_duration measured in subframes is the SMTC duration of the target cell for CGI idenitifcation. NOTE 4: Nslotsubframe,μ is as defined in TS 38.211 [6]. |  |  |  |  |

8.2.1.2.8.2    Interruptions when identifying CGI of an E-UTRA cell

When a UE is identifying CGI of an E-UTRA FDD cell or E-UTRA FDD cell with autonomous gaps, within time period [Tidentify_CGI_interRAT] in clause TBD in TS36.133 [15], the UE shall be able to transmit at least the number of ACK/NACKs specified in Table 8.2.1.2.8.2-1 on PCell, PSCell or any activated SCell in the frequency range where autonomous gaps are used, provided that:

-	there is continuous DL data allocation,

-	no DRX cycle is used,

-	no measurement gaps are configured,

-	only one code word is transmitted in each slot,

-	2 slot ACK/NACK feedback is configured,

-	20 ms SMTC period is configured.

Table 8.2.1.2.8.2-1: Minimum number of ACK/NACKs transmitted by the UE during [Tidentify_CGI_interRAT]

| Minimum number of transmitted ACK/NACKs | Configuration of the serving cell in which the transmitted ACK/NACKs are counted |  |
| --- | --- | --- |
|  | Duplex mode configuration | SCS |
| [84] | FDD | 15 kHz |
| [193] | FDD | 30 kHz |
| [402] | FDD | 60 kHz |
| [28] | TDD Note 1 | 15 kHz |
| [81] | TDD Note 1 | 30 kHz |
| [159] | TDD Note 1 | 60 kHz |
| [233] | TDD Note 2 | 60 kHz |
| [491] | TDD Note 2 | 120 kHz |
| NOTE 1: TDD UL-DL configuration is as specified in Table A.3.3.1-1 of TS 38.101-1 [18]. NOTE 2: TDD UL-DL configuration is as specified in Table A.3.3.1-1 of TS 38.101-2 [19]. |  |  |

< End of change #1 >

< Start of change #2 >

### 8.2.2.2.7	 Interruptions during measurements with autonomous gaps

8.2.2.2.7.1    Interruptions when identifying CGI of an NR cell

When a UE is identifying CGI of an NR cell with autonomous gaps, the UE is allowed interruptions on PCell, PSCell or any activated SCell:

-	with interrupted slots up to ratio of interruption length X1 slot specified in Table 8.2.2.2.7.1-1 over SMTC period of target cell during MIB decoding time period TMIB_intra specified in clause [9.2.7],  or TMIB_inter specified in clause [9.3.9].

-	the length of each interruption during MIB decoding time period shall not exceed the number of X1 slots specified in Table  8.2.2.2.7.1-1.

-	with interrupted slots up to ratio of interruption length Y1 slot specified in Table 8.2.2.2.7-1 over 20 ms during SIB1 decoding time period TSIB1_intra specified in clause [9.2.7] or TSIB1_inter specified in clause [9.3.9], for SSB and CORESET for RMSI scheduling multiplexing patterns 1.

-	the length of each interruption during SIB1 decoding time period shall not exceed the number of Y1 slots specified in Table  8.2.2.2.7.1-1.

-	with interrupted slots up to ratio of interruption length Y2 slot specified in Table 8.2.2.2.7-1 over SMTC period of victim serving cell during SIB1 decoding time period TSIB1_intra specified in clause [9.2.7] or TSIB1_inter specified in clause [9.3.9], for SSB and CORESET for RMSI scheduling multiplexing patterns 2 and 3.

-	the length of each interruption during SIB1 decoding time period shall not exceed the number of Y2 slots specified in Table  8.2.2.2.7.1-1.

Table 8.2.2.2.7.1-1: Interruption length X1, Y1 and Y2 during measurements with autonomous gaps

|  | NR Slot length (ms) of victim cell | Interruption length X1 (slots) | Interruption length Y1 (slots) | Interruption length Y2 (slots) |
| --- | --- | --- | --- | --- |
| 0 | 1 | 2 + TSMTC_duration * Nslotsubframe,μ | 4 | 3 |
| 1 | 0.5 | 3 + TSMTC_duration * Nslotsubframe,μ | 5 | 4 |
| 2 | 0.25 Note1 | 5 + TSMTC_duration * Nslotsubframe,μ | 7 | 6 |
|  | 0.25 Note2 | 3 + TSMTC_duration * Nslotsubframe,μ | 5 | 4 |
| 3 | 0.125 | 5 + TSMTC_duration * Nslotsubframe,μ | 7 | 6 |
| NOTE 1: Victim cell is on FR1. NOTE 2: Victim cell is on FR2. NOTE 3: TSMTC_duration measured in subframes is the SMTC duration of the target cell for CGI idenitifcation. NOTE 4: Nslotsubframe,μ is as defined in TS 38.211 [6]. |  |  |  |  |

8.2.2.2.7.2    Interruptions when identifying CGI of an E-UTRA cell

When a UE is identifying CGI of an E-UTRA FDD cell or E-UTRA FDD cell with autonomous gaps, within time period [Tidentify_CGI_interRAT] in clause TBD in TS36.133 [15], the UE shall be able to transmit at least the number of ACK/NACKs specified in Table 8.2.2.2.7.2-1 on PCell, PSCell or any activated SCell in the frequency range where autonomous gaps are used, provided that:

-	there is continuous DL data allocation,

-	no DRX cycle is used,

-	no measurement gaps are configured,

-	only one code word is transmitted in each slot,

-	2 slot ACK/NACK feedback is configured,

-	20 ms SMTC period is configured.

Table 8.2.2.2.7.2-1: Minimum number of ACK/NACKs transmitted by the UE during [Tidentify_CGI_interRAT]

| Minimum number of transmitted ACK/NACKs | Configuration of the serving cell in which the transmitted ACK/NACKs are counted |  |
| --- | --- | --- |
|  | Duplex mode configuration | SCS |
| [84] | FDD | 15 kHz |
| [193] | FDD | 30 kHz |
| [402] | FDD | 60 kHz |
| [28] | TDD Note 1 | 15 kHz |
| [81] | TDD Note 1 | 30 kHz |
| [159] | TDD Note 1 | 60 kHz |
| [233] | TDD Note 2 | 60 kHz |
| [491] | TDD Note 2 | 120 kHz |
| NOTE 1: TDD UL-DL configuration is as specified in Table A.3.3.1-1 of TS 38.101-1 [18]. NOTE 2: TDD UL-DL configuration is as specified in Table A.3.3.1-1 of TS 38.101-2 [19]. |  |  |

< End of change #2 >

< Start of change #3 >

### 8.2.3.2.8	 Interruptions during measurements with autonomous gaps

8.2.3.2.8.1    Interruptions when identifying CGI of an NR cell

When a UE is identifying CGI of an NR cell with autonomous gaps, the UE is allowed interruptions on PCell, PSCell or any activated SCell:

-	with interrupted slots up to ratio of interruption length X1 slot specified in Table 8.2.3.2.8.1-1 over SMTC period of target cell during MIB decoding time period TMIB_intra specified in clause [9.2.7],  or TMIB_inter specified in clause [9.3.9].

-	the length of each interruption during MIB decoding time period shall not exceed the number of X1 slots specified in Table  8.2.3.2.8.1-1.

-	with interrupted slots up to ratio of interruption length Y1 slot specified in Table 8.2.3.2.8-1 over 20 ms during SIB1 decoding time period TSIB1_intra specified in clause [9.2.7] or TSIB1_inter specified in clause [9.3.9], for SSB and CORESET for RMSI scheduling multiplexing patterns 1.

-	the length of each interruption during SIB1 decoding time period shall not exceed the number of Y1 slots specified in Table  8.2.3.2.8.1-1.

-	with interrupted slots up to ratio of interruption length Y2 slot specified in Table 8.2.3.2.8-1 over SMTC period of victim serving cell during SIB1 decoding time period TSIB1_intra specified in clause [9.2.7] or TSIB1_inter specified in clause [9.3.9], for SSB and CORESET for RMSI scheduling multiplexing patterns 2 and 3.

-	the length of each interruption during SIB1 decoding time period shall not exceed the number of Y2 slots specified in Table  8.2.3.2.8.1-1.

Table 8.2.3.2.8.1-1: Interruption length X1, Y1 and Y2 during measurements with autonomous gaps

|  | NR Slot length (ms) of victim cell | Interruption length X1 (slots) | Interruption length Y1 (slots) | Interruption length Y2 (slots) |
| --- | --- | --- | --- | --- |
| 0 | 1 | 2 + TSMTC_duration * Nslotsubframe,μ | 4 | 3 |
| 1 | 0.5 | 3 + TSMTC_duration * Nslotsubframe,μ | 5 | 4 |
| 2 | 0.25 Note1 | 5 + TSMTC_duration * Nslotsubframe,μ | 7 | 6 |
|  | 0.25 Note2 | 3 + TSMTC_duration * Nslotsubframe,μ | 5 | 4 |
| 3 | 0.125 | 5 + TSMTC_duration * Nslotsubframe,μ | 7 | 6 |
| NOTE 1: Victim cell is on FR1. NOTE 2: Victim cell is on FR2. NOTE 3: TSMTC_duration measured in subframes is the SMTC duration of the target cell for CGI idenitifcation. NOTE 4: Nslotsubframe,μ is as defined in TS 38.211 [6]. |  |  |  |  |

8.2.3.2.8.2    Interruptions when identifying CGI of an E-UTRA cell

When a UE is identifying CGI of an E-UTRA FDD cell or E-UTRA FDD cell with autonomous gaps, within time period [Tidentify_CGI_interRAT] in clause TBD in TS36.133 [15], the UE shall be able to transmit at least the number of ACK/NACKs specified in Table 8.2.3.2.8.2-1 on PCell, PSCell or any activated SCell in the frequency range where autonomous gaps are used, provided that:

-	there is continuous DL data allocation,

-	no DRX cycle is used,

-	no measurement gaps are configured,

-	only one code word is transmitted in each slot,

-	2 slot ACK/NACK feedback is configured,

-	20 ms SMTC period is configured.

Table 8.2.3.2.8.2-1: Minimum number of ACK/NACKs transmitted by the UE during [Tidentify_CGI_interRAT]

| Minimum number of transmitted ACK/NACKs | Configuration of the serving cell in which the transmitted ACK/NACKs are counted |  |
| --- | --- | --- |
|  | Duplex mode configuration | SCS |
| [84] | FDD | 15 kHz |
| [193] | FDD | 30 kHz |
| [402] | FDD | 60 kHz |
| [28] | TDD Note 1 | 15 kHz |
| [81] | TDD Note 1 | 30 kHz |
| [159] | TDD Note 1 | 60 kHz |
| [233] | TDD Note 2 | 60 kHz |
| [491] | TDD Note 2 | 120 kHz |
| NOTE 1: TDD UL-DL configuration is as specified in Table A.3.3.1-1 of TS 38.101-1 [18]. NOTE 2: TDD UL-DL configuration is as specified in Table A.3.3.1-1 of TS 38.101-2 [19]. |  |  |

< End of change #3 >

< Start of change #4 >

### 8.2.4.2.8	 Interruptions during measurements with autonomous gaps

8.2.4.2.8.1    Interruptions when identifying CGI of an NR cell

When a UE is identifying CGI of an NR cell with autonomous gaps, the UE is allowed interruptions on PCell, PSCell or any activated SCell:

-	with interrupted slots up to ratio of interruption length X1 slot specified in Table 8.2.4.2.8.1-1 over SMTC period of target cell during MIB decoding time period TMIB_intra specified in clause [9.2.7],  or TMIB_inter specified in clause [9.3.9].

-	the length of each interruption during MIB decoding time period shall not exceed the number of X1 slots specified in Table  8.2.4.2.8.1-1.

-	with interrupted slots up to ratio of interruption length Y1 slot specified in Table 8.2.4.2.8-1 over 20 ms during SIB1 decoding time period TSIB1_intra specified in clause [9.2.7] or TSIB1_inter specified in clause [9.3.9], for SSB and CORESET for RMSI scheduling multiplexing patterns 1.

-	the length of each interruption during SIB1 decoding time period shall not exceed the number of Y1 slots specified in Table  8.2.4.2.8.1-1.

-	with interrupted slots up to ratio of interruption length Y2 slot specified in Table 8.2.4.2.8-1 over SMTC period of victim serving cell during SIB1 decoding time period TSIB1_intra specified in clause [9.2.7] or TSIB1_inter specified in clause [9.3.9], for SSB and CORESET for RMSI scheduling multiplexing patterns 2 and 3.

-	the length of each interruption during SIB1 decoding time period shall not exceed the number of Y2 slots specified in Table  8.2.4.2.8.1-1.

Table 8.2.4.2.8.1-1: Interruption length X1, Y1 and Y2 during measurements with autonomous gaps

|  | NR Slot length (ms) of victim cell | Interruption length X1 (slots) | Interruption length Y1 (slots) | Interruption length Y2 (slots) |
| --- | --- | --- | --- | --- |
| 0 | 1 | 2 + TSMTC_duration * Nslotsubframe,μ | 4 | 3 |
| 1 | 0.5 | 3 + TSMTC_duration * Nslotsubframe,μ | 5 | 4 |
| 2 | 0.25 Note1 | 5 + TSMTC_duration * Nslotsubframe,μ | 7 | 6 |
|  | 0.25 Note2 | 3 + TSMTC_duration * Nslotsubframe,μ | 5 | 4 |
| 3 | 0.125 | 5 + TSMTC_duration * Nslotsubframe,μ | 7 | 6 |
| NOTE 1: Victim cell is on FR1. NOTE 2: Victim cell is on FR2. NOTE 3: TSMTC_duration measured in subframes is the SMTC duration of the target cell for CGI idenitifcation. NOTE 4: Nslotsubframe,μ is as defined in TS 38.211 [6]. |  |  |  |  |

8.2.4.2.8.2    Interruptions when identifying CGI of an E-UTRA cell

When a UE is identifying CGI of an E-UTRA FDD cell or E-UTRA FDD cell with autonomous gaps, within time period [Tidentify_CGI_interRAT] in clause TBD in TS36.133 [15], the UE shall be able to transmit at least the number of ACK/NACKs specified in Table 8.2.4.2.8.2-1 on PCell, PSCell or any activated SCell in the frequency range where autonomous gaps are used, provided that:

-	there is continuous DL data allocation,

-	no DRX cycle is used,

-	no measurement gaps are configured,

-	only one code word is transmitted in each slot,

-	2 slot ACK/NACK feedback is configured,

-	20 ms SMTC period is configured.

Table 8.2.4.2.8.2-1: Minimum number of ACK/NACKs transmitted by the UE during [Tidentify_CGI_interRAT]

| Minimum number of transmitted ACK/NACKs | Configuration of the serving cell in which the transmitted ACK/NACKs are counted |  |
| --- | --- | --- |
|  | Duplex mode configuration | SCS |
| [84] | FDD | 15 kHz |
| [193] | FDD | 30 kHz |
| [402] | FDD | 60 kHz |
| [28] | TDD Note 1 | 15 kHz |
| [81] | TDD Note 1 | 30 kHz |
| [159] | TDD Note 1 | 60 kHz |
| [233] | TDD Note 2 | 60 kHz |
| [491] | TDD Note 2 | 120 kHz |
| NOTE 1: TDD UL-DL configuration is as specified in Table A.3.3.1-1 of TS 38.101-1 [18]. NOTE 2: TDD UL-DL configuration is as specified in Table A.3.3.1-1 of TS 38.101-2 [19]. |  |  |

< End of change #4 >