3GPP TSG-RAN WG4 Meeting #94-e	R4-2001274

Online, 24 February – 6 March, 2020

Source:	ZTE

Title:	Further discussion on mandatory of measurement gap patterns for NR

Agenda item:	8.15.1.6

Document for:	Discussion

1. Introduction

In the RAN4#93 meeting, there were good progress on mandatory of measurement gap patterns for NR in Rel-16. The agreements and open issues are captured in the way forward [1].

- Confirmed R15 capability information elements cannot be changed in meaning by any new capability signaling introduced in R16. - Confirmed R15 gap pattern applicability rule is not changed due to introduction of additional mandatory gap patterns in R16.

- Introduce a new Rel-16 UE capability to indicate measurement gap patterns for 'NR only measurements' - NR only measurement is to be clarified. - Applicability of additional mandatory gap patterns and new UE capability signalling - Additional mandatory gap patterns and new UE capability in Rel-16 are applicable for at least NR SA and NR-DC mode - FFS applicablity for EN-DC, NE-DC, and LTE SA mode - FFS Mandatory gap patterns for FR1 - FFS Mandatory gap patterns for FR2

In this contribution we further provide our views on open issues for mandatory of measurement gap patterns in R16.

2. Discussion

UE capability and applicability

It was agreed to introduce a new Rel-16 UE capability to indicate measurement gap patterns for ‘NR only measurement’. What NR only measurement is needs further clarification.

The NR only measurement means a supported gap pattern can only be used for NR measurement. When there is E-UTRA measurement being configured together, the supported gap pattern cannot be configured for the UE.

In NR SA, the supported gap patterns are signaled by signaling supportedGapPattern (22 bits).

- In NR SA - NR RRC signalling supportedGapPattern (22 bits) is used to signal whether UE supports gap patterns 2 to 23.

For gap patterns with MGL < 6ms, depending on UE capability, a UE may not be able to do LTE measurements with the gap pattern. In order for the UE to indicate whether supported gap patterns with MGL < 6ms can be configured for E-UTRA measurements, it was agreed to introduce a Rel-16 UE capability.

This new UE capability can be used not only for additional mandatory gap patterns but also for other optional gap patterns. Because if a gap pattern can only be used for NR only measurements, the UE should indicate to the network, even though it is an option gap pattern. Otherwise the UE may just indicate the gap pattern is not supported.

Proposal 1. NR measurements only means UE cannot be configured with gap patterns with MGL<6ms for E-UTRA measurements, or for both NR and E-UTRA measurement together.

Proposal 2. New UE capability is used to indicate if a gap pattern among gap patterns #2 ~ #11 can only be configured for NR measurement only.

It was agreed additional mandatory gap patterns and new UE capability in Rel-16 are applicable for at least NR SA and NR-DC mode. On EN-DC, NE-DC and LTE SA mode, further study is needed.

For LTE SA mode, the signalling for indicating measurement gap patterns are as follows.

- LTE RRC signalling shortMeasurementGap is used to signal whether UE supports gap patterns 2 and 3 (for per-UE cap and FR1 gap of per-FR gap). - LTE RRC signalling measGapPatterns is used to signal whether UE supports gap patterns 4 to 11 (for per-UE cap and FR1 gap of per-FR gap).

LTE RRC signalling already support indication of short measurement gap patterns (MGL = 3 ms) 2 and 3. There are other short measurement gaps patterns e.g. 6-10, can be used for E-UTRA measurements depending on configuration of measurements. However there is concern on changing LTE design. We think it is valid concern. So change of LTE RRC signalling is not necessary.

Proposal 3. No new LTE RRC signalling is to be introduced, i.e. no additional mandatory gap patterns and new UE capability for LTE.

In EN-DC, per-UE measurement gap pattern can only be configured by E-UTRA PCell. For per-FR gap pattern, FR1 gap pattern can only be configured by E-UTRA PCell. NR PSCell can only configure FR2 gap pattern. Therefore new UE capability is not applicable for EN-DC. However for additional measurement gap patterns, there will be more FR2 gap patterns to be mandatory additionally, so it should be applicable to EN-DC.

Proposal 4. New UE capability is not applicable for EN-DC.

Proposal 5. Additional mandatory of gap patterns in FR2 is applicable for EN-DC.

In NE-DC, all the NR measurements can only be configured by NR PCell. This is the same as NR SA mode. So the new UE capability and additional mandatory of gap patterns is applicable for NE-DC.

Proposal 6. Additional mandatory gap patterns and new UE capability in Rel-16 are applicable for NE-DC mode

Mandatory of gap patterns

The only mandatory gap patterns are GP#0 and GP#1 which are 6ms MGL with 40ms and 80ms MGRP respectively. Later RAN4 further agreed additional gap patterns GP#13 and GP#14 for measurements on FR2 carrier frequencies and send LS [3] to RAN2.

It is beneficial to mandate more gap patterns due to the actual SSB configuration in practical network. Now the MGL of mandatory gap patterns are the longest in FR1 and FR2 which are 6ms and 5.5ms respectively. However the possible SSBs in half frame is much less than 6ms.

In FR1 the SSBs in half frame as maximum take up 2ms for 15kHz SSB SCS, and 1ms for 30kHz SSB SCS for frequency below 3GHz. It at maximum takes up 4ms for 15kHz SSB SCS, and 2ms for 30kHz SSB SCS. It can be seen that in most cases in FR1 the SSBs in half frame are 2ms. In these cases 3ms MGL should be enough for synchronous network, and 4ms MGL should be enough for asynchronous network. With mandatory of only GP#0 and GP#1 in FR1, 6ms MGL will have to be used. There are two aspects of impact by using unnecessary longer measurement gap length. On one hand in sync network 3ms during 40/80 ms MGRP are wasted in vain. It means 7.5% and 3.75% throughput loss. On the other hand it means increase of UE power saving.

Observation 1. There are throughput loss and increased UE power consumption when longer MGL than necessary has to be used if the suitable gap pattern is not supported by UE.

Observation 2. 3ms MGL and 4ms MGL are necessary in FR1 for sync and async network.

It has already been mandatory to support 40ms and 80ms MGRP with 6ms MGL. Supporting shorter MGL with 40ms/80ms MGRP doesn’t put any UE complexity in terms of memory to store the data within measurement gap. The efforts to support shorter MGL is minimized.

Observation 3. No UE complexity issue is identified to support short MGL.

Since 40ms/80ms MGRP are typical configurations in practical network, gap patterns of combination of 40ms/80ms MGRP and necessary MGL should be supported. For 20ms MGRP it may be used in certain special cases and the overhead is quite high. Consequently the throughput loss could be very high especially together with 6ms MGL. We don’t think 20ms MGRP is a typical case in FR1. For 160ms MGRP there could be use case, e.g. if UE is configured to measure a frequency with 160ms SMTC periodicity. It would be better to support this MGRP. Since UE complexity is mainly related to measurement gap length, support of 160ms MGRP is also feasible.

Observation 4. 40/80ms MGRP should be necessarily supported. 160ms MGRP is also preferable to be supported.

Proposal 7. In FR1 gap patterns with 40/80ms MGRP and 3ms/4ms MGL, i.e. GP#2, GP#3, GP#7 and GP#8 should be additionally supported.

Proposal 8. In FR1 gap patterns with 160ms MGRP and 3ms/4ms/6ms MGL, i.e. GP#5, GP#9 and GP#11 are preferably to be supported.

In FR2 the discussion is similar to FR1. The SSBs in half frame at maximum take up almost 5ms for 120kHz SSB SCS and around 2ms for 240 kHz SSB SCS. So the 3.5ms MGL and 5.5MGL are both useful. No UE complexity issue can be seen to support shorter MGL.

The 20ms MGRP may be useful in FR2 if the channel changes very quickly. It would be better to support gap patterns with 20ms MGRP. Though 5.5ms MGL with 20ms MGRP could cause throughput loss, it may not need to be mandatory supported. Similar to FR1, 160ms MGRP is also useful and is preferably to be supported.

Proposal 9. In FR2 gap patterns with 40/80ms MGRP and 3.5ms MGL, i.e. GP#17 and GP#18 should be additionally supported. Gap pattern with 20ms MGRP and 3.5ms/5.5ms MGL, i.e. GP#12 and GP#16, should also be additionally supported.

Proposal 10. In FR2 gap patterns with 160ms MGRP and 3.5ms/5.5ms MGL, i.e. GP#15 and GP#19 should be additionally supported.

3. Conclusion

In this contribution, we provided our views on mandatory of measurement gap patterns in R16. Based on the observations following proposals are present.

Proposal 1. NR measurements only means UE cannot be configured with gap patterns with MGL<6ms for E-UTRA measurements, or for both NR and E-UTRA measurement together.

Proposal 2. New UE capability is used to indicate if a gap pattern among gap patterns #2 ~ #11 can only be configured for NR measurement only.

Proposal 3. No new LTE RRC signalling is to be introduced, i.e. no additional mandatory gap patterns and new UE capability for LTE.

Proposal 4. New UE capability is not applicable for EN-DC.

Proposal 5. Additional mandatory of gap patterns in FR2 is applicable for EN-DC.

Proposal 6. Additional mandatory gap patterns and new UE capability in Rel-16 are applicable for NE-DC mode

Observation 1. There are throughput loss and increased UE power consumption when longer MGL than necessary has to be used if the suitable gap pattern is not supported by UE.

Observation 2. 3ms MGL and 4ms MGL are necessary in FR1 for sync and async network.

Observation 3. No UE complexity issue is identified to support short MGL.

Observation 4. 40/80ms MGRP should be necessarily supported. 160ms MGRP is also preferable to be supported.

Proposal 7. In FR1 gap patterns with 40/80ms MGRP and 3ms/4ms MGL, i.e. GP#2, GP#3, GP#7 and GP#8 should be additionally supported.

Proposal 8. In FR1 gap patterns with 160ms MGRP and 3ms/4ms/6ms MGL, i.e. GP#5, GP#9 and GP#11 are preferably to be supported.

Proposal 9. In FR2 gap patterns with 40/80ms MGRP and 3.5ms MGL, i.e. GP#17 and GP#18 should be additionally supported. Gap pattern with 20ms MGRP and 3.5ms/5.5ms MGL, i.e. GP#12 and GP#16, should also be additionally supported.

Proposal 10. In FR2 gap patterns with 160ms MGRP and 3.5ms/5.5ms MGL, i.e. GP#15 and GP#19 should be additionally supported.

4. References

- R4-1914794	Way forward on R16 NR RRM enhancements – mandatory of gap patterns, ZTE