3GPP TSG-RAN WG4 Meeting #94-e				  											R4-2001303

Online, 24th Feb. 2020 – 6th Mar. 2020

Source: 	Nokia, T-Mobile

Title: 		TP for 37.716-21-21 to introduce DC_2-46-66_n41-n71

Agenda Item: 			9.8.2 [DC_R16_xBLTE_2BNR_yDL2UL-Core]

Document for:	Approval

# 1	Introduction

This contribution is a TP for TR 37.716-21-21 to introduce DC_2-46-66_n41-n71.

# 2	Text Proposal

*********************** Start of the TP ***************************************************

## 8.x	DC_2-46-66_n41-n71

### 8.x.1	Operating bands for DC

Table 8.x.1-1: DC band combination of LTE 2DL/1UL + inter-band NR 2DL/1UL

| E-UTRA and NR DC Band combination | E-UTRA and NR DC Band | Uplink (UL) band |  |  | Downlink (DL) band |  |  | Duplex mode |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | BS receive / UE transmit |  |  | BS transmit / UE receive |  |  |  |
|  |  | FUL_low – FUL_high |  |  | FDL_low – FDL_high |  |  |  |
| DC_2-46-66_n41-n71 | 2 | 1850 MHz | – | 1910 MHz | 1930 MHz | – | 1990 MHz | FDD |
|  | 46 | 5150 MHz | – | 5925 MHz | 5150 MHz | – | 5925 MHz | TDD |
|  | 66 | 1710 MHz | – | 1780 MHz | 2110 MHz | – | 2200 MHz | FDD |
|  | n41 | 2496 MHz |  | 2690 MHz | 2496 MHz |  | 2690 MHz | TDD |
|  | n71 | 663 MHz | – | 698 MHz | 617 MHz | – | 652 MHz | FDD |

### 8.x.2	Co-existence studies

Co-existence studies are captured in TR 37.716-11-11 as well as for lower order combinations in TR 37.716-21-21.

### 8.x.3	∆TIB and ∆RIB values

The TIB,c and RIB,c values are given in the tables below based on lower order combinations including the same bands.

Table 8.x.3-1: ΔTIB,c

| Inter-band DC Configuration | E-UTRA and NR Band | ΔTIB,c [dB] |
| --- | --- | --- |
| DC_2-46-66_n41-n71 | 2 | 0.5 |
|  | 46 | 0 |
|  | 66 | 0.5 |
|  | n41 | 0.41 |
|  |  | 0.92 |
|  | n71 | 0.6 |
| NOTE 1: The requirement is applied for UE transmitting on the frequency range of 2545 – 2690 MHz. NOTE 2: The requirement is applied for UE transmitting on the frequency range of 2496 – 2545 MHz. |  |  |

Table 8.x.3-2: ΔRIB

| Inter-band DC Configuration | E-UTRA and NR Band | ΔRIB [dB] |
| --- | --- | --- |
| DC_2-46-66_n41-n71 | 2 | 0.3 |
|  | 46 | 0 |
|  | 66 | 0.3 |
|  | n41 | 0.51 |
|  |  | 12 |
|  | n71 | 0.5 |
| NOTE 1: The requirement is applied for UE transmitting on the frequency range of 2545 – 2690 MHz. NOTE 2: The requirement is applied for UE transmitting on the frequency range of 2496 – 2545 MHz. |  |  |

### 8.x.4	MSD

MSD was studied in lower order band combinations. No need for additional MSD analysis.

*********************** End of the TP ***************************************************

# 3	References