3GPP TSG-RAN WG4 Meeting #94-e                                                  R4-2001307

Athens, Greek 24th to 28th Feb. 2020

Agenda Item:	8.13.1.6

Source: 	MediaTek Inc.

Title: 	Switching time between NR between FR1 uplink carriers

Document for:	Approval

- 1. Introduction

WF for switching time was agreed in RAN4#3 [2]. Switching time between case 1 and case 2 has been added in RAN4 objective. The contribution is to provide some views for remaining issues.

- 2. Discussion

In [2], some issues were not discussed during RAN4#93:

Continue discussion on the issues under the following sub-sections of the WF in [2]:

Length of UL switching period for defining UE RF requirements:

Non-zero value

Option A: {35us, 140 us, 250us} or {1, 4, and 7} OFDM symbols for 30kHz SCS

Option B: {35us, 140 us} or {1, 4} OFDM symbols for 30kHz SCS

Option C: only 35us

FFS whether to define 0us switching period

About non-zero value, considering various design for UE implementations, it might be difficult to achieve consensus on a single non-zero value. So our preference is to adopt option B {35us, 140 us} or {1, 4} OFDM symbols for 30kHz SCS. And since concurrent two UL hardware chains is common understanding in existing specs, we do not think 0us shall be kept in Rel-16 switching time. But to further consider future UE develop, we do not preclude feasibility of three concurrent uplink chain.

Proposal 1: For non-zero value, it is proposed to adopt option B and reported by UE per uplink band combination. 0us shall be removed in Rel-16. Keep FFS on whether to define 0us switching period for Rel-17 or later release.

Considering about uplink/downlink interruption, there are three options in the WF[2]:

- Option A: Define different capabilities for UEs with and without DL reception interruption

  - If UE does not report this capability, it means there is no DL reception interruption.

- Option B: DL reception interruption is not allowed.

- Option C:

  - No DL reception interruption for the following duplex mode combinations: (carrier 1 + carrier 2)

- SUL+TDD

- FDD+TDD

- TDD+TDD with synchronous UL-DL configurations the same UL-DL pattern

- Other duplex mode combinations: allowed for one or two DL carriers based on UE capability reporting

Following the justifications in [3], downlink interruption shall be allowed for FDD band in case its uplink chain is to be switched to another band in which uplink frequency is in close proximity of the FDD downlink band. For example, if there’s uplink switching for DC_B1A_n40A, the FDD downlink path LO may suffer such pulling mechanism. It would be difficult to say any FDD+TDD combination in option C does not need downlink interruption. As for uplink interruption during switching period, no matter case 2 is running in TX diversity mode or UL MIMO, the LO would be affected due to load change of additional routing to 2nd TX chain to keep both uplink carriers coherent in case 2. So we do have concern on option C for FDD+TDD combination. Uplink interruption shall be allowed for both TX carriers during switching period. Interruption requirement of different combinations are shown in table 1 for further discussions. Overall speaking, our preference is option A to have different UE capability report for downlink interruption per band combination.

| Scenario ID | Case 1 |  | Case 2 |  | Synchronized UL/DL | Interruption during switching |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  | Carrier 1 | Carrier 2 | Carrier 1 | Carrier 2 |  | DL | UL |
| 1 | FDD | TDD |  | TDD 2xUL | N/A | Carrier 1 | All UL |
| 2 | FDD | FDD |  | FDD 2xUL | N/A | All DL | All UL |
| 3 | TDD | TDD |  | TDD 2xUL | Yes | No | All UL |
| 4 | TDD | TDD |  | TDD 2xUL | No | Carrier 1 | All UL |
| 5 | TDD | FDD |  | FDD 2xUL | N/A | Carrier 2 | All UL |
| 6 | SUL | TDD |  | TDD 2xUL | N/A | No | All UL |
| 7 | SUL | FDD |  | FDD 2xUL | N/A | Carrier 2 | All UL |

Table 1. UL/DL interruption

With above justifications, we have following proposal about UL/DL interruption:

Proposal 2: For EN-DC without SUL, inter-band UL CA and standalone SUL in case 1, it is proposed to adopt option A. Uplink interruption shall be allowed on both UL carriers.

As for LS R4-1916083, as further look into the content, it is common understanding that uplink outage is allowing for both carriers during uplink switching time. The sentence in the LS “In RAN4#92bis, it was agreed that the location of Tx switching period should be semi-statically configured by RRC on one specific carrier of the two uplink carriers, i.e., carrier 1 or carrier 2, in case of SA CA and SUL” shall be corrected to (omit)…”the location of Tx switching period should be semi-statically configured by RRC on either case 1 or case 2 in case of SA CA and SUL”.

Proposal 3: For SA CA and SUL, the location of Tx switching period should be semi-statically configured by RRC on eigher case 1 or case 2.

3. Conclusion

In this contribution, we provide our views and proposals:

Proposal 1: For non-zero value, it is proposed to adopt option B and reported by UE per uplink band combination. And still keep FFS on whether to define 0us switching period

Proposal 2: For EN-DC without SUL, inter-band UL CA and standalone SUL in case 1, it is proposed to adopt option A. Uplink interruption shall be allowed on both UL carriers. Table 1 lists possible scenarios for further discussions.

Proposal 3: For SA CA and SUL, the location of Tx switching period should be semi-statically configured by RRC on eigher case 1 or case 2.

Reference:

- R4-1911615 Switching time between NR between FR1 uplink carriers_v0, RAN4#92-bis

- R4-1916084 WF on Tx switching between case 1 and 2, RAN4#93

- R4-1807057 UL switching time for DC_3A_n3A single switched UL operation