3GPP TSG-RAN WG4 Meeting #94-e	R4-2001316

# Electronic meeting, 24 February – 6 March 2020

| CR-Form-v12.0 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.101-1 | CR | 255 | rev | - | Current version: | 15.8.2 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | X | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Correction of transmitter characteristics for UL-MIMO: powerclass 2 and fallback |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Ericsson |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R4 |  |  |  |  |  |  |  |  |  |
| Work item code: | NR_newRAT-Core |  |  |  |  |  | Date: |  |  | 2020-02-24 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-15 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) Rel-12 (Release 12) Rel-13 (Release 13) Rel-14 (Release 14) Rel-15 (Release 15) Rel-16 (Release 16) |  |
| Reason for change: |  | For UL-MIMO 1. the applicability of the MOP requirement when TPMI = 0 is sent is ambigous it is not whether requirements apply for both single-layer and dual-layer transmission or only the latter 2. the specification the fallback single-antenna port requirement is unclear: the current statement copied from 36.101 3. there is no requirement that full power shall be delivered according to the power class when PUSCH is scheduled by DCI 0_1 and the UE is configured with a single antenna port following 38.213. UEs implementing power class 2 with two power class 3 chains in UL-MIMO configuration are not accommodated by the specification. Should these indicate power class 2 in the field ue-PowerClass of the UE-NR-Capability, the MOP capability in fall-back is ambigous (23 or 26 dBm). The unwanted emissions requirements are measured at each antenna connector and not as the total of emissions from all antenna ports (connectors) as required by regulations in several regions/countries. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Clause 6.2D.1: 1. remove the ambiguity on applicability of the requirements when TPMI = 0 is sent: requirements for both single-layer and dual-layer transmission is specified with two ports configured. 2. specify the fall-back single-antenna port behaviour: performance according to the power class capability indicated must be met when PUSCH is scheduled for single port transmission by DCI 0_0 (verified per connector) 3. add a requirement that full power shall according to the power class indicated shall be delivered when PUSCH is scheduled by DCI 0_1 and the UE is configured with a single antenna port following 38.213. The behavour according to items 2 and 3 are specified in the remaining subclauses of 6.2D. A UE implementing power class 2 with two power class 3 chains in UL-MIMO configuration is accommodated by allowing compliance with power class 2 requirements when the said UE transmits two layers also for a power class 3 indication in the field ue-PowerClass of its UE-NR-Capability. In fallback or single-port configuration this UE has to be compliant with power class 3, i.e. according to the capability indicated (a UE compliant with power class 2 under these conditions can indicate power class 2). Furthermore, for a UE indicating power class 3 capability but compliant with power class 2 requirements for dual-layer transmission, the Pcmax for PUSCH with dual antenna-ports configured is modified such that power class 2 behaviour is reported in the PHR (scaling applies across the port according to 38.213 to 23 dBm per port) in order to limit the ambiguity in signaling. For EN-DC band combinations, the above UE would still indicate NR power class 3, the expected behaviour. The requirements on unwanted emissions are specified as the total of the emissions from UE antenna ports (connectors) consistent with regulatory requirements. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Ambiguous UL-MIMO MOP requirements and unclear fallback behaviour. Inaccurate PHR reporting for UEs indicating power class 3 capability but compliant with power class 2 requirements for dual-layer transmission. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 6.2D.1, 6.2D.2, 6.2D.3, 6.2D.4, 6.3D.1, 6.3D.3, 6.3D.4, 6.4D.1, 6.4D.2, 6.5D.1, 6.5D.2, 6.5D.3, 6.5D.4 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  |  |  |  |
| affected: |  |  | X | Test specifications |  |  |  |  |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  |  |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

< start of changes >

## 6.2D	Transmitter power for UL MIMO

### 6.2D.1	UE maximum output power for UL MIMO

For a UE with two transmit antenna connectors in closed-loop spatial multiplexing scheme, the maximum output power for any transmission bandwidth within the channel bandwidth is specified in Table 6.2D.1-1. The requirements shall be met with the UL MIMO configurations specified in Table 6.2D.1-2 with PUSCH scheduled by DCI format 0_1 and the transmission precoder selected from the codebooks for two antenna ports. For UEs indicating power class 3 in the ue-PowerClass field of the UE-NR-Capability IE, the UE shall meet the requirements 6.2D.1-1 for either power class 2 or power class 3. The maximum output power is measured as the sum of the maximum output powers from both UE antenna connectors. The period of measurement shall be at least one sub frame (1 ms).

Table 6.2D.1-1: UE Power Class for UL MIMO in closed loop spatial multiplexing scheme

| NR band | Class 1 (dBm) | Tolerance (dB) | Class 2 (dBm) | Tolerance (dB) | Class 3 (dBm) | Tolerance (dB) | Class 4 (dBm) | Tolerance (dB) |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| n41 |  |  | 26 | +2/-31 | 23 | +2/-31 |  |  |
| n77 |  |  | 26 | +2/-3 | 23 | +2/-3 |  |  |
| n78 |  |  | 26 | +2/-3 | 23 | +2/-3 |  |  |
| n79 |  |  | 26 | +2/-3 | 23 | +2/-3 |  |  |
| NOTE 1: The transmission bandwidths confined within FUL_low and FUL_low + 4 MHz or FUL_high – 4 MHz and FUL_high, the maximum output power requirement is relaxed by reducing the lower tolerance limit by 1.5 dB NOTE 2: Power class 3 is the default power class unless otherwise stated |  |  |  |  |  |  |  |  |

Table 6.2D.1-2: UL MIMO configuration in closed-loop spatial multiplexing scheme

| Transmission scheme | Number of layers | DCI format | TPMI index |
| --- | --- | --- | --- |
| Codebook based | 1 | DCI format 0_1 | 0,1 |
| Codebook based | 2 | DCI format 0_1 | 0 |
| NOTE 1: The UE is configured with one SRS resource with the parameter nrofSRS-Ports set to two SRS antenna ports. |  |  |  |

When PUSCH is scheduled [for single-layer transmission] using TPMI 0 or 1 according to Table 6.2D.1-2, the output power shall be 3 dB less than the maximum output power given by Table 6.2D.1-1, as is specified in [8] subclause 7.1.

### For each power class as indicated by the ue-PowerClass field of the UE-NR-Capability IE, the UE shall meet the requirements in 6.2.1 when PUSCH is scheduled for single antenna-port transmission by DCI 0_0 or by DCI 0_1 when the UE is configured for single port operation.

### 6.2D.2	UE maximum output power reduction for UL MIMO

For UE with two transmit antenna connectors in closed-loop spatial multiplexing scheme, the allowed Maximum Power Reduction (MPR) for the maximum output power in Table 6.2D.1-1 is specified in Table 6.2.2-1 and Table 6.2.2-2 for the respective power class for both single-layer and dual layer transmission. The requirements shall be met with UL MIMO configurations specified in subclause 6.2D.1. For UEs indicating power class 3 in the ue-PowerClass field of the UE-NR-Capability IE and compliant with the requirements 6.2D.1-1 for power class 2 for dual-layer transmission, Table 6.2.2-2 may be used.  The maximum output power is measured as the sum of the maximum output power at each UE antenna connector.

For the UE maximum output power modified by MPR, the power limits specified in subclause 6.2D.4 apply.

For each power class as indicated by the ue-PowerClass field of the UE-NR-Capability IE, the UE shall meet the requirements in 6.2.2 when PUSCH is scheduled for single antenna-port transmission by DCI 0_0 or by DCI 0_1 when the UE is configured for single port operation.

### 6.2D.3	UE additional maximum output power reduction for UL MIMO

For UE with two transmit antenna connectors in closed-loop spatial multiplexing scheme, the A-MPR values specified in subclause 6.2.3 shall apply to the maximum output power specified in Table 6.2D.1-1 for both single-layer and dual-layer transmission. The requirements shall be met with the UL MIMO configurations specified in subclause 6.2D.1. For UEs indicating power class 3 in the ue-PowerClass field of the UE-NR-Capability IE and compliant with the requirements 6.2D.1-1 for power class 2 for dual-layer transmission shall meet the requirements 6.2.3 for power class 2. The maximum output power is measured as the sum of the maximum output power at each UE antenna connector. Unless stated otherwise, an A-MPR of 0 dB shall be used.

For the UE maximum output power modified by A-MPR, the power limits specified in subclause 6.2D.4 apply.

For each power class as indicated by the ue-PowerClass field of the UE-NR-Capability IE, the UE shall meet the requirements in 6.2.3 when PUSCH is scheduled for single antenna-port transmission by DCI 0_0 or by DCI 0_1 when the UE is configured for single port operation.

### 6.2D.4	Configured transmitted power for UL MIMO

The definitions of configured maximum output power PCMAX,c, the lower bound PCMAX_L,c, and the higher bound PCMAX_H,c specified in subclause 6.2.4 shall apply to UE supporting UL MIMO, where

-	PPowerClass, ΔPPowerClass and ∆TC,c are specified in subclause 6.2.4 unless otherwise stated;

-	MPRc is specified in subclause 6.2D.2;

-	A-MPRc is specified in subclause 6.2D.3.

For UEs indicating power class 3 in the ue-PowerClass field of the UE-NR-Capability IE and compliant with the requirements 6.2D.1-1 for power class 2 for dual-layer transmission, ΔPPowerClass = -3 dB for PUSCH configured for dual antenna-port transmissions.

The measured configured maximum output power PUMAX,c for serving cell c shall be within the following bounds:

PCMAX_L,c  –  MAX{TL, T LOW(PCMAX_L,c)}  ≤  PUMAX,c  ≤  PCMAX_H,c  +  T HIGH(PCMAX_H,c)

where TLOW(PCMAX_L,c) and THIGH(PCMAX_H,c) are defined as the tolerance and applies to PCMAX_L,c and PCMAX_H,c separately, while TL is the absolute value of the lower tolerance in Table 6.2D.1-1 for the applicable operating band. For single-layer transmissions on two antenna ports, the upper and lower bounds of the measured configured maximum output power PUMAX,c as specified above shall be reduced by 3 dB.

For UE with two transmit antenna connectors in closed-loop spatial amultiplexing scheme, the tolerance is specified in Table 6.2D.4-1. The requirements shall be met with UL MIMO configurations specified in subclause 6.2D.1.

Table 6.2D.4-1: PCMAX,c tolerance in closed-loop spatial multiplexing scheme

| PCMAX,c (dBm) | Tolerance TLOW(PCMAX_L,c) (dB) | Tolerance THIGH(PCMAX_H,c) (dB) |
| --- | --- | --- |
| PCMAX,c = 26 | 3.0 | 2.0 |
| 23 ≤ PCMAX,c < 26 | 3.0 | 2.0 |
| 22 ≤ PCMAX,c < 23 | 5.0 | 2.0 |
| 21 ≤ PCMAX,c < 22 | 5.0 | 3.0 |
| 20 ≤ PCMAX,c < 21 | 6.0 | 4.0 |
| 16 ≤ PCMAX,c < 20 | 5.0 |  |
| 11 ≤ PCMAX,c < 16 | 6.0 |  |
| -40 ≤ PCMAX,c < 11 | 7.0 |  |

For each power class as indicated by the ue-PowerClass field of the UE-NR-Capability IE, the UE shall meet the requirements in 6.2.4 when PUSCH is scheduled for single antenna-port transmission by DCI 0_0 or by DCI 0_1 when the UE is configured for single port operation.

< text omitted >

## 6.3	Output power dynamics

## 6.3D	Output power dynamics for UL MIMO

### 6.3D.1	Minimum output power for UL MIMO

For UE with two transmit antenna connectors in closed-loop spatial multiplexing scheme, the minimum output power is defined as the sum of the mean power at each transmit connector in one sub-frame (1 ms). The minimum output power shall not exceed the values specified in Table 6.3.1-1.

The UE shall meet the requirements in 6.3.1 when PUSCH is scheduled for single antenna-port transmission by DCI 0_0 or by DCI 0_1 when the UE is configured for single port operation.

### 6.3D.2	Transmit OFF power for UL MIMO

The transmit OFF power is defined as the mean power at each transmit antenna connector in a duration of at least one sub-frame (1 ms) excluding any transient periods.

The transmit OFF power at each transmit antenna connector shall not exceed the values specified in Table 6.3.2-1.

### 6.3D.3	Transmit ON/OFF time mask for UL MIMO

The ON/OFF time mask requirements in subclause 6.3.3 apply at each transmit antenna connector.

For UE with two transmit antenna connectors in closed-loop spatial multiplexing scheme, the general ON/OFF time mask requirements specified in subclause 6.3.3.1 apply to each transmit antenna connector. The requirements shall be met with the UL MIMO configurations described in subclause 6.2D.1.

The UE shall meet the requirements in 6.3.3 when PUSCH is scheduled for single antenna-port transmission by DCI 0_0 or by DCI 0_1 when the UE is configured for single port operation.

### 6.3D.4	Power control for UL MIMO

The power control tolerance applies to the sum of output power at each transmit antenna connector.

The power control requirements specified in subclause 6.3.4 apply to UE with two transmit antenna connectors in closed-loop spatial multiplexing scheme. The requirements shall be met with UL MIMO configurations described in subclause 6.2D.1.

The UE shall meet the requirements in 6.3.4 when PUSCH is scheduled for single antenna-port transmission by DCI 0_0 or by DCI 0_1 when the UE is configured for single port operation.

## 6.4	Transmit signal quality

< text omitted >

## 6.4D	Transmit signal quality for UL MIMO

### 6.4D.1	Frequency error for UL MIMO

The basic measurement interval of modulated carrier frequency is 1 UL slot.  The mean value of basic measurements of UE modulated carrier frequency at each transmit antenna connector shall be accurate to within ± 0.1 PPM observed over a period of 1 ms of cumulated measurement intervals compared to the carrier frequency received from the NR Node B.

### 6.4D.2	Transmit modulation quality for UL MIMO

The transmit modulation quality requirements are specified at each transmit antenna connector.

The UE shall meet the requirements in 6.4.2 when PUSCH is scheduled for single antenna-port transmission by DCI 0_0 or by DCI 0_1 when the UE is configured for single port operation..

The transmit modulation quality is specified in terms of:

-	Error Vector Magnitude (EVM) for the allocated resource blocks (RBs)

-	EVM equalizer spectrum flatness derived from the equalizer coefficients generated by the EVM measurement process

-	Carrier leakage (caused by IQ offset)

-	In-band emissions for the non-allocated RB

In case the parameter 3300 or 3301 is reported from UE via txDirectCurrentLocation IE (as defined in TS 38.331 [7]), carrier leakage measurement requirement in subclause 6.4D.2.2 and 6.4D.2.3 shall be waived, and the RF correction with regard to the carrier leakage and IQ image shall be omitted during the calculation of transmit modulation quality.

#### 6.4D.2.1	Error Vector Magnitude

For UE with two transmit antenna connectors in closed-loop spatial multiplexing scheme, the Error Vector Magnitude requirements specified in Table 6.4.2.1-1 which is defined in subclause 6.4.2.1 apply at each transmit antenna connector. The requirements shall be met with the UL MIMO configurations specified in Table 6.2D.1-2

#### 6.4D.2.2	Carrier leakage

For UE with two transmit antenna connectors in closed-loop spatial multiplexing scheme, the Relative Carrier Leakage Power requirements specified in Table 6.4.2.2-1 which is defined in subclause 6.4.2.2 apply at each transmit antenna connector. The requirements shall be met with the UL MIMO configurations specified in Table 6.2D.1-2

#### 6.4D.2.3	In-band emissions

For UE with two transmit antenna connectors in closed-loop spatial multiplexing scheme, the In-band Emission requirements specified in Table 6.4.2.3-1 which is defined in subclause 6.4.2.3 apply at each transmit antenna connector. The requirements shall be met with the uplink MIMO configurations specified in Table 6.2D.1-2

#### 6.4D.2.4	EVM equalizer spectrum flatness for UL MIMO

For UE with two transmit antenna connectors in closed-loop spatial multiplexing scheme, the EVM Equalizer Spectrum Flatness requirements specified in Table 6.4.2.4-1 and Table 6.4.2.4-2 which are defined in subclause 6.4.2.4 apply at each transmit antenna connector. The requirements shall be met with the UL MIMO configurations specified in Table 6.2D.1-2

### 6.4D.3	Time alignment error for UL MIMO

For UE(s) with multiple transmit antenna connectors supporting UL MIMO, this requirement applies to frame timing differences between transmissions on multiple transmit antenna connectors in the closed-loop spatial multiplexing scheme.

The time alignment error (TAE) is defined as the average frame timing difference between any two transmissions on different transmit antenna connectors.

For UE(s) with multiple transmit antenna connectors, the Time Alignment Error (TAE) shall not exceed 130 ns.

### 6.4D.4	Requirements for coherent UL MIMO

For coherent UL MIMO, Table 6.4D.4-1 lists the maximum allowable difference between the measured relative power and phase errors between different antenna ports in any slot within the specified time window from the last transmitted SRS on the same antenna ports, for the purpose of uplink transmission (codebook or non-codebook usage) and those measured at that last SRS. The requirements in Table 6.4D.4-1 apply when the UL transmission power at each antenna port is larger than 0 dBm for SRS transmission and for the duration of time window.

Table 6.4D.4-1: Maximum allowable difference of relative phase and power errors in a given slot compared to those measured at last SRS transmitted

| Difference of relative phase error | Difference of relative power error | Time window |
| --- | --- | --- |
| 40 degrees | 4 dB | 20 msec |

The above requirements when all the following conditions are met within the specified time window:

-	UE is not signaled with a change in number of SRS ports in SRS-config, or a change in PUSCH-config

-	UE remains in DRX active time (UE does not enter DRX OFF time)

-	No measurement gap occurs

-	No instance of SRS transmission with the usage antenna switching occurs

-	Active BWP remains the same

-	EN-DC and CA configuration is not changed for the UE (UE is not configured or de-configured with PSCell or SCell(s))

## 6.5	Output RF spectrum emissions

< text omitted >

## 6.5D	Output RF spectrum emissions for UL MIMO

### 6.5D.1	Occupied bandwidth for UL MIMO

The requirements for occupied bandwidth apply to the transmitted spectrum as measured as the sum of the powers from all UE  antenna connectors. The occupied bandwidth is defined as the bandwidth containing 99 % of the total integrated mean power of the transmitted spectrum on the assigned channel at each transmit antenna connector.

For UE with two transmit antenna connectors in closed-loop spatial multiplexing scheme, the occupied bandwidth as measured as the sum of the powers from all UE antenna connectors shall be less than the channel bandwidth specified in table 6.5.1-1. The requirements shall be met with UL MIMO configurations described in subclause 6.2D.1.

The UE shall meet the requirements in 6.5.1 when PUSCH is scheduled for single antenna-port transmission by DCI 0_0 or by DCI 0_1 when the UE is configured for single port operation.

### 6.5D.2	Out of band emission for UL MIMO

The requirements for Out of band emissions resulting from the modulation process and non-linearity in the transmitters apply to the sum of the emissions from all UE antenna connectors.

For UEs with two transmit antenna connectors in closed-loop spatial multiplexing scheme, the requirements in subclasuse 6.5.2 apply to the sum of the emissions from all  UE antenna connectors. The requirements shall be met with UL MIMO configurations described in subclause 6.2D.1.

The UE shall meet the requirements in 6.5.2 when PUSCH is scheduled for single antenna-port transmission by DCI 0_0 or by DCI 0_1 when the UE is configured for single port operation.

### 6.5D.3	Spurious emission for UL MIMO

The requirements for Spurious emissions which are caused by unwanted transmitter effects such as harmonics emission, parasitic emissions, intermodulation products and frequency conversion products apply to the sum of the emissions from all UE antenna connectors.

For UEs with two transmit antenna connectors in closed-loop spatial multiplexing scheme, the requirements specified in subclasuse 6.5.3 apply to apply to the sum of the emissions from all UE antenna connectors. The requirements shall be met with the UL MIMO configurations described in subclause 6.2D.1.

The UE shall meet the requirements in 6.5.3 when PUSCH is scheduled for single antenna-port transmission by DCI 0_0 or by DCI 0_1 when the UE is configured for single port operation.

### 6.5D.4	Transmit intermodulation for UL MIMO

The transmit intermodulation requirements are specified at each transmit antenna connector and the wanted signal is defined as the sum of output power at each transmit antenna connector.

For UEs with two transmit antenna connectors in closed-loop spatial multiplexing scheme, the requirements specified in subclause 6.5.4 apply to each transmit antenna connector. The requirements shall be met with the UL MIMO configurations described in subclause 6.2D.1.

The UE shall meet the requirements in 6.4.2 when PUSCH is scheduled for single antenna-port transmission by DCI 0_0 or by DCI 0_1 when the UE is configured for single port operation.

< end of changes >