TSG-RAN Working Group 4 (Radio) meeting #94-e	  R4-2001319

Electronic meeting, 20 February – 6 March 2020

Source:	Ericsson

Title:	The NR-U channel raster and allowed intra-cell GB for wideband operation

Agenda item:	8.1.1.2

Document for:	Approval

# Background

RAN4 has agreed on a set of channel-raster points for 20, 40, 60, and 80 MHz carrier bandwidth that closely align (within ± 20 kHz) with the WiFi 20 MHz channel raster [1]. Moreover, for wideband operation, minimum intra-cell guard bands and ‘usable PRB’ per LBT sub-band for which radio requirements apply given the said channel raster were agreed in the WF [2] as detailed in Table 1:

Table 1

This was designated a default pattern. RAN4 also decided that the number of usable RBs per sub-channel and the number of intra-carrier guard PRBs can vary for different SCS due to configuration according to Table 2:

Table 2

An RB shift for PRB grid alignment given the allowed ±200 kHz shift of the Wi-Fi channel raster specified in the EN harmonised standard [3] is FFS. In this contribution we consider the need for channel raster (RB) shift and conclude that it is not needed given the allowed range of intra-cell guard bands in Table 2.

# Shifting the NR-U carrier frequencies and intra-cell guard band restrictions

A nominal channel raster was specified in the v2.1.1 (2017-05) of EN harmonised standard [3] to faciltiate co-channel coexistence between Wi-Fi and LAA by LBT: their center frequencies should be aligned such that the Wi-Fi sub-carrier and the LAA PRB grid overlap but within an allowed maximum offset of ±200 kHz to cater for the LAA nominal carrier spacing (19.8 and 20.1 MHz).

The need for nested guardbands (the guard band of a narrower channel fully contained within the guard band of a wider carrier) has been discussed out of concern with adjancent channel interference between operations with different channel bandwidths in NR-U, LAA and Wi-Fi deployments. However, the risk of harmful out-of-band emission and interference due to blocking and reduced receiver selectivity (FFT only) would only be increased marginally with an offset limited to  ±200 kHz given the spectrum utilization of coexisting technologies.

Figure 1 shows 20 MHz NR-U carriers with a 51 PRB (18.36 MHz) transmission bandwidth on the agreed channel raster (within ±20 kHz of Wi-Fi channel center frequencies) and LAA 20 MHz carriers with 100 PRB (18 MHz) transmission bandwidth; the LAA carriers are shifted to obey the 19.8 MHz and 20.1 MHz nominal carrier spacing. The carriers are just within the allowed ±200 kHz offset, but the nested GB requirement is not useful since the LAA carriers are narrower than the NR-U carriers and fully contained within the grid of the latter.

Figure 1: NR-U/LAA grid alignment.

For NR-U/NR-U the minimum intra-cell guard band is 5 PRB (1.8 MHz) according to Table 1, insisting on the nested guard band property for all possible shifts within the allowed ±200 kHz offset would require a redesign of the raster and larger guard bands for only a maginally reduced risk of interference. Should increased protection (adjacent channel interference rejection) of e.g. CORESET#0 with carriers on the agreed channel raster be desired nevertheless, a larger guard band could be configured in accordance with Table 2. A similar concusion can be drawn for coexistence with a Wi-Fi carrier with a spectrum utilization comparable to that of NR-U

We observe that

Observation 1: shifting the carrier frequencies within a ±200 kHz offset is not useful for NR-U/LAA coexistence since the spectrum utilization of LAA is smaller than that of NR-U.

Observation 2: shifting the carrier frequencies within a ±200 kHz offset is not useful for NR-U/NR-U coexistence, should increased protection of e.g. CORESET#0 be desired nevertheless, a larger guard band could be configured with carriers at the agreed channel raster.

Another reason for carrier grid alignment can be sharing of radio between NR-U and LAA, but since the LTE channel raster grid spacing (100 kHz) is not divisible by the NR channel raster grid spacing (15 kHz), we can immediately make the following

Observation 3: alignment of NR-U and LAA subcarrier grids by shifting, notwithstanding a possible difference in SCS, can not be achieved since NR-U carriers can only be shifted in terms of NR-ARFCN steps.

and we propose

Proposal 1: a nominal channel raster for which minimum requirements apply is specified without a ±200 kHz shift (offset); increased adjacent interference rejection between NR-U and victims, if desired, can be achieved by increasing the guard within the restrictions already agreed by RAN4.

The restrictions on the guard bands and usable PRB (RB-sets) in Table 2 are still needed for making sure that NR-U PRB grids overlap with Wi-Fi sub-carrier grids (co-channel interference by LBT) while keeping a separation between sub-carrier grids in adjacent LBT sub-bands (reduce risk adjacent channel interference). There may not be any restriction on the guard-band size in the RRC specification, but the minimum requirements specified by RAN4 only apply for carriers on the nominal channel raster with intra-cell guard bands as detailed in Table 1 and Table 2. The inter-cell guard bands should be as specified for licensed bands.

# Proposed changes to the specifications

We propose not to add a ±200 kHz offset to the channel raster. The nominal intra-cell guard bands can be specified as follows:

< start of changes to 38.101-1 >

## 5.3	UE channel bandwidth

### 5.3.1	General

The UE channel bandwidth supports a single NR RF carrier in the uplink or downlink at the UE. From a BS perspective, different UE channel bandwidths may be supported within the same spectrum for transmitting to and receiving from UEs connected to the BS. Transmission of multiple carriers to the same UE (CA) or multiple carriers to different UEs within the BS channel bandwidth can be supported.

From a UE perspective, the UE is configured with one or more BWP / carriers, each with its own UE channel bandwidth. The UE does not need to be aware of the BS channel bandwidth or how the BS allocates bandwidth to different UEs.

The placement of the UE channel bandwidth for each UE carrier is flexible but can only be completely within the BS channel bandwidth.

The relationship between the channel bandwidth, the guardband and the maximum transmission bandwidth configuration is shown in Figure 5.3.1-1.

Figure 5.3.1-1: Definition of the channel bandwidth and the maximum transmission bandwidth configuration for one NR channel

### 5.3.2	Maximum transmission bandwidth configuration

The maximum transmission bandwidth configuration NRB for each UE channel bandwidth and subcarrier spacing is specified in Table 5.3.2-1.

Table 5.3.2-1: Maximum transmission bandwidth configuration NRB

| SCS (kHz) | 5 MHz | 10 MHz | 15 MHz | 20 MHz | 25 MHz | 30 MHz | 40 MHz | 50 MHz | 60 MHz | 70 MHz | 80 MHz | 90 MHz | 100 MHz |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | NRB | NRB | NRB | NRB | NRB | NRB | NRB | NRB | NRB | NRB | NRB | NRB | NRB |
| 15 | 25 | 52 | 79 | 106 | 133 | 160 | 216 | 270 | N/A | N/A | N/A | N/A | N/A |
| 30 | 11 | 24 | 38 | 51 | 65 | 78 | 106 | 133 | 162 | 189 | 217 | 245 | 273 |
| 60 | N/A | 11 | 18 | 24 | 31 | 38 | 51 | 65 | 79 | 93 | 107 | 121 | 135 |

### 5.3.3	Minimum guardband and transmission bandwidth configuration

The minimum guardband for each UE channel bandwidth and SCS is specified in Table 5.3.3-1,

Table 5.3.3-1: Minimum guardband for each UE channel bandwidth and SCS (kHz)

| SCS (kHz) | 5 MHz | 10 MHz | 15 MHz | 20 MHz | 25 MHz | 30 MHz | 40 MHz | 50 MHz | 60 MHz | 70 MHz | 80 MHz | 90 MHz | 100 MHz |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 15 | 242.5 | 312.5 | 382.5 | 452.5 | 522.5 | 592.5 | 552.5 | 692.5 | N/A | N/A | N/A | N/A | N/A |
| 30 | 505 | 665 | 645 | 805 | 785 | 945 | 905 | 1045 | 825 | 965 | 925 | 885 | 845 |
| 60 | N/A | 1010 | 990 | 1330 | 1310 | 1290 | 1610 | 1570 | 1530 | 1490 | 1450 | 1410 | 1370 |

NOTE:	The minimum guardbands have been calculated using the following equation: (BWChannel x 1000 (kHz) - NRB  x SCS x 12) / 2 - SCS/2, where NRB  are from Table 5.3.2-1.

Figure 5.3.3-1: Void

The number of RBs configured in any channel bandwidth shall ensure that the minimum guardband specified in this clause is met.

Figure 5.3.3-2: UE PRB utilization

In the case that multiple numerologies are multiplexed in the same symbol due to BS transmission of SSB, the minimum guardband on each side of the carrier is the guardband applied at the configured channel bandwidth for the numerology that is received immediately adjacent to the guard.

If multiple numerologies are multiplexed in the same symbol and the UE channel bandwidth is >50 MHz, the minimum guardband applied adjacent to 15 kHz SCS shall be the same as the minimum guardband defined for 30 kHz SCS for the same UE channel bandwidth.

Figure 5.3.3-3 Guard band definition when transmitting multiple numerologies

NOTE:	Figure 5.3.3-3 is not intended to imply the size of any guard between the two numerologies. Inter-numerology guard band within the carrier is implementation dependent.

If for operations with shared spectrum access [alt: in Band n46] a UE supporting [wideband operation] is configured with intra-cell guard bands for channel bandwidths greater than 20 MHz as specified in [38.211 or 38.214], the nominal guard bands and the corresponding sizes (transmission bandwidth configuration) of the RB sets separated by the said guard bands are as specified in Table 5.3.3-2 for each UE channel bandwidth and sub-carrier spacing for the downlink and uplink. The intra-cell guard band configuration in Table 5.3.3-2 is the default indication of the IE [intraCellGuardBands] in [38.331].

Table 5.3.3-2: Nominal intra-cell guard bands for wideband operation

| SCS (kHz) | 20 MHz | 40 MHz | 60 MHz | 80 MHz | 100 MHz |  |
| --- | --- | --- | --- | --- | --- | --- |
| 15 | 106 (106) | [105-6-105] (216) | N/A | N/A | N/A |
| 30 | 51 (51) | [50-6-50] (106) | [50-6-50-6-50] (162) | [50-6-50-5-50-6-50] (217) | TBD |  |
| [60] | 24 (24) | [23-5-23] (51) | [23-5-23-5-23] (79) | [23-5-23-5-23-5-23] (107) |  |  |
| [60 (alt)] | [25] (25) | [24-5-24] (51) | [24-3-25-3-24] (79) | [24-4-24-3-24-4-24] (107) |  |  |
| NOTE 1: The intra-cell guard band configuration is denoted TBW0-GB0-…-GBN_RBset-2-TBWN_RBset-1 for N_RBset > 1 number of RB-sets with TBWr the maximum transmission bandwidth (PRB) of RB-set r and GBr the guard band (PRB) above the upper edge of RB-set r. The RB-set 0 is starting at the first common resource block (CRB) of the resource grid. The total transmission bandwidth configuration (size of resource grid) including guard bands is given in between brackets. NOTE 2: CRB indices of [the intra-cell guard bands and RB sets] are configured by the IE [intraCellGuardBands] [38.331] |  |  |  |  |  |

For each UE channel bandwidth and sub-carrier spacing, the maximum transmission bandwidth configuration including intra-cell guard band(s), if configured, and corresponding RB-set(s) shall be in accordance with sub-clause 5.3.2 with a minimum inter-cell guard band of the UE channel bandwidth as specified in Table 5.3.3-1 for the uplink and downlink. Minimum requirements specified for wideband operation in Clause 6 and Clause 7 also apply for intra-cell guard bands larger than the nominal sizes in Table 5.3.3-2 as listed in Table 5.3.3-3 for each sub-carrier spacing; each guard band in order of CRB index must be larger than or equal to the corresponding nominal guard band specified in Table 5.3.3-2 for each channel bandwidth.

Table 5.3.3-3: Applicable intra-cell guard bands for wideband operation

| Parameter | Unit | SCS |  |  |
| --- | --- | --- | --- | --- |
|  |  | 15 kHz | 30 kHz | 60 kHz |
| GB | PRB | [6,7] | [5,6,7] | [3,4,5] |
| Transmission bandwidth (size) of RB-set | PRB | [104,105] | [49,50,51] | [22,23,24] |

If the UE is not configured with intra-cell guard bands in the uplink and/or downlink, the maximum transmission bandwidth configuration for the uplink and downlink shall be in accordance with sub-clause 5.3.2 with a minimum inter-cell guard band of the UE channel bandwidth as specified in Table 5.3.3-1.

### 5.3.4	RB alignment

< end of changes to 38.101-1 >

# Proposal

We make the following proposals

Proposal 1: a nominal channel raster for which minimum requirements apply is specified without a ±200 kHz shift (offset); increased adjacent interference rejection between NR-U and victims, if desired, can be achieved by increasing the guard within the restrictions already agreed by RAN4.

Proposal 2: changes to 38.101-1 in accordance with Section 3.

# References

1.    R4-1916167, “dCR channel raster in band n46 for NR-U operation”, Qualcomm Incorporated2.    R4-1916160, “WF on Guardbands for NR-U”, Nokia

3.    ETSI EN 301 893 v2.1.1