3GPP TSG RAN WG4 Meeting#94-e															R4-2001329

E-meeting, February 24th – March 6th, 2020

Source:	Nokia, Nokia Shanghai Bell

Title:	On QCL Chain

Agenda item:	6.10.1

Document for:	Approval

# 1	Introduction

RAN4 introduced applicability of QCL in section 3.6.7 a couple of meetings ago. At the point of agreement, it was noted that agreed text was not completely clear according to the common understanding of what QCL chain covered.

In last meeting we brought a CR trying to clarify the wording according to the understanding during the agreement. However, it was not agreed. In this paper we discuss what is not clear in the current applicability rule and propose an update of the wording.

# 2	Discussion

RAN4 introduced a maximum depth in chain of QCL assumptions (TCI states) as follows [3GPP TS 38.133]:

3.6.7 Applicability of QCL For the requirements specified in this version of the specification, a reference signal is considered to be QCLed to another reference signal if it is in the same TCI chain as the other reference signal, provided that the number of Reference Signals in the chain is no more than 4. A TCI chain consists of an SSB, and one or more CSI-RS resources, and the TCI state of each Reference Signal includes another Reference Signal in the same TCI chain. DMRS of PDCCH or PDSCH is QCLed with the reference signal in its active TCI state and any other reference signal that is QCLed, based on above criteria, with the reference signal in the active TCI state.

Based on the specification text it is unclear what actually means “the number of Reference Signals in the chain is no more than that 4”. This was already addressed at the time when the text was agreed, and understanding was that it should be clarified.

It is to be noted that a TCI state may have two QCL source RSs, for instance one for QCL-TypeA and the other QCL-TypeD. Now, if one counted the reference signals providing different QCL types, the maximum number of 4 would be fulfilled already by two TCI states. However, that would be restricting the beamforming opportunities and flexibility significantly. For example, Let’s consider e.g. a multi-TRP case where SSB beam is a wide beam (e.g. superposition beam of the multiple TRPs) and targeted transmit beam for PDCCH and PDSCH a very narrow beam as illustrated in Figure 1.

Figure 1 SSB beam pattern and beam pattern for PDCCH/PDSCH.

For this case, the steps described in Figure 2 would be needed where it’s assumed that due to discrepancy in SSB and selected P2 CSI-RS beam is significant a TRS is configured upon narrow transmit beam for PDCCH/PDSCH.

Figure 2 Steps to configure TRS upon narrow (based on P2 CSI-RS) transmit beam.

Above described configuration would require the following QCL chain, e.g. for QCL-TypeD, for the PDCCH/PDSCH DMRS reception:

- SSBTRSCSI-RS with repetitionTRS

The depth of the QCL chain is 4 and the number of TCI states in the TCI chain is also 4. Thus, it is proposed to clarify in the specification that the QCL chain depth restriction is for the certain QCL type. This clarification would be aligned with the understanding in the group when agreeing the text in 38.133.

QCL chain depth restriction is for the certain QCL type.

Agree to following text proposal to 38.133, section 3.6.7.

## 2.1 Text Proposal

| 3.6.7 Applicability of QCL For the requirements specified in this version of the specification, a reference signal is considered to be QCLed to another reference signal if it is in the same TCI chain as the other reference signal, provided that the number of Reference Signals of the same QCL type in the chain is no more than 4. A TCI chain consists of an SSB, and one or more CSI-RS resources, and the TCI state of each Reference Signal includes another Reference Signal in the same TCI chain. DMRS of PDCCH or PDSCH is QCLed with the reference signal in its active TCI state and any other reference signal that is QCLed, based on above criteria, with the reference signal in the active TCI state. |  |
| --- | --- |

# 3	Conclusions

In this paper we have discussed the clear part of the current applicability of QCL rule and propose an update of the wording.

- QCL chain depth restriction is for the certain QCL type.

Agree to following text proposal to 38.133, section 3.6.7.

| 3.6.7 Applicability of QCL For the requirements specified in this version of the specification, a reference signal is considered to be QCLed to another reference signal if it is in the same TCI chain as the other reference signal, provided that the number of Reference Signals of the same QCL type in the chain is no more than 4. A TCI chain consists of an SSB, and one or more CSI-RS resources, and the TCI state of each Reference Signal includes another Reference Signal in the same TCI chain. DMRS of PDCCH or PDSCH is QCLed with the reference signal in its active TCI state and any other reference signal that is QCLed, based on above criteria, with the reference signal in the active TCI state. |  |
| --- | --- |

# References

- 38.133.