3GPP TSG-RAN WG4 Meeting#94-e                	R4-2001341

e-Meeting, February 24th – March 6th, 2020

Source:	Nokia, Nokia Shanghai Bell

Title:	NR Inter-RAT measurements for early measurement reporting

Agenda item:	8.6.3.2.2

Document for:	Approval

Introduction

One goal of the LTE_NR_DC_CA_enh WI is to introduce the early reporting feature as defined in LTE into NR. Additional goal is to enable early reporting of NR inter-RAT carriers into the existing LTE measurements for early reporting feature. In this paper we discuss the principles behind such introduction.

Discussion

In last RAN4#93 meeting [3] provided input on how to capture the new requirements needed to enable configuring a UE and UE measurement requirements related to support of early reporting of NR inter-RAT measurement, when UE is operating in LTE SA. The purpose of configuring the UE to early report of NR inter-RAT would be to enable network to faster configure the UE in EN-DC mode.

Configuration Scenarios

In the RAN4#93 meeting in Reno it was agreed to not discuss further the aspect of UE capability related to the different scenarios and what the UE should support:

- UE supporting ca-IdleModeMeasurements and measurement of NR inter-RAT

  - A UE supporting ca-IdleModeMeasurements (optional) and support EN-DC (optional) and supporting early measurements and reporting of NR inter-RAT cells.

  - A UE supporting ca-IdleModeMeasurements (optional) and support EN-DC (optional) not supporting early measurements and reporting of NR inter-RAT cells.

- UE not supporting ca-IdleModeMeasurements but supporting measurement of NR inter-RAT

  - A UE supporting EN-DC (optional) and supporting measurements of NR-inter-RAT for early reporting.

  - A UE supporting EN-DC (optional) not support measurements of NR-inter-RAT for early reporting (optional).

As it is not expected that the discussion will be finalized before late in WI stage, we suggest continuing the necessary RAN4 work and define the RAN4 requirement covering each case. For cases 1.b an 2.b there would be no changes to the existing requirements and RAN4 only need to consider case 1.a and 2.a

Define RAN4 requirements for an EN-DC capable UE supporting ca-IdleModeMeasurements and supporting early measurements and reporting of NR inter-RAT cells.

Define RAN4 requirements for an EN-DC capable UE not supporting ca-IdleModeMeasurements and supporting measurements of NR-inter-RAT for early reporting.

Number of NR Inter-RAT carriers

Next to discuss is the number of NR inter-RAT carriers the UE supporting NR inter-RAT measurements for early reporting should be able to measure as minimum.

Hence, for the UE which do support early measurement reporting (EMR) of NR inter-RAT carrier, RAN4 need to discuss the number of carriers for two cases:

- UE support EMR of NR inter-RAT carriers in addition to ca-IdleModeMeasurements.

- UE only support EMR of NR inter-RAT carriers.

EMR of NR Inter-RAT and LTE carriers

For option1, this means that the UE supporting ca-IdleModeMeasurements and EN-DC should support EMR for LTE inter-frequency and NR inter-RAT measurements in idle mode. Hence, for this case additional RAN4 measurement requirements are added (NR inter-RAT measurements).

Two options were listed regarding how the requirement could look like:

- For a UE supporting EMR of NR inter-RAT carriers in addition to ca-IdleModeMeasurements, existing number of LTE carriers as defined under ca-IdleModeMeasurements, are kept unchanged

- For a UE supporting EMR of NR inter-RAT carriers in addition to ca-IdleModeMeasurements, the UE support a number of EMR carriers which may be LTE or NR (total number of euCA EMR carriers remain unchanged)

The number of carriers supported for EMR is already very limited and sharing the carriers among LTE and NR might be limiting the benefit of the feature and possible gain. We see that the principle should be that the requirements already defined for ca-IdleModeMeasurements should remain unchanged. The UE should then additionally support one or more NR inter-RAT EMR measurements.

For a UE supporting EMR of NR inter-RAT carriers in addition to ca-IdleModeMeasurements, the existing number of LTE carriers as defined under ca-IdleModeMeasurements are kept unchanged.

For a UE supporting EMR of NR inter-RAT carriers in addition to ca-IdleModeMeasurements shall support NR inter-RAT measurements on one or more NR carriers in addition to the existing inter-frequency carriers as defined under ca-IdleModeMeasurements.

EMR of NR inter-RAT carriers only

For option2 where the UE only support EMR of NR inter-RAT carriers would mean that the UE would not need to LTE inter-frequency carriers for EMR.

- For a UE supporting EMR of NR inter-RAT carriers but not supporting ca-IdleModeMeasurements, shall support measurements on a number of NR carriers

In this case RAN4 only need to discuss the number of NR inter-RAT carriers the UE at least should be able to measure for EMR.

For a UE supporting EMR of NR inter-RAT carriers and not supporting ca-IdleModeMeasurements, shall support measurements on one or more NR carriers.

Number of carriers

The simplest approach will of course be to define that the UE should be able measure the same number of NR inter-RAT carriers for EMR in both options discussed above.

One way to achieve this would be to define a maximum number of carriers to be measured for EMR while not defining the explicit number of inter-frequency or inter-RAT carriers. For example, the UE is able to measure at least X carriers for EMR and it is up to network configuration to distribute this between LTE and NR carriers. E.g. UE support EMR on 3 carriers of which [a] can be overlapping and [y] can be non-overlapping.

A UE supporting LTE EMR would of course be following the LTE EMR requirements as baseline.

Another way is to explicitly define the number of NR inter-RAT carriers for the two options. We suggest following for the two cases:

- The UE supporting EMR of NR inter-RAT carriers and ca-IdleModeMeasurements shall support the LTE EMR requirements and NR EMR of at least 2 NR inter-RAT carriers of which zero or one NR inter-RAT carrier may be a non-overlapping carrier and at most 1 NR inter-RAT carrier can be overlapping.

- The UE only supporting EMR of NR inter-RAT carriers shall support NR EMR of at least 2 NR inter-RAT carriers of which zero or one NR inter-RAT carrier may be a non-overlapping carrier and at most 1 NR inter-RAT carrier can be overlapping.

Hence, we propose following regarding number of EMR carriers the UE at least shall be able support:

The UE supporting EMR of NR inter-RAT carriers and ca-IdleModeMeasurements shall support the LTE EMR requirements and NR EMR of at least 2 NR inter-RAT carriers of which zero or one NR inter-RAT carrier may be a non-overlapping carrier and at most 1 NR inter-RAT carrier can be overlapping.

The UE only supporting EMR of NR inter-RAT carriers shall support NR EMR of at least 2 NR inter-RAT carriers of which zero or one NR inter-RAT carrier may be a non-overlapping carrier and at most 1 NR inter-RAT carrier can be overlapping.

Detected Cell Status

Another important aspect to discuss is the requirement for the detected cell status for EMR when the UE transitions from RRC Connected mode to Idle or Inactive mode and after UE has entered Idle/Inactive mode.

The existing requirements under ca-IdleModeMeasurements as baseline, defines requirements for the detected cell status for the idle mode CA measurement when UE transitions from RRC Connected mode to Idle mode and after UE has entered Idle mode in 36.133 section 4.9.2.1.

We believe it is important to keep the same principles for NR inter-RAT EMR. Namely that given certain conditions the NR cell, which is a detected cell in Connected mode prior to connection release, shall remain detected after UE enters Idle/Inactive mode and during Idle/Inactive mode. Keeping this principle is important to ensure reasonable measurement delay for EMR.

An NR inter-RAT cell which is a detected cell in Connected mode prior to connection release, shall remain detected after UE has entered Idle/Inactive mode.

Additional Considerations

While the LTE inter-frequency measurements to performed and reported are clear (already defined), the NR inter-RAT measurements are not yet defined. A number of baseline assumptions can be made:

- NR inter-RAT measurements for early reporting only rely on SSB based measurements.

- UE can be configured to report with and without Index.

- Beam related relaxations need to be accounted.

Requirements, and whether they differ based on UE measurement capability, should also be discussed further in RAN4.

Conclusion

One goal of the LTE_NR_DC_CA_enh WI is to introduce the early reporting feature as defined in LTE into NR. This includes enabling early reporting of NR inter-RAT carriers in LTE measurements for early reporting feature. In this paper we have discussed the principles such introduction could be done.

- Define RAN4 requirements for an EN-DC capable UE supporting ca-IdleModeMeasurements and supporting early measurements and reporting of NR inter-RAT cells.

Define RAN4 requirements for an EN-DC capable UE not supporting ca-IdleModeMeasurements and supporting measurements of NR-inter-RAT for early reporting.

For a UE supporting EMR of NR inter-RAT carriers in addition to ca-IdleModeMeasurements, the existing number of LTE carriers as defined under ca-IdleModeMeasurements are kept unchanged.

For a UE supporting EMR of NR inter-RAT carriers in addition to ca-IdleModeMeasurements shall support NR inter-RAT measurements on one or more NR carriers in addition to the existing inter-frequency carriers as defined under ca-IdleModeMeasurements.

For a UE supporting EMR of NR inter-RAT carriers and not supporting ca-IdleModeMeasurements, shall support measurements on one or more NR carriers.

The UE supporting EMR of NR inter-RAT carriers and ca-IdleModeMeasurements shall support the LTE EMR requirements and NR EMR of at least 2 NR inter-RAT carriers of which zero or one NR inter-RAT carrier may be a non-overlapping carrier and at most 1 NR inter-RAT carrier can be overlapping.

The UE only supporting EMR of NR inter-RAT carriers shall support NR EMR of at least 2 NR inter-RAT carriers of which zero or one NR inter-RAT carrier may be a non-overlapping carrier and at most 1 NR inter-RAT carrier can be overlapping.

An NR inter-RAT cell which is a detected cell in Connected mode prior to connection release, shall remain detected after UE has entered Idle/Inactive mode.

References

- R4-1912121, Introduction of NR Inter-RAT measurements for early measurement reporting, Nokia, Alcatel-Lucent Shanghai Bell

- R4-1912122, Introduction of NR Inter-RAT measurement for Idle Mode CA measurements, Nokia, Alcatel-Lucent Shanghai Bell

- R4-1914481, Discussion on NR Inter-RAT measurements for early measurement reporting, Nokia, Alcatel-Lucent Shanghai Bell

- R4-1915842, WF on MR-DC Early measurement reporting, Nokia, Alcatel-Lucent Shanghai Bell