3GPP TSG-RAN WG4 Meeting #94-e	R4-2001349

Electronic meeting, 24 February – 6 March 2020

Agenda Item:	7.10.3.1

Source:	Ericsson

Title:	Discussion on the remaining issues on DL quality report for eMTC

Document for:	Discussion

# 1	Introduction

RAN4#93 agreed with the way forward on the DL channel quality reporting [1]. This contribution discusses the remaining open issues for DL channel quality report for eMTC.

- For 2-bit DL quality report mapping - MPDCCH Aggregation level is assumed to be 24. - Wait for RAN2 decision on whether 3 or 4 values can be reported - For the DL quality report accuracy - When RL is reported, i.e., MPDCCH AL=24 MPDCCH repetition; Pm-Dsg (%); Conditions. R (Note 1); ≤ 1; -6 ≤ Es/Iot (Note 2). R/[4]; > 1; -6 ≤ Es/Iot (Note 2). R; ≤ 1; -15 ≤ Es/Iot ≤ -6. R/[8]; > 1; -15 ≤ Es/Iot ≤ -6. Note 1: R is the reported MPDCCH repetition level that UE has reported. Note 2: FFS whether to set the upper limit, e.g., 3dB. - For FFS in Note 2: - Option 1: -6 ≤ Es/Iot (No upper limit) - Option 2: -6 ≤ Es/Iot ≤ -3 dB - When RL is reported, i.e., MPDCCH RL=1 MPDCCH aggregation; Pm-Dsg (%); Conditions. Lk (Note 1); ≤ 1; FFS. Lk-[2]; > 1; FFS. Note 1: Lk∈1,2,4,8,12,16,24 is the reported MPDCCH AL level where k is the index to the AL set 1 ≤ k ≤ 7.. - For FFS in the conditions, - Option 1: Es/Iot ≥ -6 dB - Option 2: Es/Iot ≥ [3] dB

# 2	Discussion

## 2.1	Side condition of DL quality reporting accuracy

One of the remaining issues is the side condition of the measurement quality requirements. Figure 1 shows the simulation results of MPDCCH BLER with AL={4,8,16,24} and Rep=1 under the static channel condition.

Figure 1	MPDCCH BLER with fixed repetition level=1 under static channel.

Considering the impairment margin of 2dB, UE can achieve MPDCCH BLER of 1% with SNR=-5.5dB with MPDCCH AL24 and Rep1. This means UE will report the aggregation level with fixed Rep=1 with SNR > -5.5dB. We propose to set Es/Iot ≥ -3 dB as the side condition of the case RL=1 considering the margin.

Regarding the side condition of AL=24, it is observed from our simulation result that UE will report the repetition number with fixed AL=24 with SNR < -5.5dB, and thus we propose to set upper limit as Es/Iot < -3dB, considering the margin.

Proposal 1: Set -6 ≤ Es/Iot ≤ -3 dB as the side condition to report the repetition level with the fixed AL=24.

Proposal 2: Set Es/Iot ≥ -3 dB as the side condition to report the aggregation level with the fixed RL=1.

## 2.2	8-bit reporting

RAN4 has received the LS from RAN1 on the 8-bit DL quality reporting for eMTC [2], as shown in Table 1. One of the issues of this table is that CE Mode B UE cannot report repetition level 1. Since we think CE Mode B UE can also report repetition level 1 during the operation, we propose to specify the mapping table as shown in Table 2.

Table 1	Reporting value (8-bit) agreed in RAN1.

| Reported level | MPDCCH repetition level | MPDCCH aggregation level | CE mode |
| --- | --- | --- | --- |
| 0 | No measurement reporting | No measurement reporting | A, B |
| 1 | 1 | 1 | A |
| 2 | 1 | 2 | A |
| 3 | 1 | 4 | A |
| 4 | 1 | 8 | A |
| 5 | 1 | 12 | A |
| 6 | 1 | 16 | A |
| 7 | 1 | 24 | A |
| 8 | 2 | 24 | A, B |
| 9 | 4 | 24 | A, B |
| 10 | 8 | 24 | A, B |
| 11 | 16 | 24 | A, B |
| 12 | 32 | 24 | A, B |
| 13 | 64 | 24 | A, B |
| 14 | 128 | 24 | A, B |
| 15 | 256 | 24 | A, B |

Table 2	Proposed reporting value (8-bit) to be specified in TS36.133.

| Reported level | MPDCCH repetition level | MPDCCH aggregation level | CE mode |
| --- | --- | --- | --- |
| 0 | No measurement reporting | No measurement reporting | A, B |
| 1 | 1 | 1 | A |
| 2 | 1 | 2 | A |
| 3 | 1 | 4 | A |
| 4 | 1 | 8 | A |
| 5 | 1 | 12 | A |
| 6 | 1 | 16 | A |
| 7 | 1 | 24 | A, B |
| 8 | 2 | 24 | A, B |
| 9 | 4 | 24 | A, B |
| 10 | 8 | 24 | A, B |
| 11 | 16 | 24 | A, B |
| 12 | 32 | 24 | A, B |
| 13 | 64 | 24 | A, B |
| 14 | 128 | 24 | A, B |
| 15 | 256 | 24 | A, B |

Propose 3: Report level 7 (RL=1 and AL24) is also applicable for CE Mode B UE.

## 2.3	2-bit reporting

The last remaining issue is the 2-bit reporting table. According to RAN2 discussion, they are still discussing whether the 2-bit channel quality report is supported or not. We propose RAN4 to wait for RAN2 decisions.

Proposal 4: RAN4 wants for RAN2 decision whether 2-bit channel quality report is supported or not.

# 3	Summary

Proposal 1: Set -6 ≤ Es/Iot ≤ -3 dB as the side condition to report the repetition level with the fixed AL=24.

Proposal 2: Set Es/Iot ≥ -3 dB as the side condition to report the aggregation level with the fixed RL=1.

Propose 3: Report level 7 (RL=1 and AL24) is also applicable for CE Mode B UE.

Proposal 4: RAN4 wants for RAN2 decision whether 2-bit channel quality report is supported or not.

# 4	References

- R4-1915889, “Way forward on Rel-16 MTC RRM enhancement”, Ericsson, Nokia, Nokia Shanghai Bell, Qualcomm Incorporated, Huawei, HiSilicon.

- R1-1911386, “LS on channel quality report for LTE-MTC”, RAN1.