3GPP TSG-RAN WG4 Meeting #94-e	R4-2001354

Electronic meeting, 24 February – 6 March 2020

Agenda Item:	8.18.1.1

Source:	Ericsson

Title:	Simulation result of NR PDSCH demodulation requirements with CA

Document for:	Discussion

# 1	Introduction

RAN4#93 agreed with the way forward on NR PDSCH demodulation requirement for CA [1] and also agreed with the simulation assumption [2]. We show our initial simulation results to decide the MCS/Rank for FR2.

# 2	Simulation results

## 2.1	FR1 FDD 15 kHz

Table 1 shows the SNRs to achieve 70% of the maximum throughput for FR1 FDD SCS=15kHz with MCS13, which is the same result as our previous contribution [3].

Table 1	SNRs to achieve 70% of the maximum throughput for FR1 FDD SCS=15kHz.

|  | 5MHz | 10MHz | 15MHz | 20MHz | 25MHz | 30MHz | 40MHz | 50MHz |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 2Rx | 10.1 | 10.1 | 10.3 | 10.5 | 10.6 | 10.7 | 10.9 | 11.3 |
| 4Rx | 6.6 | 6.5 | 6.6 | 6.7 | 6.8 | 6.8 | 6.9 | 7.2 |

## 2.2	FR1 TDD 15 kHz

Table 2 shows the SNRs to achieve 70% of the maximum throughput for FR1 TDD SCS=15kHz with MCS13.

Table 2	SNRs to achieve 70% of the maximum throughput for FR1 TDD SCS=15kHz.

|  | 5MHz | 10MHz | 15MHz | 20MHz | 25MHz | 30MHz | 40MHz | 50MHz |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 2Rx | 10.37 | 10.52 | 10.65 | 10.68 | 10.91 | 11.02 | 11.27 | 11.53 |
| 4Rx | 6.79 | 6.87 | 6.97 | 6.9 | 7.09 | 7.09 | 7.14 | 7.4 |

## 2.3	FR1 TDD 30 kHz

Table 3 shows the SNRs to achieve 70% of the maximum throughput for FR1 TDD SCS=30kHz with MCS13, which is the same result as our previous contribution [3].

Table 3	SNRs to achieve 70% of the maximum throughput for FR1 TDD SCS=30kHz.

|  | 5MHz | 10MHz | 15MHz | 20MHz | 25MHz | 30MHz | 40MHz | 50MHz | 60MHz | 80MHz | 90MHz | 100MHz |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 2Rx | 10.2 | 10.4 | 10.3 | 10.5 | 10.6 | 10.7 | 11.0 | 11.0 | 11.3 | 11.4 | 11.3 | 11.7 |
| 4Rx | 6.5 | 6.6 | 6.6 | 6.7 | 6.9 | 6.8 | 6.9 | 7.2 | 7.2 | 7.4 | 7.3 | 7.7 |

## 2.4	FR2 TDD 120 kHz

For FR2 TDD SCS=120kHz, there are two options for MCS and rank.

- Option 1: MCS 13 (16QAM 1/2) with rank 1

- Option 2: MCS10 (16QAM 1/3) with rank 2

For FR2, when we decide the test parameters, it is important to consider the achievable SNR levels on OTA as shown in Appendix.

Table 4 shows the SNRs to achieve 70% of the maximum throughput for each parameter option. According to the TE vendor inputs, the smallest achievable SNR level is 10dB in the case of n260 with 800MHz CBW. If we consider the impairment margins etc, it is safer to choose MCS/rank so that SNR < 8.0dB. According to our simulation results and considering the maximizing the maximum throughput, we propose to set MCS10 with rank 2.

Table 4	Required SNRs to achieve 70% of the maximum throughput.

| MCS | Rank | 50MHz | 100MHz | 200MHz | 400MHz |
| --- | --- | --- | --- | --- | --- |
| 10 (16QAM 1/3) | 2 | 7.14 | 7.05 | 7.05 | 7.12 |
| 13 (16QAM 1/2) | 1 | 4.81 | 4.7 | 4.39 | 4.42 |

Proposal 1: MCS10 with rank 2 for the PDSCH demodulation requirements with CA in FR2.

# 3	Summary

Proposal 1: MCS10 with rank 2 for the PDSCH demodulation requirements with CA in FR2.

# References

- R4-1915861, “Way forward on PDSCH CA normal demodulation requirements”, Intel.

- R4-1915863, “Simulation assumption for NR CA PDSCH normal demodulation requirements”, Intel.

- R4-1914347, “Simulation results of NR PDSCH demodulation requirements with CA”, Ericsson.

# Appendix	Achievable SNR levels on OTA

| Operating band | SNR (dB) / Aggregated Channel bandwidth |  |  |
| --- | --- | --- | --- |
|  | 200 MHz | 400 MHz | 800 MHz |
| n257 | 21.8 | 18.8 | 15.8 |
| n258 | 21.8 | 18.8 | 15.8 |
| n260 | 16 | 13 | 10 |
| n261 | 21.8 | 18.8 | 15.8 |