3GPP TSG-RAN WG4 Meeting #94-e	R4-2001358

Electronic, Meeting, 24 February – 6 March 2020

Agenda Item:	8.17.2.1.3

Source:	Ericsson

Title:	Discussion on PDSCH demodulation performance with HST single tap

Document for:	Discussion

# 1	Introduction

One of the remaining issues in HST WI is NW assistance signaling for HST single tap. RAN4#93 discussed the several options as captured in the way forward [1]. We discuss the remaining issue for the UE demodulation requirements with HST single tap.

HST single tap​ - Single tap requirements definition​ - Option A: Define requirements based on worst case and UE performs multi-shot TRS-based time/freq tracking​ - Option B: Define requirements under assumption UE is aware on HST single tap conditions and can adjust time/freq tracking algorithms ​ - Further study how UE can become aware on conditions​ - Option 1: UE detects the conditions​ - Option 2: Rely on agreed NR HST RRM enhancement network assistance signaling​ - Option 3: Additional network assistance is provided​ - Further study UE demodulation performance under HST single tap conditions for the case of single-shot and multi-shot TRS-based tracking​

# 2	Discussion

In our understanding there are two issues on HST single tap scenarios:

- Whether to introduce a network assisted signaling addressing the scenario the network supports the joint transmission (JT) and dynamic point switching (DPS) in the HST-SFN deployment.

- Doppler shift estimation based on one TRS transmission (single-shot TRS) or multiple TRS transmissions (multi-shot TRS).

Firstly RAN4 has already agreed to introduce a network assistance signaling to inform UE on SFN deployment with joint transmission (JT) scheme. Since this is a per-cell signaling, the network may broadcast this signaling even if it switches between JT and DPS per UE.

Figure 1 illustrates an example of TCI configuration supporting JT and DPS under the HST-SFN deployment. In this example, we assume 3 TRPs. For JT, the network configures one common TRS0/TCI0, transmitted from three TRPs with the same resource. On the other hand, the network configures TRS1/TCI1, TRS2/TCI2, and TRS3/TCI3 for the purpose of DPS. In this example TRS1, TRS2, and TRS3 are transmitted from TRP0, TRP1, and TRP2, respectively.

Figure 1	Example of TCI configuration supporting JT and DPS under the HST-SFN deployment.

If the network activates all the TCIs and UE is capable of monitoring 4 TCIs simultaneously, UE needs to monitor 4 TRS resources for time/frequency tracking. Since the network signaling indicates the network uses HST-SFN, UE may need to analyze TRS is transmitted from single TRP (single path) or multiple TRPs (multi path). If UE judges TRS is transmitted from one TRP, then UE can assume the corresponding TCI is used for DPS and applies the normal frequency tracking algorithm. On the other hand, if UE judges TRS is transmitted from multiple TRPs, then UE can assume the corresponding TCI is used for JT and may apply the advanced frequency tracking depending on the UE capability.

It would be beneficial for UE to get the additional information where TRS is transmitted from one TRP or multiple TRPs. However we are not sure RAN1 supports additional transmission scheme in the future release, and therefore we are reluctant to introduce a network assisted signaling. Based on our analysis, we don’t think it is necessary to introduce a new network assigned signaling indicating TRS is transmitted from a single TRP or multiples TRPs.

Proposal 1: Do not introduce any additional network assisted signaling for HST single tap scenario.

Another issue is the frequency offset estimation based on single TRS transmission or multiple TRS transmission. We think it is up to UE implementation and we should not mandate any specific receiver algorithm. Our proposal is to review the performance difference among companies. If it is observed the large span among companies, then we may come back to the receiver assumption.

Considering the discussion above, our preference is

- Option B: Define requirements under assumption UE is aware on HST single tap conditions and can adjust time/freq tracking algorithms ​

  - Option 1: UE detects the conditions​

Proposal 2: For PDSCH demodulation requirements with the HST single tap, define requirements under the assumption UE is aware on HST single tap conditions and can adjust time/frequency tracking algorithms. It is up to UE implementation how to detect the condition.

# 3	Summary

Proposal 1: Do not introduce any additional network assisted signaling for HST single tap scenario.

Proposal 2: For PDSCH demodulation requirements with the HST single tap, define requirements under the assumption UE is aware on HST single tap conditions and can adjust time/frequency tracking algorithms. It is up to UE implementation how to detect the condition.

# References

- R4-1915926, “WF on UE demodulation for NR HST”, CMCC.