3GPP TSG-RAN WG4 Meeting #94	R4-2001385

online, 24th Feb – 6th Mar 2020

Source: 	Nokia, Nokia Shanghai Bell

Title: 					Problems with CA channel arrangement

Agenda Item: 			8.13.1.2 Intra-band contiguous DL CA for FR1

Document for:	Discussion

# 1	Introduction

In the channel arrangement formulas for NR contiguous carrier aggregation, there are ambiguities and errors that should be corrected. According to the formulas currently found in TS 38.101-1, TS 38.101-2, and TS 38.104,

- There are ambiguities and errors in formulas related to CA channel arrangement, resulting in risk of misinterpretation.

- According to TS 38.101-1 and TS 38.101-2, the UL aggregated channel bandwidth exceeds the sum of CC bandwidths with several combinations of CC bandwidths and SCSs. This should not be allowed.

- The aggregated channel bandwidth depends on the SCSs used in the CCs, which is undesirable.

Note: What is said in this contribution about parameters Foffset,low and Foffset,high (in formulas for single subblock) applies also to Foffset,block,low and Foffset,block,high (in formulas for multiple subblocks).

# 2	Discussion

Problem 1: In TS 38.101-1 clause 5.4A.1, TS 38.101-2 clause 5.4A.1, and TS 38.104 clause 5.4.1.2, the wording “for the said 𝜇 value” has caused confusion.

Proposed solution 1: Replace “for the said 𝜇 value” with “for 𝜇=𝜇0”.

Problem 2: In 38.101-1 and 38.101-2, statement BWGB = max(BWGB,Channel(k)) does not specify whether the maximum should be taken across all values of SCS or across all values of k or across all values of both SCS and k.

Proposed solution 2: For the UE Define BWGB=maxk(BWGB,Channelk(0)), indicating that the maximum is taken across the CCs, and the included minimum guard band widths correspond to 0, the largest μ value among the subcarrier spacing configurations supported in the operating band for both of the channel bandwidths (see the definition of CA channel spacing).

Problem 3: The definitions of channel spacing, Foffset,low, and Foffset,high result in subblock bandwidth that exceeds the sum of CC bandwidth in the UL. This occurs with numerous combinations of channel bandwidths and SCSs in both FR1 and FR2. This should not be allowed.

See the table provided in Appendix for the problematic aggregated channel bandwidths.

The excessive subblock bandwidth results from

- using NRB and minimum guard band widths based on different SCS values in the formulas of Foffset,low and Foffset,high.

- using minimum guard band widths based on different SCSs in the channel spacing formula and the Foffset,low and Foffset,high formulas.

Fig. 1 illustrates the guard exchange mechanism used in 38.101-1 and 38.101-2 (implemented jointly by the formulas of channel spacing and Foffset,low/high) and rounding of the channel spacing.

Proposed solution 3: To be solved.

This problem does not exist in the base station because 38.104 has a different definition of Foffset,low and Foffset,high. [1][2]

Fig. 1: Guard exchange and rounding of channel spacing

# 4	Conclusions

The CA channel arrangement formulas contain typos, ambiguities, and errors that need to be fixed.

# 5	References

[1] R4-1812252 Draft CR to 38.104: Corrections on the descriptions of BS channel bandwidth for CA, ZTE Corporation, RAN4#88bis

[2] R4-1815562 CR to TS 38.104 on Combined updates from RAN4 #88bis and #89, Ericsson, RAN4#89

# 6	Appendix

The tables below present FR1 and FR2 UL subblock bandwidths according to the CA channel arrangement formulas of TS 38.101-1 V16.2.0 and TS 38.101-2 V16.2.0 for all possible combinations of CC bandwidths and SCSs. The minimum SCS-based channel raster is assumed because it maximizes the channel spacing. In the tables, red color indicates that the subblock bandwidth exceeds the sum of CC bandwidths.