3GPP TSG-RAN4 Meeting #94-e	R4-2001405

# Online, , 24th Feb 2020 - 6th Mar 2020

| CR-Form-v12.0 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.133 | CR | 0503 | rev | - | Current version: | 16.2.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | X | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | NR CGI measurements with autonomous gaps for 38.133 |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Ericsson |  |  |  |  |  |  |
| Source to TSG: | RAN WG4 |  |  |  |  |  |  |
| Work item code: | NR_RRM_enh-Core |  |  |  | Date: |  | 2020-02-14 |
| Category: | B |  |  |  | Release: |  | Rel-16 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) Rel-12 (Release 12) Rel-13 (Release 13) Rel-14 (Release 14) Rel-15 (Release 15) Rel-16 (Release 16) |  |

| Reason for change: |  | Requirements are added for the interruption caused to NR when a UE reads the CGI of an E-UTRA target cell with autonomous gaps |  |  |  |
| --- | --- | --- | --- | --- | --- |
| Summary of change: |  | Requirements are added with the same numerical ACK/NACK count as for MIB+CGI reading in the RSTD procedure with autononmous gaps |  |  |  |
| Consequences if not approved: |  | LTE CGI reading is not fully specified for EN-DC and NE-DC operations. |  |  |  |
| Clauses affected: |  | 8.2.5 (new) |  |  |  |
|  |  | Y | N |  |  |
| Other specs |  |  | x | Other core specifications | TS/TR ... CR ... |
| affected: |  |  | x | Test specifications | TS/TR ... CR ... |
| (show related CRs) |  |  | x | O&M Specifications | TS/TR ... CR ... |
| Other comments: |  |  |  |  |  |

Change 1

##### 8.2.5	Interruptions to NR serving cells due to E-UTRAN CGI reading with autonomous gaps

The requirements in this section apply when the UE is configured with standalone NR, EN-DC or NE-DC. The UE shall identify and report the CGI when requested by an NR or E-UTRA PCell or PSCell for the purpose ‘reportCGI’. The UE may make autonomous gaps in downlink reception and uplink transmission for receiving MIB and SIB1 message according to clause 5.5.3.1 of TS 36.331 [16]. Note that a UE is not required to use autonomous gap if the request is configured by an LTE PCell or PSCell and si-RequestForHO is set to false. If autonomous gaps are used for measurement with the purpose of ‘reportCGI’, regardless of whether SCell(s) are configured or not, the UE shall be able to identify a new CGI of E-UTRA cell within the time specified and in the conditions specified in [15]:

Within the time Tidentify,cgi, over which the UE identifies the new CGI of E-UTRA cell, the UE shall transmit on any NR PCell, NR PSCell, and each of activated NR SCell(s) in the frequency range where autonomous gaps are created at least NACK/NACK, ECGI,, FDD or NACK/NACK, ECGI, TDD ACK/NACKs specified in Table 8.2.5-1 or 8.2.5-2. The requirements apply, provided that:

-	there is continuous DL data allocation,

-	no DRX cycle is used,

-	no measurement gaps are configured,

-	only one code word is transmitted in each slot,

-	2 slot ACK/NACK feedback is configured,

-	20 ms SMTC period is configured,

Table 8.2.5-1: Number of ACK/NACKs transmitted by the UE during Tidentify,CGI for EUTRA-FDD CGI reading

| NACK/NACK, ECGI, FDD | Configuration of the serving cell in which the transmitted ACK/NACKs are counted |  |
| --- | --- | --- |
|  | Duplex mode configuration | SCS |
| 84 | FDD | 15 kHz |
| 193 | FDD | 30 kHz |
| 402 | FDD | 60 kHz |
| 28 | TDD Note 1 | 15 kHz |
| 81 | TDD Note 1 | 30 kHz |
| 159 | TDD Note 1 | 60 kHz |
| 233 | TDD Note 2 | 60 kHz |
| 491 | TDD Note 2 | 120 kHz |
| NOTE 1: TDD UL-DL configuration is as specified in Table A.3.3.1-1 of TS 38.101-1 [18]. NOTE 2: TDD UL-DL configuration is as specified in Table A.3.3.1-1 of TS 38.101-2 [19]. |  |  |

Table 8.2.5-2: Number of ACK/NACKs transmitted by the UE during Tidentify,CGI for EUTRA-TDD CGI reading

| NACK/NACK, ECGI, TDD | Configuration of the serving cell in which the transmitted ACK/NACKs are counted |  |
| --- | --- | --- |
|  | Duplex mode configuration | SCS |
| 84 | FDD | 15 kHz |
| 193 | FDD | 30 kHz |
| 402 | FDD | 60 kHz |
| 28 | TDD Note 1 | 15 kHz |
| 81 | TDD Note 1 | 30 kHz |
| 159 | TDD Note 1 | 60 kHz |
| 233 | TDD Note 2 | 60 kHz |
| 491 | TDD Note 2 | 120 kHz |
| NOTE 1: TDD UL-DL configuration is as specified in Table A.3.3.1-1 of TS 38.101-1 [18]. NOTE 2: TDD UL-DL configuration is as specified in Table A.3.3.1-1 of TS 38.101-2 [19]. |  |  |