3GPP TSG-RAN WG4 #94-e	R4-2001413

Online, 24th February – 6th March 2020

Title: 					Remaining open issues on DAPS handover for NR

Source: 				Ericsson

Agenda item:	8.3.2.1

Document for:	Discussion

# Introduction

In RAN4#93, progress was made on NR DAPS handover, involving simultaneous reception and transmission and much smaller interruption than legacy handover. CRs were technically endorsed [2]. The CRs contain a small number of TBDs and editor’s notes which need to be addressed, and most of the remaining open issues were covered by a way forward [1]. In this paper, we present our analysis of the remaining issues.

# Discussion

Most of the outstanding issues relate to intrafrequency DAPS handover, and the interruption requirement. We begin with discussion on the relationship between CBW and BWP from UE perspective:

FFS on the interruption requirement when the relationship between CBW of target and source cell is different the relationship between BWP of target and source cell.

The minimum requirement for a UE is that it receives the active BWP, and it is decided for DAPs handover that BWsource ≥ BWtarget. However, BWP was introduced for UE power saving and it is not mandatory that the UE saves power by implementing BWP switching. For example, a first release of UE may still receive the full channel BW. It can happen that either CBWsource<CWBtarget or CBWsource>CWBtarget. This does not seem a very common deployment scenario (the most common intrafrequency deployment is going to be CBWsource=CWBtarget. Still it may happen that the scenario occurs, for instance when moving from urban to rural areas or similar.

If we consider a hypothetical UE which does not implement BWP based switching of the RF bandwidth, it can be seen from this description that it may either need to increase RF bandwidth or reduce RF bandwidth during the DAPS handover. This results in either an interruption when the target cell is added (BW increasing case) or when the source cell is released (BW reducing case). Hence, to cover such UE it would be necessary to reintroduce more generic interruption requirements covering both cases.

Proposal 1 : To support cases where the relationship of CBW of target and source cell is different to different the relationship between BWP of target and source cell, generic interruption requirements are necessary. An interruption is allowed both when the target cell is added, and the source cell is released. This allows the UE always to have its RF configured to receive CBW.

RAN4 could discuss whether P1 is strictly necessary, since even if the implementation does not normally follow BWP, the following implementation would be possible

- UE configured to receive CBW of source cell

- UE performs DAPS handover with CWBtarget>CBWsource and BWPtarget≤BWPsource

- Despite 2, UE does not increase RF BW at target addition, since it is already sufficient to receive the active BWP

- If any BWP switch (DCI/timer/RRC based) is triggered after the DAPS handover, the UE may then reconfigure to receive the full CWBtarget

These steps represent a relatively straightforward implementation, so we think RAN4 should discuss if P1 is strictly necessary or whether this UE behaviour can be assumed.

Proposal 2 : RAN4 should discuss if P1 is strictly necessary

Interruption for intra-frequency HO with BWsource ≥ BWtarget:

Next we consider the duration of the interruption. In the way forward there are two options for intra-frequency HO with BWsource ≥ BWtarget:

Option 1: corresponds to 1ms interruption

Option 2: Corresponds to 0.5ms interruption, except for 15kHz SCS, where this is rounded up to a slot (1ms).

Since 500ms is sufficient for gap based measurement retuning, which potentially involves a change to centre frequency and received BW our view is that it is sufficient to allow the UE to reconfigure its receiver.

Proposal 3 : The interruption for intra-frequency DAPS handover is specified as:

|  | NR Slot length (ms) | Interruption length X (slotsnote 1) |
| --- | --- | --- |
| 0 | 1 | [1] |
| 1 | 0.5 | [1] |
| 2 | 0.25 Note 2 | [2] |
| Note 1: The same SCS of source cell and target cell is assumed. Note 2: Both source cell and target cell are on FR1. |  |  |

Note that according to P1/P2 there still needs to be discussion on whether there is a single interruption, or an interruption at target addition and  source release in DAPS handover.

Side condition: power imbalance between the two cells should be within X dB. X is FFS

We propose that X (long term average power imbalance between the two cells) should be within [6]dB. Firstly, since the side condition for cell detection and measurement is Es/Iot=-6dB, it would  be difficult for the UE to detect or report weaker cells. Secondly, power imbalance for LTE CA is 6dB so the value seems reasonable from a receiver dynamic range perspective. Larger imbalance would require more dynamic range from the UE receiver

Proposal 4: power imbalance between the two cells should be within [6] dB.

Source cell release command

This is currently TBD in CR, also when checking the RAN2 running CR (R2-1914833), the command does not seem to be decided yet.

# Conclusions

In this paper, we discuss remaining open issues for DAPS handover. We propose

Proposal 1 : To support cases where the relationship of CBW of target and source cell is different to different the relationship between BWP of target and source cell, generic interruption requirements are necessary. An interruption is allowed both when the target cell is added, and the source cell is released. This allows the UE always to have its RF configured to receive CBW.

Proposal 2 : RAN4 should discuss if P1 is strictly necessary

Proposal 3 : The interruption for intra-frequency DAPS handover is specified as:

|  | NR Slot length (ms) | Interruption length X (slotsnote 1) |
| --- | --- | --- |
| 0 | 1 | [1] |
| 1 | 0.5 | [1] |
| 2 | 0.25 Note 2 | [2] |
| Note 1: The same SCS of source cell and target cell is assumed. Note 2: Both source cell and target cell are on FR1. |  |  |

Proposal 4: power imbalance between the two cells should be within [6] dB.

# References

- R4-1913436, “Way forward on NR mobility enhancement”, Intel

- R4-1915604	, “CR on introducing DAPS handover requirements in TS38.133”	Huawei, HiSilicon