3GPP TSG-RAN WG4 #94-e	R4-2001418

Online, 24th February – 6th March 2020

Title: 					Test aspects of sRVCC for NR

Source: 				Ericsson

Agenda item:	8.10.2

Document for:	Discussion

# Introduction

In RAN4#93bis and RAN4#94, CRs were agreed to complete the core work on sRVCC handover to 3G from NR. In this paper, we discuss some testing aspects of sRVCC.

# Discussion

## Overview of mobility operation of SRVCC

The fundamental purpose of SRVCC is to hand off an ongoing packet switched voice call (VoNR or VoLTE) to the CS capable network if a UE  reaches the edge of NR coverage (and cannot, or is not desired to switch to a VoLTE call).From a radio perspective, the UE reports 3G measurements to the gNB, and the gNB initiates handover preparation on the reported 3G cell and is provided with a 3G handover command, which it forwards to the UE.  There are also a large number of additional actions to reconfigure higher layers and audio codecs to ensure voice call continuity, however these do not have impact to RAN4. Should the network and UE support rSRVCC (reverse SRVCC), the call can subsequently be handed back from 3G CS to LTE PS and from LTE to NR, however it is important to recognize that direct 3G to NR mobility is not supported (3G RRC does not support NR measurement configuration and reports, for example).

Observation 1: Although SRVCC is supported in release 16, there is no corresponding rSRVCC procedure or other mobility directly from 3G to NR specified

## Testing aspects

Similarly to other interRAT RRM procedures, it is beneficial to define RRM testcases to ensure that RAN4 requirements are met, and mobility between NR and 3G is operating correctly. For mobility between LTE and NR, testcases are specified in 38.133 section A.8 which verify mobility from LTE standalone to NR. Based on observation 1, it would not be possible to define the analogous tests in the direction of 3G to NR mobility, so we propose

Proposal 1:RRM tests are introduced to verify NR to 3G mobility

Since it would not be possible to provide a calibrated 3G signal in an OTA environment, such tests should be conducted and use FR1 NR.

## Test list

| Test number | Title | Test Purpose | Description | Responsible company |
| --- | --- | --- | --- | --- |
| 1 | NR to UTRA FDD Inter-RAT measurement reporting | Verify UTRA cell detection requirements in section 9.4.6.2.1 | The test consists of two time phases, T1 and T2. During T1 the NR cell (cell 1) is powered on and a connection to the UE is established with a neighbour list including the UTRA cell (cell2). Cell 2 is powered off during T1. DRX is not used. During T2 cell 2 is powered on, such that it meets the side conditions in section 9.4.6.2.1 for cell detection. The time from the start of T2 until the UE transmits an event triggered measurement report containing cell 2 is verified. At the end of T2, cell 2 is powered off and the UE is restarted such that it will select cell 1 ready for the next iteration of the test. AWGN propagation conditions are used for cell 1 and cell2 |  |
| 2 | NR to UTRA FDD InterRAT handover | Verify UTRA handover requirements in section 6.1.2.2 | Cell 1 is the NR and Cell 2 is an inter-RAT UTRA FDD neighbour cell. The test consists of three successive time periods, with time durations of T1, T2 and T3 respectively. At the start of time duration T1, the UE does not have any timing information of Cell 2. Starting T2, Cell 2 becomes detectable and the UE is expected to detect and send a measurement report. Gap pattern configuration GP0 is configured before T2 begins to enable inter-RAT frequency monitoring. A RRC message implying handover shall be sent to the UE during period T2 after the UE has reported the UTRA cell. The start of T3 is the instant when the last TTI containing the RRC message implying handover is sent to the UE. The handover message shall contain Cell 2 as the target cell. The delay from the start of T3 until a PRACH is sent to cell 2 is verified. AWGN propagation conditions are used for cell 1 and cell 2. | Ericsson |

# Conclusions

In this contribution we discuss testcases for SRVCC mobility from NR to 3G. We propose the following proposal and test case list

Proposal 1:RRM tests are introduced to verify NR to 3G mobility

# References

- R4-1912840, “Way forward on RRM enhancement: mandatory of gap patterns”, ZTE