3GPP TSG-RAN WG4 Meeting #94-e											  	     		    R4-2001432

Electronic Meeting, 24 February – 6 March, 2020

Source: 	Nokia, Nokia Shanghai Bell

Title: 		TP to TR 38.xxx IAB-Node blocking, power class and coexistence requirements in FR2

Agenda item:			8.5.2

Document for:			Approval

# 1	Introduction

Part of the Integrated Access and Backhaul work item is defining the RF requirements needed to guarantee coexistence between IAB-Nodes and legacy networks. In this contribution we provide further simulation results as agreed in RAN4#93 and discuss the receiver blocking of both legacy NR network and IAB-Nodes. Also coexistence requirements for IAB-Nodes are proposed.

# 2	Discussion

In RAN4#93 it was discussed that layout 1 as agreed in [1] is problematic from blocking performance perspective, as IAB-MT Tx beam is constantly pointing towards the victim legacy NR base station, due to it being co-located with the IAB-parent. Additionally, an IAB-Node cannot be co-located with another IAB-Node or base station operating in the same band, as co-location blocking will have severe performance impact. To confirm these results blocking powers at rel-15 gNB receiver were simulated. The main used simulation parameters have been summarized in Table 1 and results are shown in Figure 1.

Table 1: Main simulation parameters used in the study.

| Parameters |  |
| --- | --- |
| Duplex mode | TDD (100% UL or 100% DL) |
| Carrier Frequency | 30 GHz |
| Beamforming | Yes |
| Simulation bandwidth | 200 MHz |
| Number of UEs in the network | 5 UEs/sector |
| gNB Tx power | 33 dBm for macro and micro |
| maximum IAB node Tx power | 33 dBm |
| maximum gNB EIRP | 57 dBm |
| maximum IAB node EIRP | 57 dBm |
| gNB antenna height | 25m (macro cells); 10m (micro cells) |
| IAB node antenna height | 25m (macro cells); 10m (micro cells) |
| gNB receiver noise figure | 10 dB |
| IAB node receiver noise figure | 10 dB |
| UE Tx power (dBm) | 22.4 dBm EIRP (13.4 dBm conducted) |
| UE noise figure (dB) | 10 |
| Subcarrier spacing | 120 kHz |
| Maximum number of IAB hops to egress node | 3 |
| Traffic Model | Full buffer |
| Transmit Method | Rank adaptive SU-MIMO with maximum rank of 2 |
| CSI Feedback | CRI/PMI/RI/CQI, 10ms period, 5ms delay |
| Maximum modulation order | 256QAM |

Figure 1: Blocking power level at fully connected network rel-15 gNB receivers in layout 1

As seen in Figure 1, the blocking power levels observed at the fully connected network receivers go beyond specified rel-15 BS in-blocking levels. As no impact in legacy network is expected, the scenario is problematic. Therefore, simulation efforts were concentrated to layout 2, also agreed in [1].

Firstly, it was confirmed that there are no coexistence issues in layout 2. For this purpose simulations were done with power control range of 13…16 dB and limiting IAB-MT Tx ACLR to 16 dBc using 20m grid shift. The results are summarized in table 2.

Table 2: Coexistence simulation results for layout 2.

|  | Baseline, 2 systems, no IAB-aggressor | 2 systems with IAB-aggressor, 13 dB power control range | 2 systems with IAB-aggressor, 16 dB power control range |
| --- | --- | --- | --- |
| Degradation, mean | 0% | 0.9% | 0.6% |
| Degradation, 5%-tile | 0% | 5.7% | 3.4% |

As seen from the results, co-existence will not be an issue in layout 2.

Observation 1: Co-existence performance can be guaranteed in layout 2 with 16 dB power control range and 16 dBc ACLR.

In addition to co-existence, also blocking power levels were simulated. The CDF of observed blocking power was measured at both IAB-MT receiver and legacy NR BS receiver. The simulations were performed with 40m and maximum grid shift. The resulting CDFs are shown in Figures 2 and 3 and the 99% and 99.9% points have been captured in table 3.

Figure 2: Blocking power at rel-15 gNB receiver

Figure 3: Blocking power at IAB-MT receiver

Table 3: 99% and 99.9% blocking power levels in layout 2

|  | gNB receiver, 40 m grid shift | gNB receiver, max grid shift | IAB-MT receiver, 40 m grid shift | IAB-MT receiver, max grid shift |
| --- | --- | --- | --- | --- |
| 99% power level | -70.2 dBm | -84.0 dBm | -52 dBm | -55 dBm |
| 99.9% power level | -57.4 dBm | -67.0 dBm | -45 dBm | -54 dBm |

High power levels at rel-15 gNB receiver are rare. However, it can be seen that irrespective of grid shift at IAB-MT receiver over 3% of the observed powers are higher than -60 dBm, and about 10% of observed power levels are higher than -70 dBm. Antenna array gain is not included in the power levels.

Due to beamforming the very highest input levels at rel-15 gNB receiver are extremely rare and there is a major difference in power level between the 99% and 99.9% points. Similarly for IAB-MT receiver there was only a single simulation case where power level was above -50 dBm. We think that here it is reasonable to base the evaluation of blocking impact on 99% point of the CDF.

Proposal 1: The blocking power evaluation is based on 99% point of the CDF.

Observation 2: IAB-MT transmissions do not cause significant degradation to rel-15 gNB receiver performance with the used assumptions.

According to TS 38.104 the blocking power levels in FR2 for different base station classes are based on the declared reference sensitivity level and a fixed offset. The resulting blocking power levels are captured in table 4:

Table 4: In-band blocking power levels in FR2

| BS class | Maximum in-band blocker level [dBm] | Minimum in-band blocker level [dBm] |
| --- | --- | --- |
| Wide area | -63 | -89 |
| Medium range | -58 | -84 |
| Local area | -53 | -79 |

Comparing the simulation results and the power range covered by the BS specifications, it can be seen that IAB-MT needs to be able to tolerate higher interferer levels than FR2 BS.

Observation 3: IAB-MT receiver need to be able to tolerate higher interferer levels than rel-15 gNB receiver.

The IAB-MT reference sensitivity requirement has not been agreed yet, and as in-band blocking is likely to be dependent on the sensitivity level, it may be difficult to conclude the complete in-band blocking requirement. In any case, the data suggests that the interferer power level would need to be increased 6 dB. It is illustrated in table 5 how the IAB-MT in-band blocking requirement would look like in case the modification is agreed to be done on top of the current BS requirement.

Table 5: General OTA blocking requirement for IAB-MT

| IAB channel bandwidth of the lowest/highest carrier received (MHz) | OTA wanted signal mean power (dBm) | OTA interfering signal mean power (dBm) | OTA interfering signal centre frequency offset from the lower/upper IAB RF Bandwidth edge or sub-block edge inside a sub-block gap (MHz) | Type of OTA interfering signal |
| --- | --- | --- | --- | --- |
| 50, 100, 200, 400 | EISREFSENS + 6 dB | EISREFSENS_50M + 39 + ΔFR2_REFSENS | ±75 | 50 MHz DFT-s-OFDM NR signal, 60 kHz SCS, 64 RBs |
| NOTE: EISREFSENS and EISREFSENS_50M are given in clause 10.3.3. |  |  |  |  |

Based on the above, the following additional proposal are made.

Proposal 2: Compared to FR2 base station, IAB-MT in-band interferer power level is increased by 6 dB.

Proposal 3: The details of in-band blocking requirement are confirmed only after the requirement for reference sensitivity is agreed.

Earlier it has been also considered that multiple power classes could be specified for IAB-MT, or some other division of different MT categories could be made given that simulation results e.g. [2, 3] indicated that coexistence can be solved by either improving ACLR or power control. However, these discussions and analyses were based on coexistence evaluations of layout 1. As shown, layout 1 does not seem to be feasible for IAB-Node deployment as the interference towards legacy NR network becomes excessive in form of excessive in-band blocking power.

It was also shown that similar solution is not required for layout 2, as coexistence issues are not nearly as prevalent as in layout 2. Therefore, it seems no more justified to specify multiple IAB-MT categories as due to the different scenario similar need to trade-off requirements cannot be found.  Based on the simulation results UE ACLR i.e. 17 dBc can be re-used for IAB-MT in FR2 as long as 13 dB power control range is used, equalling minimum power of 20 dBm TRP, when the channel is fully allocated.

Proposal 4: Only single IAB-MT power class or category is specified

Proposal 5: Adopt 17 dBc as IAB-MT ACLR minimum requirement.

Proposal 6: Adopt 20 dBm as minimum output power for IAB-MT with the side condition of channel being fully allocated.

# 3	Conclusion

In this contribution coexistence and blocking simulation results were provided. Based on the results and related discussion the following observation and proposals were made.

Observation 1: Co-existence performance can be guaranteed in layout 2 with 16 dB power control range and 16 dBc ACLR.

Observation 2: IAB-MT transmissions do not cause significant degradation to rel-15 gNB receiver performance with the used assumptions.

Observation 3: IAB-MT receiver need to be able to tolerate higher interferer levels than rel-15 gNB receiver.

Proposal 1: The blocking power evaluation is based on 99% point of the CDF.

Proposal 2: Compared to FR2 base station, IAB-MT in-band interferer power level is increased by 6 dB.

Proposal 3: The details of in-band blocking requirement are confirmed only after the requirement for reference sensitivity is agreed.

Proposal 4: Only single IAB-MT power class or category is specified

Proposal 5: Adopt 17 dBc as IAB-MT ACLR minimum requirement.

Proposal 6: Adopt 20 dBm as minimum output power for IAB-MT.

In addition, the text proposal at the end of the document is proposed to be introduced into the IAB TR.

# 4	References

[1]	R4-1907825 WF on simulation assumptions for IAB co-existence study

[2]	R4-1914260 IAB-Node coexistence simulation results, Nokia, Nokia Shanghai Bell

[3]	R4-1913408 IAB MT RF co-existence requirements in FR2, Qualcomm Incorporated

TP to TR 38.8xx NR; Background for integrated access and backhaul radio transmission and reception

< Start of text proposal >

# 2	References

The following documents contain provisions which, through reference in this text, constitute provisions of the present document.

-	References are either specific (identified by date of publication, edition number, version number, etc.) or non-specific.

-	For a specific reference, subsequent revisions do not apply.

-	For a non-specific reference, the latest version applies. In the case of a reference to a 3GPP document (including a GSM document), a non-specific reference implicitly refers to the latest version of that document in the same Release as the present document.

[1]	3GPP TR 21.905: "Vocabulary for 3GPP Specifications".

[2]	3GPP TR 38.874: “Study on Integrated Access and Backhaul”

# 3	Definitions of terms, symbols and abbreviations

< Unchanged sections omitted >

## 6.2	Simulation assumption

The simulations were performed for two network layouts. Layout 1 was a coordinated layout with co-located base stations as macro layer and IAB-Node micro-nodes at 40 to 60m distance from the macros. Layout 2 was a homogeneous hexagonal micro-layer with grid shifts varying from 20 meters to 40 meters. Layout 1 and Layout 2 are shown in Figures 6,2-1 and 6.2-2.

Using these layouts and parameters it was studied what ACLR and ACS are required for IAB-Nodes so that performance is degraded maximum 5% compared to a scenario where two operators coexist while no IAB-Nodes deployed. Therefore, it was also studied that IAB-Nodes do not cause interference towards legacy networks.

Figure 6.2-1: Layout 1 with macro and micro layers.

Figure 6.2-2: Layout 2 with homogeneous hexagonal micro-layer

The parameters agreed for layout 1 are captured in Table 6.2-1 and parameters for layout 2 in Table 6.2-2. Additional common parameters are in Table 6.2-3.

Table 6.2-1: Simulation parameters for layout 1

| Layout | Both operators with 19 hexagonal macro cell with wrap around and additionally for one of the operators 1 IAB-Node per cell |
| --- | --- |
| Inter-BS distance | 200m for FR2, 500 m for FR1 |
| Minimum distance between IAB-Nodes | 40m |
| Minimum distance between Macro BS and UE | 10m |
| Minimum distance between IAB-Node and UE | 10m |
| BS antenna configurations | FR1: (Mg,Ng,M,N,P) = (1,1,8,8,1), (dH,dV)=(0.5,0.8)λ FR2: (Mg,Ng,M,N,P) = (1,1,8,16,1), (dH,dV)=(0.5,0.5)λ Note 1,2 |
| BS antenna height | 25 m |
| BS antenna element gain + connector loss | FR1: 5 dBi (assuming antenna 1.8dB loss) FR2: 3 dBi (assuming antenna 1.8dB loss) |
| BS receiver noise figure | FR1: 5 dB FR2: 10 dB |
| IAB-Node antenna height | 10 m |
| IAB-Node antenna element gain + connector loss | 3 dBi (assuming antenna 1.8dB loss) |
| IAB-Node receiver noise figure | FR1: 5 dB FR2: 10 dB |
| Note 1: Mg = number of antenna panels in elevation, Ng – number of antenna panels in azimuth, M = number of antenna elements/subarrays in elevation, N= number of antenna elements/subarrays in azimuth, P = number of polarizations. Note 2: TX power is specified per polarization, a single polarization may be simulated under the assumption of polarization match. |  |

Table 6.2-2: Simulation parameters for layout 2

| Layout | Homogeneous micro layer for each operator |
| --- | --- |
| Inter-BS distance | 200m |
| Grid shift | 20m, 40m |
| Minimum distance between serving Node and UE | 10m |
| BS antenna configurations | FR1: (Mg,Ng,M,N,P) = (1,1,8,8,1), (dH,dV)=(0.5,0.8)λ FR2: (Mg,Ng,M,N,P) = (1,1,8,16,1), (dH,dV)=(0.5,0.5)λ Note 1,2 |
| BS antenna height | 10 m |
| BS antenna element gain + connector loss | FR1: 3 dBi (assuming antenna 1.8dB loss) |
| BS receiver noise figure | FR1: 5 dB FR2: 10 dB |
| IAB-Node antenna height | 10 m |
| IAB-Node antenna element gain + connector loss | 3 dBi (assuming antenna 1.8dB loss) |
| IAB-Node receiver noise figure | FR1: 5 dB FR2: 10 dB |
| Note 1: Mg = number of antenna panels in elevation, Ng – number of antenna panels in azimuth, M = number of antenna elements/subarrays in elevation, N= number of antenna elements/subarrays in azimuth, P = number of polarizations. Note 2: TX power is specified per polarization, a single polarization may be simulated under the assumption of polarization match. |  |

Table 6.2-3: Common parameters

| Duplex mode | TDD, half-duplex operation in IAB-Nodes |
| --- | --- |
| Carrier frequency | 4 GHz, 30 GHz |
| Path-loss model | See TR 38.874 [2] |
| BS Tx power | 33 dBm for FR2, 46 dBm for FR1 |
| UE Tx power | 22.4 dBm EIRP for FR2, 23 dBm for FR1 |
| UE antenna configuration | FR1: Omni FR2: (Mg,Ng,M,N,P) = (1,1,2,2,1), (dH,dV)=(0.5,0.5)λ |
| UE receiver noise figure | 10 dB |
| Power control for IAB-MT | SNR target 22 dB with γ = 1 |
| Power control for UE | SNR target 15 dB with γ = 1 |
| Power control for DL transmissions | No |

## 6.3	Simulation result

#### 6.3.1	Simulation results from Nokia

Simulations were performed in FR2 for both layout 1 and layout 2. In layout 1 it was observed that IAB-Nodes cause excessively high interference towards legacy networks. The simulation result is captured in Figure 6.3.1-1

Figure 6.3-1: Blocking power level at fully connected network rel-15 gNB receivers in layout 1

Due to the interference level going beyond the BS type 2-O in-band blocking levels it was concluded that IAB-Nodes cannot be deployed using layout 1 at least when the parameters in section 6.2 are applied. Therefore the simulation efforts were concentrated in layout 2.

Firstly, it was confirmed to no co-existence issues exist in layout 2, and the results are captured in Table 6.3-1.

Table 6.3-1: Coexistence simulation results for layout 2, uplink transmissions

|  | Baseline, 2 systems, no IAB-aggressor | 2 systems with IAB-aggressor, 13 dB power control range for IAB-MT | 2 systems with IAB-aggressor, 16 dB power control range for IAB-MT |
| --- | --- | --- | --- |
| Throughput degradation, mean | 0% | 0.9% | 0.6% |
| Throughput degradation, 5%-tile | 0% | 5.7% | 3.4% |

In downlink simulations throughput degradation was negligible. Absolute interference power levels were also studied and results are captured in Table 6.3-2 and Figures 6.3-2 and 6.3-3.

Table 6.3-2: 99% and 99.9% blocking power levels in layout 2

|  | gNB receiver, 40 m grid shift | gNB receiver, max grid shift | IAB-MT receiver, 40 m grid shift | IAB-MT receiver, max grid shift |
| --- | --- | --- | --- | --- |
| 99% power level | -70.2 dBm | -84.0 dBm | -52 dBm | -55 dBm |
| 99.9% power level | -57.4 dBm | -67.0 dBm | -45 dBm | -54 dBm |

Figure 6.3-2: Blocking power at gNB receiver

Figure 6.3-3: Blocking power at IAB-MT receiver

Based on the results it was observed that IAB-MT receiver needs to be capable to tolerate higher interference levels than a BS type 2-O receiver, and the difference is approximately 6 dB. Therefore it is recommended that the interference level in IAB-MT in-band blocking requirement is set 6 dB higher relative to BS-type 2-O requirements, as otherwise there exists a risk for system performance degradation.

Furthermore, based on the study it is recommended to re-use BS type 2-O ACS requirements for IAB-MT and adopt a minimum requirement of 17 dBc for IAB-MT ACLR. In the simulations 33 dBm maximum TRP and 13 dB power control range was used for IAB-MT. Therefore it is also recommended that IAB-MT shall be mandated to be capable to  transmit at minimum power level of 20 dBm when the full channel is allocated.

## 6.4	Summary on co-existence study

< End of text proposal >