3GPP TSG-RAN WG4 Meeting #94-e	R4-2001455

# Electronic Meeting, 24th Feb - 6th Mar, 2020

Title: 	Discussion and simulation results on NR UE HST performance requirements under single-tap

Source: 	Huawei, HiSilicon

Agenda item:	8.17.2.1.3

Document for:	Discussion

Background

During RAN4#93 meeting, way forward [1] for NR Rel-16 UE demodulation was approved. In this contribution, we share our views about the UE demodulation requirements for NR Rel-16 HST under single-tap channel model.

Discussion

Maximum Doppler shift

- Maximum Doppler frequency - For 15KHz SCS, 500km/h - Option 1: 1250Hz - Option 2: 875Hz

As per TS 36.101[2], HST-SFN scenario for 500km/h speed is defined as follows:

Table B.3B-1: HST-SFN scenario for higher speed Parameter; Value. 1000 m. 50 m. 500 km/h. 972 Hz.

As we know, 972Hz is targeting 500km/h for carrier frequency 2.1GHz. However in last RAN4 meeting, as per WF [3], NR BS HST performance requirement is on discussion and the agreement on carrier frequency is shown below:

- Maximum Doppler shift - Single tap HST 500km/h - 15kHz SCS : 1740Hz - 30kHz SCS: 3334Hz

We list the relationship between Doppler and carrier frequency with velocity 500km/h for all candidates.

Table 2.1-1: Doppler, velocity and carrier frequency

| Doppler | velocity | carrier frequency |
| --- | --- | --- |
| 1250Hz | 500km/h | 2.7GHz |
| 875Hz | 500km/h | 1.9GHz |
| 870Hz | 500km/h | 1.88GHz |

To align with BS, we should define maximum Doppler shift half of maximum Doppler shift defined for BS side for 500km/h, i.e. 870Hz for FDD 15 kHz.

Observation 1: To align with BS, it is suitable to set the maximum Doppler shift 870Hz.

Figure 2.1-1: CRS in LTE and TRS in NR

From a practical point of view, if UE is configured with DRX, and UE is not expected to receive TRS within a period of time, it is possible to lead to a Doppler shift jump from the maximum Doppler shift value to the minimum Doppler shift value. For maximum Doppler shift greater than 875Hz which is half of maximum UE frequency tracking capability for 15 kHz SCS, the Doppler shift jump exceeds the ‘2π’ limit. BS can assume UE can always receive TRS without configuring DRX, however, it is not practical since it is not good for saving energy for UE. If UE missed several TRS period at some bad situations, such as a lower SNR, above situation will also occur, i.e. there is no enough margin for UE.

Observation 2：There is no enough margin for UE for maximum Doppler shift greater than 875Hz for 15kHz SCS if UE is configured with DRX or at some bad situations, such as a lower SNR.

In LTE, CRS is used for frequency tracking. The nearest distance is 3 symbols, corresponding to frequency tracking capability 2330Hz. So maximum Doppler shift need to be less than 1165Hz which is half of 2330Hz and 972Hz is an appropriate value.

In NR, TRS is used for frequency tracking. The smallest distance of TRS is 4 symbols, corresponding to frequency tracking capability 1750Hz. It is clearly that frequency tracking capability of TRS is less than that of CRS. So maximum Doppler shift need to be less than 875Hz which is half of 1750Hz.

Observation 3：It is needed to define maximum Doppler shift less than 875Hz.

Based on the preceding analysis, we can get the following proposal:

Proposal 1: For single-tap scenario, it is proposed to adopt maximum Doppler shift 870Hz for FDD 15 kHz.

MCS

- MCS ( for Rank 1) - Option 1: MCS 17 - Option 2: MCS 13 - MCS should be decided based on whether the maximum throughput can be achieved

As per simulation results in Section 3, we can get the following proposal:

Observation 4: For single-tap, both MCS 13 and 17 are feasible. MCS 13 has better performance considering balance between throughput and SNR.

Proposal 2: Adopt MCS 13 for single-tap.

Simulations

- Updated simulation assumption for HST-single tap Parameter; Value. FDD 15KHz SCS; TDD 30KHz SCS. Antenna configuration; 1x2; 1x4 (baseline) 2x2; 2x4. DMRS type; type 1. Number of DMRS symbols; DMRS 1+1+1. MCS; MCS 4; MCS 13; MCS 17 based on 64QAM table. TDD pattern; 7D1S2U, S: 6D 4G 4U. Propagation condition; HST-single tap. TRS periodicity; 10ms, 2slot pattern. PDSCH mapping; Type A, Start symbol 2, Duration 12. Ds and Dmin; Ds= 300m, Dmin=2m. Rank; Rank = 1. BW; 10MHz; 40MHz. Maximum Doppler shift; Option 1: 1250Hz Option 2: 875Hz; 1667Hz. Testing metric; SNR @70% of maximum throughput.

We provide our updated simulations as per the simulation assumptions in [1] for HST single-tap as following:

Table 3-1: Ideal Simulation results for NR PDSCH under HST single-tap

| Case Number | CHBW/ SCS | Max Doppler shift fd (Hz) | MIMO | MCS | SNR@70% Max TP |
| --- | --- | --- | --- | --- | --- |
| 1 | 10MHz/15kHz | 875 | 1Tx 2Rx ULA Low | 13 | 4.40 |
| 2 | 10MHz/15kHz | 875 | 1Tx 4Rx ULA Low | 13 | 2.14 |
| 3 | 10MHz/15kHz | 875 | 2Tx 2Rx ULA Low | 13 | 4.40 |
| 4 | 10MHz/15kHz | 875 | 2Tx 4Rx ULA Low | 13 | 1.68 |
| 5 | 10MHz/15kHz | 875 | 1Tx 2Rx ULA Low | 17 | 8.67 |
| 6 | 10MHz/15kHz | 875 | 1Tx 4Rx ULA Low | 17 | 6.50 |
| 7 | 10MHz/15kHz | 875 | 2Tx 2Rx ULA Low | 17 | 8.67 |
| 8 | 10MHz/15kHz | 875 | 2Tx 4Rx ULA Low | 17 | 6.49 |
| 9 | 10MHz/15kHz | 1250 | 1Tx 2Rx ULA Low | 13 | 4.52 |
| 10 | 10MHz/15kHz | 1250 | 1Tx 4Rx ULA Low | 13 | 2.41 |
| 11 | 10MHz/15kHz | 1250 | 2Tx 2Rx ULA Low | 13 | 4.51 |
| 12 | 10MHz/15kHz | 1250 | 2Tx 4Rx ULA Low | 13 | 2.35 |
| 13 | 10MHz/15kHz | 1250 | 1Tx 2Rx ULA Low | 17 | 8.71 |
| 14 | 10MHz/15kHz | 1250 | 1Tx 4Rx ULA Low | 17 | 6.71 |
| 15 | 10MHz/15kHz | 1250 | 2Tx 2Rx ULA Low | 17 | 8.70 |
| 16 | 10MHz/15kHz | 1250 | 2Tx 4Rx ULA Low | 17 | 6.70 |
| 17 | 40MHz/30kHz | 1667 | 1Tx 2Rx ULA Low | 13 | 4.45 |
| 18 | 40MHz/30kHz | 1667 | 1Tx 4Rx ULA Low | 13 | 2.40 |
| 19 | 40MHz/30kHz | 1667 | 2Tx 2Rx ULA Low | 13 | 4.48 |
| 20 | 40MHz/30kHz | 1667 | 2Tx 4Rx ULA Low | 13 | 2.40 |
| 21 | 40MHz/30kHz | 1667 | 1Tx 2Rx ULA Low | 17 | 9.37 |
| 22 | 40MHz/30kHz | 1667 | 1Tx 4Rx ULA Low | 17 | 8.33 |
| 23 | 40MHz/30kHz | 1667 | 2Tx 2Rx ULA Low | 17 | 9.48 |
| 24 | 40MHz/30kHz | 1667 | 2Tx 4Rx ULA Low | 17 | 8.33 |

Figure 3-1: Ideal Simulation results for NR PDSCH under HST single-tap

Proposals

In this contribution, we discuss and provide our simulation results on NR UE HST performance requirements under single-tap channel model. Our observations and proposals are:

Observation 1: To align with BS, it is suitable to set the maximum Doppler shift 870Hz.

Observation 2：There is no enough margin for UE for maximum Doppler shift greater than 875Hz for 15kHz SCS if UE is configured with DRX or at some bad situations, such as a lower SNR.

Observation 3：It is needed to define maximum Doppler shift less than 875Hz.

Observation 4: For single-tap, both MCS 13 and 17 are feasible. MCS 13 has better performance considering balance between throughput and SNR.

Proposal 1: For single-tap scenario, it is proposed to adopt maximum Doppler shift 870Hz for FDD 15 kHz.

Proposal 2: Adopt MCS 13 for single-tap.

Reference

- R4-1915926, WF on UE demodulation for NR HST, RAN4#93, CMCC

- TS 36.101, User Equipment (UE) radio transmission and reception

- R4-1915886, Way forward on NR HST PUSCH demodulation requirements, RAN4#93, Huawei, HiSilicon

- R4-1915120, Discussion and simulation results on NR UE HST performance requirements under single-tap, RAN4#93, Huawei, HiSilicon