3GPP TSG-RAN WG4 Meeting #94-e	R4-2001490

# E-meeting, 24 Feb. – 6 Mar., 2020

Source:	Sony, Ericsson

Title:	Views on SSB only and CSI-RS only beam correspondence

Agenda item:	8.14.1.2

Document for:	Discussion

- Background

In RAN4 #92bis, the following agreements have been captured in the WF [1] for SSB only beam correspondence test:

- RAN4 could specify BC based on SSB only unless RAN4 find critical technical problems, considering:

  - Side condition feasibility

  - Common understanding of UE behavior

  - Performance difference with SSB + CSI-RS based BC

| Parameter | Value |
| --- | --- |
| SSB periodicity | 20 ms [1] |
| Use P1 CSI-RS configuration? | No |
| Use P3 CSI-RS configuration? | Alt. 1: Yes Alt. 2: No |
| Use tracking CSI-RS? | Yes |
| Tracking CSI-RS QCL info | qcl-TypeD=SSB |
| Tracking CSI-RS min SNR | TBD |
| PDCCH/PDSCH DM-RS QCL info | qcl-TypeD=TRS |
| SSB min SNR level | Alt. 1: 6 dB [1] [10] Alt. 2: 13 dB [14] |
| (note) | RAN4 didn’t assume more than [1] SSB indices should be transmitted SSB use configuration for Rel-15 which is specified in 38.508 per agreed in RAN4 #92bis meeting |

In this contribution, we share our views on the above topics for Rel-16 beam correspondence.

- Discussion

## Beam correspondence with SSB only

The goal of the WID for FR2 UE beam correspondence enhancement is to ensure that the UE can perform BC based on DL reference signals (SSB or CSI-RS) configured by the network. For SSB only test, it is important for UE to perform BC and be able to form a “fine” beam since beam management CSI-RS is not compulsory for gNB to configure. To ensure the UE can perform BC with SSB only, the most straightforward way is to not configure CSI-RS in the P3 process of beam management (UE beam refinement).

Proposal 1: 	Do not configure CSI-RS in P3 for SSB only BC test.

Figure. 1. The standard deviation of estimates of RSRP with different numbers of RBs and SNR levels.

The ability of a UE to sustain BC depends mainly on L1-RSRP accuracy [2]. Furthermore, the L1-RSRP accuracy of a UE depends only on the received signal SNR and the number of resource elements (REs) that the UE can use (see Fig. 1). Different types of DL reference signals will show no impact on the RSRP accuracy if the RE number and SNR are the same.

In a real network, SSBs are cell-specific and configured at fixed frequency locations, while CSI-RSs are configured for a specific UE and flexible in the time and frequency domains. Nonetheless, as the AI discussion is limited to test conditions, a clear line of sight (LOS) exists between the test probe and the device under test, and consequently the channel is time-invariant, and the frequency response is flat. Therefore, we think there will be no difference between SSB and CSI-RS in terms of L1-RSRP measurement accuracy, as long as the number of REs and the SNR level are the same in both cases.

Observation 1: 	There is no inherent difference in terms of beam correspondence performance between SSB and CSI-RS under OTA test environment.

The different number of resource blocks (RBs) or resource elements (REs) in the Rel-15 BC test (SSB + CSI-RS) and Rel-16 BC test (SSB only and CSI-only) can, however, potentially affect the performance. The L1-RSRP can be seen as an averaged received signal strength over multiple REs. In a noisy environment, the estimates of the RSRP will be distorted and a certain number of REs will be needed to get an accurate estimation on the L1-RSRP. Simulations of the standard deviation  of estimates of RSRP as a function of SNR and the number of REs are shown in Fig. 1. With Rel-15 side condition, where SNR = 6dB, the standard deviation converges to a very small value (around 0.5 dB) whenever the number of REs is larger than 20. Therefore, we think it is okay to remain at the same SNR level as rel-15 (i.e. 6 dB).

Observation 2: The standard deviation of the RSRP estimates coverges quickly when the number of REs is larger than 20 and the SNR = 6 dB.

It is also worth noting that if the same side condition as rel-15 can be re-used, the Rel-15 BC test could be skipped if the UE passes Rel-16 BC SSB only tests. Increasing the Rel-16 SNR level can hardly be seen as an enhancement of the BC testing capability, and it would certainly make it impossible to translate Rel-16 test results to Rel-15.

Observation 3: Rel-15 BC test is declared automatically passed if a UE passes Rel-16 BC test using the same SSB configuration and SNR as in Rel-15.

Proposal 2: 	SSB min SNR level = 6dB in Rel. 16.

Proposal 3: 	If the Rel-16 SSB BC test is done with the same SSB configuration and side condition as Rel-15, then the UE is allowed to skip the Rel-15 BC test if it passes the Rel-16 SSB BC test.

## Beam correspondence with CSI-RS only

The CSI-RS only BC test is more complicated than the SSB only BC test. The major issue here is how to implement a true CSI-RS only BC test. Two possible methods are proposed from last RAN4 meeting:

  - Method 1: DUT is configured with an active BWP containing no SSB.

  - Method-2: SSB and CSI-RS are present, but SSB’s PSD is back-off by X dB from CSI-RS.

To our understanding, implementations of the CSI-RS only BC test shall reflect real deployment scenarios. In other words, when can the UE, in a real deployment, only use CSI-RS for BC but not SSB?

One possible case is when a UE is configured to a bandwidth part (BWP) which does not contain any SSBs. This, very naturally, leads to method 1 in the WF, where the DUT is configured with an active BWP containing no SSBs. However, one issue here is that such a scenario may not be common in real life, which makes the meaning of such a test questionable. Another problem for this method is that the UE cannot be forced to only receive on the BWP it is configured. The UE may autonomously switch the BWP based the BWP ID that contained in the QCL relation to find a BWP containing SSBs. Only if the above UE behaviors are acceptablecould method 1 be a better choice for the CSI-RS only BS test.

Observation 4: Testing CSI-RS only BC through configuring the UE in a BWP without SSBs may not be representative of common real deployments, and it may fail to implement a true CSI-RS only BC test.

On the other hand, method 2, it is indirectly implies a CSI-RS only BC test by lowering the SNR of SSB signals such that the L1-RSRP accuracy is so low that the UE needs to measure over the CSI-RS for beam selection. Logically, this method can push the UE to use CSI-RS rather than SSB for beam management. It may also reflect some real-life scenarios since the SSB could be transmitted over a wide beam and CSI-RS on a fine beam. From the test point view, to enable the UE to rely on CSI-RS only, the SNR of SSB must be sufficiently low enough such that the UE has difficulties to obtain accurate L1-RSRP estimates, but still sufficiently high to be decoded for the UE to receive information during initial access. One way to set up the test is to re-use the Rel-15 side condition with lower the SNR of SSB.

Observation 5: Lowering the SNR of SSB can encourage the UE to use CSI-RS for beam selection.

However, back to the original question that we had at the beginning of this section: What is the scenario that the CSI-RS only BC test aims to cover? As SSB is an always present DL reference signal in the cell, RAN4 shall identify the scenario where UE can not use SSB for beam selection and decide the test method according to the desired scenario.

Proposal 4: 	RAN4 shall identify the scenario where UE can only use CSI-RS for beam selection and decide the test method according to the desired scenario.

- Conclusion

In this contribution, we shared our views on SSB only and CSI-RS only beam correspondence. The following observations and proposals have been given:

Observation 1: 	There is no inherent difference in terms of beam correspondence performance between SSB and CSI-RS under OTA test environment.

Observation 2: The standard deviation of the RSRP estimates coverges quickly when the number of REs is larger than 20 and the SNR = 6 dB.

Observation 3: Rel-15 BC test is declared automatically passed if a UE passes Rel-16 BC test using the same SSB configuration and SNR as in Rel-15.

Observation 4: Testing CSI-RS only BC through configuring the UE in a BWP without SSBs may not be representative of common real deployments, and it may fail to implement a true CSI-RS only BC test.

Proposal 1: 	Do not configure CSI-RS in P3 for SSB only BC test.

Proposal 2: 	SSB min SNR level = 6dB in Rel. 16.

Proposal 3: 	If the Rel-16 SSB BC test is done with the same SSB configuration and side condition as Rel-15, then the UE is allowed to skip the Rel-15 BC test if it passes the Rel-16 SSB BC test.

Proposal 4: 	RAN4 shall identify the scenario where UE can only use CSI-RS for beam selection and decide the test method according to the desired scenario.

- References

- R4-1916172 “WF on side conditions of SSB based and CSI-RS based beam correspondence tests” Apple Inc.,

- R4-1909219    “Beam Correspondence, SNR versus RSRP”, Sony, Ericsson