3GPP TSG-RAN WG4 Meeting #94-e	R4-2001524

Online, 24th February – 6th March 2020

Source:	Ericsson, Rogers

Title:	TP for TR 37.716-11-11: DC_12_n78

Agenda item:			9.3.2

Document for:	Approval

# 1. Introduction

Rel-16 LTE-NR DC combinations LTE 1 band + NR 1 band combination are defined in the WID [1]. This contribution is a text proposal for TR 37.716-11-11 to include DC_12_n78.

# 2. Text Proposal

# ---Start of changes---

### 6.1.x	DC_12A_n78A

#### 6.1.x.1	Operating bands for DC

Table 6.1.x.1-1: DC band combination of 1 LTE band + 1 NR band

| EN-DC band | E-UTRA Band | NR Band | Single UL allowed |
| --- | --- | --- | --- |
| DC_12_n78 | 12 | n78 | No |

#### 6.1.x.2	Configuration for DC

Table 6.x.2.2-1:  Inter-band EN-DC configurations of 1 LTE band + 1 NR band

| EN-DC configuration | Uplink EN-DC configuration (NOTE 1) | E-UTRA configuration | NR configuration |
| --- | --- | --- | --- |
| DC_12A_n78A | DC_12A_n78A | 12A | n78A |

#### 6.1.x.3	Maximum output power for DC

Table 6.1.x.3-1: Maximum output power for inter-band EN-DC of 1 LTE band + 1 NR band

| DC configuration | Power class 3 (dBm) | Tolerance (dB) |
| --- | --- | --- |
| DC_12A_n78A | 23 | +2/-3 |

#### 6.1.x.4	Spurious emission band UE co-existence for DC

Table 6.1.x.4-1: Spurious emissions for inter-band EN-DC of 1 LTE band + 1 NR band

| E-UTRA and NR DC Configuration | Spurious emission |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  | Protected band | Frequency range (MHz) |  |  | Maximum Level (dBm) | MBW (MHz) | NOTE |
| DC_12A_n78A | E-UTRA Band 5, 26, 41 | FDL_low | - | FDL_high | -50 | 1 |  |

#### 6.1.x.5	MSD analysis for DC

For 2UL/2DL UE coexistence study 2nd, 3rd, 4th and 5th order harmonics and 2nd, 3rd, 4th and 5th order intermodulation products were calculated and presented in Table 6.1.x.5-1

Table 6.1.x.5-1: Harmonic and IMD analysis

| UE UL carriers | fx_low | fx_high | fy_low | fy_high |
| --- | --- | --- | --- | --- |
| UL frequency (MHz) | 699 | 716 | 3300 | 3800 |
| 2nd harmonics frequency limits | 2* fy_low | 2* fy_high | 2*fx_low | 2*fx_high |
| 2nd harmonics frequency limits (MHz) | 1398 | 1432 | 6600 | 7600 |
| 3rd harmonics frequency limits | 3* fy_low | 3* fy_high | 3*fx_low | 3*fx_high |
| 3rd harmonics frequency limits (MHz) | 2097 | 2148 | 9900 | 11400 |
| 4th harmonics frequency limits | 4* fy_low | 4* fy_high | 4*fx_low | 4*fx_high |
| 4th harmonics frequency limits (MHz) | 2796 | 2864 | 13200 | 15200 |
| 5th harmonics frequency limits | 5* fy_low | 5* fy_high | 5*fx_low | 5*fx_high |
| 5th harmonics frequency limits (MHz) | 3495 | 3580 | 16500 | 19000 |
| 6th harmonics frequency limits | 6* fy_low | 6* fy_high | 6*fx_low | 6*fx_high |
| 6th harmonics frequency limits (MHz) | 4194 | 4296 | 19800 | 22800 |
| 7th harmonics frequency limits | 7* fy_low | 7* fy_high | 7*fx_low | 7*fx_high |
| 7th harmonics frequency limits (MHz) | 4893 | 5012 | 23100 | 26600 |
| 2nd order IMD products | ¦fy_high – fx_low¦ | ¦fy_low – fx_high¦ | ¦fy_low + fx_low¦ | ¦fy_high + fx_high¦ |
| IMD frequency limits (MHz) | 3101 | 2584 | 3999 | 4516 |
| 3rd order IMD products | ¦fy_high – 2*fx_low¦ | ¦fy_low – 2*fx_high¦ | ¦2*fy_low – fx_high¦ | ¦2*fy_high – fx_low¦ |
| IMD frequency limits (MHz) | 2402 | 1868 | 5884 | 6901 |
| 3rd order IMD products | ¦2*fx_low + fy_low¦ | ¦2*fx_high + fy_high¦ | ¦2*fy_low + fx_low¦ | ¦2*fy_high + fx_high¦ |
| IMD frequency limits (MHz) | 4698 | 5232 | 7299 | 8316 |
| Two-tone 4th order IMD products | ¦2*fx_low –2* fy_high¦ | ¦2*fx_high – 2*fy_low¦ | ¦2*fx_low +2* fy_low¦ | ¦2*fx_high +2* fy_high¦ |
| IMD frequency limits (MHz) | 6202 | 5168 | 7998 | 9032 |
| Two-tone 4th order IMD products | ¦3*fx_low –1* fy_high¦ | ¦3*fx_high – 1*fy_low¦ | ¦3*fy_low – 1*fx_high¦ | ¦3*fy_high – 1*fx_low¦ |
| IMD frequency limits (MHz) | 1703 | 1152 | 9184 | 10701 |
| Two-tone 4th order IMD products | ¦3*fx_low +1* fy_low¦ | ¦3*fx_high +1* fy_high¦ | ¦3*fy_low + 1*fx_low¦ | ¦3*fy_high + 1*fx_high¦ |
| IMD frequency limits (MHz) | 5397 | 5948 | 10599 | 12116 |
| Two-tone 5th order IMD products | ¦fx_low – 4*fy_high¦ | ¦fx_high – 4*fy_low¦ | ¦fy_low – 4*fx_high¦ | ¦fy_high – 4*fx_low¦ |
| IMD frequency limits (MHz) | 14501 | 12484 | 436 | 1004 |
| Two-tone 5th order IMD products | ¦fx_low + 4*fy_low¦ | ¦fx_high + 4*fy_high¦ | ¦fy_low + 4*fx_low¦ | ¦fy_high + 4*fx_high¦ |
| IMD frequency limits (MHz) | 13899 | 15916 | 6096 | 6664 |
| Two-tone 5th order IMD products | ¦2*fx_low – 3*fy_high¦ | ¦2*fx_high – 3*fy_low¦ | ¦2*fy_low – 3*fx_high¦ | ¦2*fy_high – 3*fx_low¦ |
| IMD frequency limits (MHz) | 10002 | 8468 | 4452 | 5503 |
| Two-tone 5th order IMD products | ¦2*fx_low + 3*fy_low¦ | ¦2*fx_high + 3*fy_high¦ | ¦2*fy_low + 3*fx_low¦ | ¦2*fy_high + 3*fx_high¦ |
| IMD frequency limits (MHz) | 11298 | 12832 | 8697 | 9748 |

Based on Table 6.1.X.5-1, it can be seen that

- 2rd order harmonic may fall into Rx frequencies of bands 50, 51, 75 and 76.

- 3rd order harmonic may fall into Rx frequencies of bands 1, 4, 10, 65 and 66,

- 2nd order IMD products may fall into Rx frequencies of bands 7, 38, 41, 69, n77 and n79.

- 3rd order IMD products may fall into Rx frequencies of bands 1, 2, 3, 4, 9, 10, 23, 25, 30, 33, 34, 35, 36, 37, 39, 40, 46, 47, 65, 66, 70 and n79.

- 4th order IMD products may fall into Rx frequencies of bands 11, 21, 24, 32, 45, 46, 47, 50, 51, 74, 75 and 76

- 5th order IMD products may fall into Rx frequencies of bands 5, 6, 8, 13, 17, 18, 19, 20, 26, 27, 28, 29, 31, 44, 46, 67, 68, 71 and n79, and also own RX frequencies of band 12.

When 2UL inter-band EN-DC UE is operating with other systems such as WiFi, Bluetooth and GNSS system, the harmonics and intermodulation products can have impact on these systems. Table 6.1.X.5-2 lists up to 7th order harmonics and IMD up to 5th order falls into one of these receiving bands.

6.1.x.5-2: Band 12 and Band n78 harmonic and IMD for ISM and GNSS bands

| Victim Systems | Frequency range [MHz] |  |  | Impact | Regions | Comments |
| --- | --- | --- | --- | --- | --- | --- |
| COMPASS (Beidou) | 1559 | - | 1591 | Yes |  | 4th IMD |
| Galileo | 1559 | - | 1591 | Yes |  | 4th IMD |
| GLONASS | 1591 | - | 1610 | Yes |  | 4th IMD |
| GPS | 1563 | - | 1587 | Yes |  | 4th IMD |
| ISM band (2.4GHz) | 2400 | - | 2483.5 | Yes |  | 3rd IMD |
|  | 2400 | - | 2494 | Yes |  | 3rd IMD |
| ISM band (5GHz) | 5150 | - | 5925 | Yes | US | 3rd, 4th, 5th IMD |
|  | 5150 | - | 5350 | Yes | Europe | 3rd, 4th, 5th IMD |
|  | 5470 | - | 5725 | Yes |  | 4th, 5th IMD |
|  | 5150 | - | 5825 | Yes | Asia | 3rd, 4th, 5th IMD |
| 45GHz Unlicensed Bands | 42300 | - | 47000 | No | China |  |
|  | 47200 | - | 48400 | No | China |  |
| 60GHz Unlicensed Bands | 57000 | - | 66000 | No | Europe |  |
|  | 57050 | - | 64000 | No | USA Canada |  |
|  | 57000 | - | 64000 | No | South Korea |  |
|  | 59000 | - | 66000 | No | Japan |  |
|  | 59000 | - | 64000 | No | China |  |
|  | 59400 | - | 62900 | No | Australia |  |

### 6.1.x.6	∆TIB and ∆RIB values

For DC_12_n78, the TIB,c and RIB,c values are reused from DC_28_n78 and are given in the tables below.

# Table 6.1.x.6-1: ΔTIB,c

| E-UTRA and NR DC Configuration | E-UTRA and NR Band | ΔTIB,c [dB] |
| --- | --- | --- |
| DC_12_n78 | 12 | 0.5 |
|  | n78 | 0.8 |

# Table 6.1.x.6-2: ΔRIB,c

| E-UTRA and NR DC Configuration | E-UTRA and NR Band | ΔRIB,c [dB] |
| --- | --- | --- |
| DC_12_n78 | 12 | 0.2 |
|  | n78 | 0.5 |

### 6.1.x.7	MSD

Based on the co-existence studies for DC_12_n78, MSD need to be defined in TS 38.101-3 like in table below. MSD value reused from DC_28A_n78A.

Table 7.3B.2.3.5.1-1: MSD test points for PCell due to dual uplink operation for EN-DC in NR FR1 (two bands)

| NR or E-UTRA Band / Channel bandwidth / NRB / MSD |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| EN-DC Configuration | EUTRA or NR band | UL Fc (MHz) | UL/DL BW (MHz) | UL LCRB | DL Fc (MHz) | MSD (dB) | IMD order |
| DC_12A_n78A | 12 | 705.5 | 5 | 25 | 735.5 | 5.5 | IMD5 |
|  | n78 | 3557.5 | 10 | 50 | 3582.5 | N/A | N/A |

---End of changes---

# Reference

[1]		RP-192742, “Revised WID on EN-DC for 2 bands DL with 2 bands UL (1 LTE band + 1 NR band)”, CHTTL