3GPP TSG-RAN WG4 Meeting #94-e	R4-2001533

Online, 24th February – 6th March 2020

Source:	Ericsson, Rogers

Title:	TP for TR 37.716-21-11: Including band combinations 2-71_n38

Agenda item:			9.4.2

Document for:	Approval

# 1. Introduction

A text proposal for TR 37.716-21-11 to add DC_2A-71A_n38A.

# 2. Text Proposal

---Start of changes---

## 5.1.X	DC_2-71_n38

### 5.1.X.1	Operating bands for EN-DC

Table 5.1.X.1-1: Band combinations EN-DC (three bands)

| EN-DC band | E-UTRA CA band | NR band | Single UL allowed |
| --- | --- | --- | --- |
| 2-71_n38 | CA_2-71 | n38 | No |

### 5.1.X.2	Configuration for DC

Table 5.1.X.2-1: Inter-band EN-DC configurations (three bands)

| EN-DC configuration | Uplink EN-DC configuration (NOTE 1) | E-UTRA CA configuration | NR band |
| --- | --- | --- | --- |
| DC_2A-71A_n38A | DC_71A_n38A DC_2A_n38A | CA_2A-71A | n38 |
| DC_2A-2A-71A_n38A | DC_71A_n38A DC_2A_n38A | CA_2A-2A-71A | n38 |

### 5.1.X.3		∆TIB and ∆RIB values

For DC_2-71_n38, the TIB,c and RIB,c values are reused from LTE combination CA_2-7-12 and CA_2-7-28, and are given in the tables below.

Table 5.1.X.3-1: ΔTIB,c

| Inter-band DC Configuration | E-UTRA and NR Band | ΔTIB,c [dB] |
| --- | --- | --- |
| DC_2-71_n38 DC_2-2-71_n38 | 2 | 0.5 |
|  | 71 | 0.3 |
|  | n38 | 0.5 |

Table 5.1.X.3-2: ΔRIB

| Inter-band DC Configuration | E-UTRA and NR Band | ΔRIB [dB] |
| --- | --- | --- |
| DC_2-71_n38 DC_2-2-71_n38 | 2 | 0 |
|  | 71 | 0 |
|  | n38 | 0 |

### 5.1.x.4	REFSENS requirements

Co-existence analysis from DC_2_n38 shows that there is no impact from DC_2_n38 UL to Band 71 DL.

Co-existence analysis from DC_71_n38 shows that there is IMD2 impact from DC_71_n38 UL to Band 2 DL.

MSD value for Band 2 DL affected by DC_71_n38 UL, are derived from CA_7-28 affecting band 3 DL in CA_3-7-28 and need to be defined in TS 38.101-3 like in table below:

Table 7.3B.2.3.5.2-1: MSD test points for Scell due to dual uplink operation for EN-DC in NR FR1 (three bands)

| NR or E-UTRA Band / Channel bandwidth / NRB / MSD |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| EN-DC Configuration | EUTRA / NR band | UL Fc (MHz) | UL/DL BW (MHz) | UL LCRB | DL Fc (MHz) | MSD (dB) | IMD order |
| DC_2A-71A_n38A DC_2A-2A-71A_n38A | 2 | 1862 | 5 | 25 | 1942 | 26 | IMD2 |
|  | 71 | 668 | 5 | 25 | 622 | N/A | N/A |
|  | n38 | 2610 | 10 | 50 | 2610 | N/A | N/A |

---End of changes---

# Reference

[1]		RP-192815, “Dual Connectivity (EN-DC) of 2 bands LTE inter-band CA (2DL/1UL) and 1 NR band (1DL/1UL)” Huawei, HiSilicon