3GPP RAN WG4 Meeting #94-e                                  R4-2001548

Online, 24th February – 6th March 2020

Agenda item:	8.15.1.4

Source: 	Huawei, HiSilicon

Title: 	Discussion on BWP switching on multiple CCs

Document for:	Discussion

- Introduction

In last meeting RAN4 93, the RRM requirements of BWP switching on multiple CCs are discussed, and the potential agreements and the remaining open issues in the WF [1]. Based on the agreements in the last meeting, we present the analysis of the delay and interruption requirements of BWP switching on multiple CCs under different scenarios.

- Discussion

2.1 Simultaneous triggering

In the last RAN4 93 meeting, the definition and requirements of simultaneous was given for DCI-based, Timer-based and RRC-based BWP switching on multiple CCs as follows:

RAN4#93 Agreement Definition of Simultaneous triggering of BWP switching on multiple CCs - For DCI based switching: - The timing difference among the first symbol of slot carrying DCI for all CCs is received within the MRTD for inter-band CA - For Timer based switching - The timing difference among the beginning of the slot where timer based BWP switching starts for all CCs is within MRTD inter-band CA - For RRC based switching - RRC based BWP switching on multiple CCs for NR-CA is triggered by 1 RRC command - FFS for NR-DC operation - Requirements are defined for BWP switching on all CCs triggered by the same method (DCI, Timer or RRC) - RRC based BWP switching on multiple CCs for NR-CA is triggered by 1 RRC command - FFS for NR-DC operation - For BWP switching delay requirements companies are encouraged to bring analysis on BWP switching delay components that can be done in parallel and sequentially - Option 1: BWP switching on multiple CCs would be N times delay of single CC - Where 1 < N < Number of CCs - FFS if BWP switching delay requirements are scaled for subset of CCs or for all CCs. - Option 2: BWP switching delay 1 CC + D*NK-1; Where D is the incremental processing delay on additional CCs; N is number of CCs; K is number of CCs that can be processed simultaneously - Other options are not precluded - Interruption requirements are FFS

For the simultaneous BWP switching on multiple CCs, it is assumed that UE can coordinate the switching processes on multiple CCs since they are triggered almost simultaneously. A potential options (Option 1 in [1]) to define the requirement under this case is simply extend the delay requirements by N times, N is the total number of CCs. It is an easy way to define the delay requirements. However it may leads to too long delay requirements since some procedures could be done in parallel. The option 2 in the considering the additional process delay and the number of CCs that can be processed in parallel. There existing different process during the BWP switching including decoding, baseband loading, RF retuning, etc. The implementation may vary among different companies and the capability for parallel process may different.

Observation 1: The implementation and the capability for parallel processing may different.

Therefore, it is better to define the BWP switching delay by counting the extra process delay of each additional CC, which is given as: BWP switching delay 1 CC +D*N-1; Where D is the incremental process delay on additional CC; N is the total number of CCs.

Proposal 1: Define the BWP switching delay by counting the extra process delay of each additional CC.

When defining the delay requirement of BWP switching on a single CC, the requirements are presented in terms of number of slots under different SCSs. As for the BWP switching on multiple CCs, it is more feasible to calculate the total absolute time delay and then be transformed to the number of slots according to the SCSs of each CCs.

Proposal 2: It is suggested to calculate the total absolute time delay regardless of SCSs of each CCs and then transforming the delay to the number of slots according to the SCSs of each CCs.

For DCI and timer-based triggering, as mentioned in the Observation 1, we propose to define unified the incremental process delay regardless of the SCSs. The suggested values of D is 100 us for the Type 1 UE and 200 us for Type 2 UE. For the Type 1 UE, the BWP switching delay of a CC where there are simultaneous triggered BWP switching delay on other CCs is: 600 + 100 (N-1) us, where N ≤8  is the total number of CCs where the BWP switching is triggered by the same method (DCI or timer). For the Type 2 UE, the BWP switching delay of a CC where there are simultaneous triggered BWP switching delay on other CCs is: 2000 + 200 (N-1) us, where N ≤8  is the total number of CCs where the BWP switching is triggered by the same method (DCI or timer). For RRC-based switching, the existing delay requirements for single CC can be reused for on each individual CC when the BWP switching is simultaneously triggered on multiple CCs by 1 RRC command in each CG.

Proposal 3:

For DCI and timer triggered BWP switching:

For Type 1 UE, the incremental delay for each additional CC is 100 us. The BWP switching delay of a CC where there are simultaneous triggered BWP switching delay on other CCs is: 600 + 100 (N-1) us, where N ≤8  is the total number of CCs where the BWP switching is triggered by the same method (DCI or timer).

For Type 2 UE, the incremental delay for each additional CC is 200 us. The BWP switching delay of a CC where there are simultaneous triggered BWP switching delay on other CCs is: 2000 + 200 (N-1) us, where N ≤8  is the total number of CCs where the BWP switching is triggered by the same method (DCI or timer).

For RRC-based switching:

The existing delay requirements for single CC can be reused for on each individual CC when the BWP switching is simultaneously triggered on multiple CCs by 1 RRC command in each CG provided that the total number of CCs is less than 8.

For the requirements of interruptions, for BWP switching on multiple CCs, the existing interruption requirements for a single CC shall apply for each BWP switching separately.

Proposal 4:

For the requirements of interruptions, for BWP switching on multiple CCs, the existing interruption requirements for a single CC shall apply for each BWP switching separately.

2.2 Non-simultaneous triggering

In Non-simultaneous triggering scenarios, some cases have been excluded according to RAN1’s description in [2].

For the RRC-based non-simultaneous triggering, it will only be defined for NR-DC where there is only 1 RRC command for each CG. In this way, 2 RRC commands are received for 2 CGs non-simultaneously. It is suggested that the existing delay requirements for single CC can be reused for on each individual CC when the BWP switching is non-simultaneously triggered on multiple CCs by 1 RRC command in each CG for NR-DC provided that the total number of CCs is less than 8.

Proposal 5: It is suggested that the existing delay requirements for single CC can be reused for on each individual CC when the BWP switching is non-simultaneously triggered on multiple CCs by 1 RRC command in each CG for NR-DC provided that the total number of CCs is less than 8.

For DCI-trigged non-simultaneous BWP switching on multiple CCs. It will not be considered for CA case. For the NR-DC case if it is decided to define the requirements for NR-DC, it is more complicated than the simultaneous triggering since the impact of the switching on other CCs varies case by case. It is suggested to follow the requirement of the simultaneous triggering.

TTotal=TSingle+1N-1D

Where TTotal is the total BWP switching delay of a CC; TSingle is the existing delay requirements for BWP switching on a single CC. N is the total number of the non-simultaneous DCI-based BWP switching processes in different CG which overlap with TTotal. D is the incremental delay which is same as the simultaneous case.

Proposal 6: The total delay for non-simultaneous DCI-based BWP switching on multiple CC for NR-DC is defined as:

TTotal=TSingle+1N-1D

Where TTotal is the total BWP switching delay of a CC; TSingle is the existing delay requirements for BWP switching on a single CC. N is the total number of the non-simultaneous DCI-based BWP switching processes in different CG which overlap with TTotal. D is the incremental delay which is same as the simultaneous case.

For the timer-based case, according to TS 38.213 [2], the UE will delays the active UL/DL BWP change triggered by the BWP inactivity timer expiration until a subframe for FR1 or half a subframe for FR2 that is immediately after the UE completes the active UL/DL BWP change in the cell or in the different cell within FR1 (or FR2). Based on this understanding, the total delay is given as:

TTotal=TDelay+TSingle+1N-1D

Where TTotal is the total BWP switching delay of a CC; TDelay is the time delayed by ongoing timer-based BWP switching with in the same frequency range in the same CG. TSingle is the existing delay requirements for BWP switching on a single CC. N is the total number of the non-simultaneous timer-based BWP switching processes in different frequency range in the same CG and timer-based BWP switching processes in different CG which overlap with TTotal. D is the incremental delay which is same as the simultaneous case.

Proposal 7: The total delay for non-simultaneous Timer-based BWP switching on multiple CC for CA and NR-DC is defined as:

TTotal=TDelay+TSingle+1N-1D

Where TTotal is the total BWP switching delay of a CC; TDelay is the time delayed by ongoing timer-based BWP switching with in the same frequency range. TSingle is the existing delay requirements for BWP switching on a single CC. N is the total number of the non-simultaneous timer-based BWP switching processes in different frequency range which overlap with TTotal. D is the incremental delay which is same as the simultaneous case.

For the requirements of interruptions, for BWP switching on multiple CCs, the existing interruption requirements for a single CC shall apply for each BWP switching separately.

Proposal 8:

For the requirements of interruptions, for BWP switching on multiple CCs, the existing interruption requirements for a single CC shall apply for each BWP switching separately.

- Conclusions

Observation 1: The implementation and the capability for parallel processing may different.

Proposal 1: Define the BWP switching delay by counting the extra process delay of each additional CC.

Proposal 2: It is suggested to calculate the total absolute time delay regardless of SCSs of each CCs and then transforming the delay to the number of slots according to the SCSs of each CCs.

Proposal 3:

For DCI and timer triggered BWP switching:

For Type 1 UE, the incremental delay for each additional CC is 100 us. The BWP switching delay of a CC where there are simultaneous triggered BWP switching delay on other CCs is: 600 + 100 (N-1) us, where N ≤8  is the total number of CCs where the BWP switching is triggered by the same method (DCI or timer).

For Type 2 UE, the incremental delay for each additional CC is 200 us. The BWP switching delay of a CC where there are simultaneous triggered BWP switching delay on other CCs is: 2000 + 200 (N-1) us, where N ≤8  is the total number of CCs where the BWP switching is triggered by the same method (DCI or timer).

For RRC-based switching:

The existing delay requirements for single CC can be reused for on each individual CC when the BWP switching is simultaneously triggered on multiple CCs by 1 RRC command in each CG provided that the total number of CCs is less than 8.

Proposal 4:

For the requirements of interruptions, for BWP switching on multiple CCs, the existing interruption requirements for a single CC shall apply for each BWP switching separately.

Proposal 5: It is suggested that the existing delay requirements for single CC can be reused for on each individual CC when the BWP switching is non-simultaneously triggered on multiple CCs by 1 RRC command in each CG for NR-DC provided that the total number of CCs is less than 8.

Proposal 6: The total delay for non-simultaneous DCI-based BWP switching on multiple CC for NR-DC is defined as:

TTotal=TSingle+1N-1D

Where TTotal is the total BWP switching delay of a CC; TSingle is the existing delay requirements for BWP switching on a single CC. N is the total number of the non-simultaneous DCI-based BWP switching processes in different CG which overlap with TTotal. D is the incremental delay which is same as the simultaneous case.

Proposal 7: The total delay for non-simultaneous Timer-based BWP switching on multiple CC for CA and NR-DC is defined as:

TTotal=TDelay+TSingle+1N-1D

Where TTotal is the total BWP switching delay of a CC; TDelay is the time delayed by ongoing timer-based BWP switching with in the same frequency range. TSingle is the existing delay requirements for BWP switching on a single CC. N is the total number of the non-simultaneous timer-based BWP switching processes in different frequency range which overlap with TTotal. D is the incremental delay which is same as the simultaneous case.

Proposal 8:

For the requirements of interruptions, for BWP switching on multiple CCs, the existing interruption requirements for a single CC shall apply for each BWP switching separately.

References

[1] R4-1915933 “Way forward on BWP switching on multiple CCs”, Intel Corporation.

[2] TS 38.213 	NR; Physical layer procedures for control