3GPP RAN WG4 Meeting #94-e                                       R4-2001564

Online, 24th February – 6th March 2020

Agenda item:	8.1.4.14

Source: 	Huawei, HiSilicon

Title: 	Discussion on SI reading in NR-U

Document for:	Discussion

- Introduction

In last meeting RAN4 93, the RRM requirements and impact of NR-U are discussed, and the potential agreements and the remaining open issues in the WF [1]. For SI reading, the simulation assumptions are given to derive the number of samples for SI reading timing. The simulation results are presented in this paper according to the simulation assumptions.

- Discussion

In the last RAN4 93 meeting, the SIB reading time was discussed and the two options are presented for the SIB reading:

Option 1: SIB reading time 1280 ms

Option 2:

- Number of PBCH samples for MIB is 5

- Number of PDSCH samples for SIB1 is based on simulation results with 95% of decoding success rate.

The simulation assumptions are as follows:

| Parameters | Unit | Value |
| --- | --- | --- |
| Channel bandwidth | MHz | 20 |
| SCS | kHz | 15 |
| Number of transmitter antennas |  | 1 |
| Number of receive antennas |  | 2 |
| Allocated resource blocks for PDSCH |  | 24 (allocated in the lower edge of the carrier) |
| MCS index |  | 4 |
| Modulation |  | QPSK |
| Target Coding Rate |  | 0.33 |
| Number of control symbols |  | 2 |
| PDSCH mapping type |  | Type A |
| Information Bit Payload |  |  |
| For slots with RMSI | Bits | 1864 |
| Number of Code Blocks per slot |  | 1 |
| Binary Channel Bits Per slot |  |  |
| For slots with RMSI | Bits | 6048 |
| Side condition (PDSCH SNR) | dB | -6 |
| Propagation channel model |  | AWGN, TDLA30-10, TDLB100-400, TDLC300-100 |
| Receiver assumption |  | - Single shot detection - Soft-combining (2, 4 and 8 samples) |
| SIB1 transmission period | ms | 20 |
| Metric |  | 95% of SIB1 acquisition success rate |
| Derived based on the PDSCH DMRS assumption: dmrs-TypeA-Position=2, dmrs-Type=1, dmrs-AdditonalPositions=2, maxLength=1, Antenna port index: 1000, and Number of PDSCH DMRS CDM group(s) without data: 1. |  |  |

Based on the simulation assumptions, the simulation results are presented as follows:

|  | AWGN | TDLA | TDLB | TDLC |
| --- | --- | --- | --- | --- |
| single | >40 | 17 | 28 | 15 |
| Soft-combine 2 | 2 | 7 | 12 | 8 |
| Soft-combine 4 | 2 | 4 | 6 | 4 |
| Soft-combine 8 | 2 | 4 | 5 | 4 |

From the results, it can be observed that

- With AWGN, SIB1 cannot be decoded without soft combining

- The performance improves with soft combining size

Based on above observations, we suggest to define the SIB1 decoding delay requirements based on soft combining assumption. SIB1 TTI is 160ms, and the default scheduling period is 20ms, which means the largest combining size is 8. However, the actual scheduling period is up to network, e.g. network may schedule SIB1 transmission every 80ms, and in this case UE can do only size-2 combination. Therefore, we suggest to define the requirements based on the minimum soft combining size, i.e. 2. This would maximize the requirement applicability.

Based on our simulation results, the number of SIB1 transmissions required for 95% decoding rate is 12, and considering the case where UE starts decoding from the last transmission in a TTI, we suggest to use 13 SIB1 transmissions for the baseband decoding delay.

Proposal: The baseband SIB1 decoding delay is 13 SIB1 transmissions, provided that the scheduling periodicity is no larger than 80ms.

- Conclusions

Proposal: The baseband SIB1 decoding delay is 13 SIB1 transmissions, provided that the scheduling periodicity is no larger than 80ms.

References

[1] R4-1915777 “WF on NR-U RRM Requirements”, Ericsson, Qualcomm Inc.

[2] TS 38.133