3GPP TSG-RAN WG4 Meeting #94-e 	R4-2001703

Online, 24 Feb - 06 Mar 2020

Source: 	Huawei

Title: 	TP to TR 37.941 : Colocation MU value derivation sub-clause updates

Agenda Item:	8.19

Document for:	Discussion

# Introduction

The external TR for OTA BS testing consolidates all the MU tables from the 3 internal TR’s. The generation of the TR is being used to review and correct all the MU calculations and tables.

The co-location section in the TR 37.843 has many empty classes and is not very easy to understand, this paper suggests some improvements to the section which can be incorporated in the new TR

Co-location requirements are only applicable to FR1 currently so the focus is on FR1 and the text from TR 37.843

Note the changes in this TP are on top of TR 37.9xx version 0.1.0 which is in itself a merger of the 3 donor TR’s.

# 2. Discussion

## 2.1	OFF power

TX OFF power is measured as an output from the CLTA. AS such the chamber is not very important. It must be capable of screening the system from outside interference which could influence the measurement, and it should avoid any reflections which may influence the coupling between the BS and the CLTA.

This can be seen in the MU budget contributors:

| UID | Uncertainty source |
| --- | --- |
| Stage 2: DUT measurement |  |
|  | Uncertainty related to the selection of the CLTA (Note) |
|  | Uncertainty related to the placement of the CLTA (Note) |
| 2 | Uncertainty related to measuring close to noise floor |
|  | Impedance mismatch between feeder cable and CLTA |
|  | Gain variations in LNA |
| 9 | Random uncertainty |
|  | Measurement receiver accuracy |
|  | Reflections in anechoic chamber |
|  | Gain variations in measurement amplifier |
| Stage 1: Calibration measurement |  |
| 7 | Noise figure measurement accuracy |

Only 1 of these error sources is related to the chamber (reflections in the anechoic chamber) and this is a very small contributor (0.01dB)

Currently in TR37.843 there are 3 chamber types listed and one set of information is distributed amongst these

IAC – reference CATR for calibration and  procedure all other sub-clauses empty

CATR – contains calibration and measurement procedures (which don’t appear to be range specific), all other sub-clauses empty

General chamber – contains MU budget assessment – all other sub-clauses empty.

Between the 3 listed chamber types there is 1 set of complete information, which is not chamber specific.

To clear this up it is proposed to list all the information under a general chamber (which I believe was the original intention)

## 2.2	Co-location spurious emissions

This is a similar measurement to the TX OFF level, the MU budget is different however as the wanted signal can be filtered out (rather than switched or limited).

The current background captured in TR 37.843 is at a similar level to the TX OFF background.

It is proposed to use a similar method as TX OFF  in the new TR to capture the co-location spurious emissions background.

## 2.3 TX IMD

The TX IMD requirement is based around measuring spurious and unwanted emissions. The MU for each of these is referenced to the appropriate TRP measurement section. The additional information concerning the co-location aspects is the accuracy of the injected interferer. AS with the other colocation requirements the accuracy of the CLTA to BS coupling is not directly affected by the chamber type. Once again the general chamber (to indicate any appropriate chamber) has been used in the analysis.

The current text in TR 37.843 has 3 chamber types, IAC, CATR and general.

IAC – diagram in general section, calibration and procedure present (somewhat chamber agnostic descriptions), incomplete MU budget (no final value)

CATR – contains calibration and measurement procedures (which don’t appear to be chamber specific), all other sub-clauses empty

Shielded anechoic chamber – this is an anomaly and appears to be attempting the same thing as the general chamber. General section, calibration ad procedure present but not MU budget analysis.

General chamber – contains MU budget assessment – all other sub-clauses empty.

It is proposed to put all the information under the general chamber (with appropriate explanations) and remove the other chamber types.

## 2.4 Co-location Blocking

The co-location blocking measurement uncertainty is based on adding (RSS) the wanted signal error and the interferer accuracy.  In this subclasue the MU tables deal with the interferer accuracy only as the wanted signal is referenced to the EIS section. As with the other colocation requirements the accuracy of the CLTA to BS coupling is not directly affected by the chamber type. Once again the general chamber (to indicate any appropriate chamber) has been used in the analysis.

The current text in TR 37.843 has 3 chamber types, IAC, CATR and general.

IAC – reference CATR for calibration and procedure all other sub-clauses empty

CATR – contains general, calibration and measurement procedures (which don’t appear to be range specific), all other sub-clauses empty

General chamber – contains MU budget assessment – all other sub-clauses empty.

It is proposed to put all the information under the general chamber (with appropriate explanations) and remove the other chamber types.

# TP to TR 37.941 v0.1.0

--- Start of changes ---

## 7.8	General OTA chamber

### 7.8.1	Measurement system description

The general OTA chamber represents any suitable OTA chamber which shields the BS and CLTA form external interference and prevents reflections within the chamber form altering the coupling between the BS and the CLTA

Figure 7.8.1-1: General OTA chamber test system set up diagram for co-location TX OFF and co-location emissions

The OTA measurement receiver must be capable of conforming that the BS is radiating the wanted signal at the required power level, however the measurement is made at the output of the CLTA. The diagram shows a switch/limiter or filter which is used to reduce the level of the wanted signal before the requirement is measured in the measurement device.

Figure 7.8.1-2: General OTA chamber test system set up diagram for co-location TX IMD

The general chamber represents any chamber capable of performing the OTA transmitter spurious emissions requirements, the OTA operating band unwanted emissions requirements and the OTA ACLR requirements to the specified accuracy.

--- Next change ---

## 8.8	General OTA calibration

The OTA path of the general chamber is calibrated according to its chamber type using the methodology in sub-clause 8.2 to 8.6.

Figure 8.8.1-1: General OTA chamber test system set up diagram for co-location TX OFF and co-location emissions

Figure 8.8.1-2: General OTA chamber test system set up diagram for co-location TX IMD

The CLTA path is calibrated as follows:

1)	Cable and matching loss calibration for CLTA for each polarization supported by the CLTA

a)	Measure CLTA reflection coefficient separately at the antenna's connector with a network analyser (or equivalent measurement equipment) to obtain ΓANT.

b)	Measure cable loss from input of CLTA (A) to point B for TX OFF and co-location spurious emissions and from point B to input of CLTA (A) for TX IMD, call this L ANT↔F which is the equivalent of 20log|S21| from the use of a network analyser.

c)	Calculate the combined total path loss from F to CLTA using the following equating

Lcal = L ANT↔F + 10log(1 - |ΓANT|2)

--- Next change ---

## 13.2	OTA transmitter OFF power – FR1

### 13.2.1	General

This subclause captures MU and TT values derivation for the OTA transmitter OFF power co-location requirement in Normal test conditions.

OTA transmitter ON/OFF power requirements apply only to TDD operation.

The OTA transmit ON/OFF power requirements are co-location requirements and specified as the power sum of the supported polarization(s) at the CLTA conducted output(s).

The FR1 transmitter OFF power and transmitter transient period is a co-location requirement while the FR2 transmitter OFF power requirement is a directional requirement.

It is not possible to separate OTA emissions from separate transceiver units therefore OTA emissions requirements are specified as the sum of the emissions from all transceiver units in the BS. For spurious emissions this has been the case for both conducted and OTA emissions. The sum of the total emissions from the BS is intended to be no greater than that of a non-AAS system with equivalent number of MIMO branches and hence emission are capped at a level consistent with the maximum MIMO capability of the appropriate RAT (i.e. 8 for E-UTRA and NR, 4 for UTRA), however conducted TX OFF level is specified per TAB connector and as such is not subject to the same cap. However as the conducted requirement is primarily concerned with protection of your own receiver and OTA is concerned with co-location the parameters have somewhat changed.

As a co-located TDD receiver should be offered the same protection as any other co-located receiver it is reasonable to apply the same cap on the TX OFF level as is applied to other unwanted emissions.

The FR1 transmitter OFF power is measured as the conducted output of the CLTA, no accurate measurements are required in the chamber itself. The MU analysis of the TX OFF measurement is therefore agnostic to the chamber used. The chambers main purpose is to shield the BS and the CLTA form external interference which may influence the result and to avoid RF reflections which may alter the coupling between the BS and the CLTA.

Hence the MU analysis for FR1 TX OFF is done using a “General chamber” under the assumption that any of the chambers discussed (with the exception of the Reverb chamber) will be suitable.

### 13.2.2	General OTA chamber

#### 13.2.2.1	Measurement system description

The general OTA chamber description is intended to cover any suitable chamber type which screens the measurement system for outside interference and does not affect the coupling between the BS and the CLTA.

Measurement system description is captured in sub-clause 7.8.

#### 13.2.2.2 	Test procedure

##### 13.2.2.2.1	Stage 1: Calibration

Calibration for wanted signal power level is the same as in subclause 9.2.

Calibration for the CLTA path is described in subclasue 8.8.

##### 13.2.2.2.2	Stage 2: measurement

The testing procedure consists of the following steps:

1)	Place BS and CLTA as specified for the co-location requirement in TS 37.145-2 [4] subclause 4.15 for AAS BS, and in TS 38.141-2 [6] subclause 4.12 for NR BS.

2)	Place range antenna in boresight direction (reference direction) at far-field distance, aligned in both polarizations with the BS.

3)	The Range antenna shall be dual (or single) polarized with the same frequency range as the BS for transmitter OFF power test case.

4)	Connect range antenna and co-location test antenna to the measurement equipment.

5)	OTA transmitter OFF power is measured at the co-location test antenna conducted output(s).

6)	The measurement device (signal analyser) characteristics shall be: Detection mode: True RMS.

7)	Set the BS to transmit:

-	For BS declared to be capable of single carrier operation only, set the BS to transmit full beam power (rated beam EIRP), both polarizations, in boresight direction (reference direction), at manufacturer's declared rated output power, Prated,t,TRP.

-	For BS declared to be capable of multi-carrier and/or CA operation, set the BS to transmit full beam power (rated beam EIRP), both polarizations, in boresight direction (reference direction) on all carriers configured using the applicable test configuration and corresponding power setting.

-	Check that specified beam power (EIRP) is obtained at range antenna RF output(s) (conducted side) for each polarization. Re-align if the specified beam power is not achieved.

8)	Measure the mean power spectral density from all co-location test antenna conducted output(s) over 70 μs filtered with a square filter of bandwidth equal to the RF bandwidth of the RIB centred on the central frequency of the RF bandwidth. 70 μs average window centre is set from 35 μs after end of one transmitter ON period + 17 μs to 35 μs before start of next transmitter ON period - 6.25 μs.

Additional factor to be considered in the measurement is that the test requirement is very close to thermal noise floor, and the measurement setup needs to be able to tolerate both very high and low signal levels. This will impact measurement uncertainty.

#### 13.2.2.3	MU value derivation, FR1

Table 13.2.2.3-1: General OTA chamber MU value derivation for TDD OFF power level measurement

| General Directional Chamber |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| UID | Uncertainty source | Uncertainty value |  |  | Distribution of the probability | Divisor based on distribution shape | ci | Standard uncertainty ui [dB] |  |  |
|  |  | f≤3 GHz | 3<f≤4.2 GHz | 4.2<f≤6 GHz |  |  |  | f≤3 GHz | 3<f≤4.2 GHz | 4.2<f≤6 GHz |
| Stage 2: DUT measurement |  |  |  |  |  |  |  |  |  |  |
| C2-9 | Uncertainty related to the selection of the CLTA (Note) | 1.5 | 1.5 | 1.5 | Rectangular | 1.73 | 1 | 0.87 | 0.87 | 0.87 |
| C2-10 | Uncertainty related to the placement of the CLTA (Note) | 1.7 | 1.7 | 1.7 | Rectangular | 1.73 | 1 | 0.98 | 0.98 | 0.98 |
| C2-12 | Uncertainty related to measuring close to noise floor Tx OFF | 1 | 1 | 1 | Gaussian | 1.00 | 1 | 1.00 | 1.00 | 1.00 |
| C2-13 | Impedance mismatch between feeder cable and CLTA | 0.14 | 0.23 | 0.25 | U-Shaped | 1.41 | 1 | 0.10 | 0.16 | 0.18 |
| C2-14 | Gain variations in LNA | 0.1 | 0.1 | 0.1 | Gaussian | 1.00 | 1 | 0.10 | 0.10 | 0.10 |
| A5-8 | Random uncertainty | 0.1 | 0.1 | 0.1 | Rectangular | 1.73 | 1 | 0.06 | 0.06 | 0.06 |
| C1-5 | Measurement Receiver (Co-location) | 0.41 | 0.74 | 0.8 | Gaussian | 1.00 | 1 | 0.41 | 0.74 | 0.80 |
| A5-20 | Reflections in anechoic chamber | 0.01 | 0.01 | 0.01 | Gaussian | 1.00 | 1 | 0.01 | 0.01 | 0.01 |
| C2-15 | Gain variations in measurement amplifier | 0.1 | 0.1 | 0.1 | Gaussian | 1.00 | 1 | 0.10 | 0.10 | 0.10 |
| Stage 1: Calibration measurement |  |  |  |  |  |  |  |  |  |  |
| C1-6 | Noise figure measurement accuracy | 0.2 | 0.2 | 0.2 | Gaussian | 1.00 | 1 | 0.2 | 0.2 | 0.2 |
| Combined standard uncertainty (1σ) [dB] |  |  |  |  |  |  |  | 1.72 | 1.83 | 1.86 |
| Expanded uncertainty (1.96σ - confidence interval of 95 %) [dB] |  |  |  |  |  |  |  | 3.37 | 3.59 | 3.64 |

UID are referenced to annex A, B or C as appropriate.

### 13.2.3	Maximum accepted test system uncertainty

Maximum test system uncertainties derivation methodology was described in subclause 5.1. The maximum accepted test system uncertainty values was derived based on test system specific values in subclasue 13.2.2.

Table 13.2.3-1: Common maximum accepted test system uncertainty value derivation for TDD OFF power level measurement

|  | Expanded uncertainty [dB] |  |  |
| --- | --- | --- | --- |
|  | f≤3 GHz | 3<f≤4.2 GHz | 4.2<f≤6 GHz |
| General Directional Chamber | 3.37 | 3.59 | 3.64 |
| Common maximum accepted test system uncertainty | 3.40 | 3.60 | 3.60 |

Fulfilling the criteria for CLTA selection and placement in is deemed sufficient for the test purposes. When these criteria are met, the measurement uncertainty related to the selection of the co-location test antenna and its alignment as shall be used for evaluating the test system uncertainty.

### 13.2.4	Test Tolerance for OTA TX OFF power

Considering the methodology described in subclause 5.1, Test Tolerance values for TX OFF were derived based on values captured in subclause 13.2.2.

The TT was decided to be the same as the MU for TX OFF in FR1.

Table 13.2.4-1: Test Tolerance values for the TX OFF in Normal test conditions

|  | f≤3 GHz | 3<f≤4.2 GHz | 4.2<f≤6 GHz |
| --- | --- | --- | --- |
| Test Tolerance (dB) | 3.4 | 3.7 | 3.7 |

An overview of the TT values for all the requirements is captured in clause 17.

--- Next change ---

## 13.3	OTA co-location spurious emissions

### 13.3.1	General

This subclause captures MU and TT values derivation for the OTA co-location spurious emissions requirement in Normal test conditions.

The FR1 OTA co-location emissions are measured as the conducted output of the CLTA, no accurate measurements are required in the chamber itself. The MU analysis of the co-location emissions measurement is therefore agnostic to the chamber used. The chambers main purpose is to shield the BS and the CLTA form external interference which may influence the result and to avoid RF reflections which may alter the coupling between the BS and the CLTA.

Hence the MU analysis for FR1 co-location emissions is done using a “General chamber” under the assumption that any of the chambers discussed (with the exception of the Reverb chamber) will be suitable.

### 13.3.2	General Chamber

#### 13.3.2.1	Measurement system description

The general OTA chamber description is intended to cover any suitable chamber type which screens the measurement system for outside interference and does not affect the coupling between the BS and the CLTA.

Measurement system description is captured in sub-clause 7.8.

#### 13.3.2.2 	Test procedure

##### 13.3.2.2.1	Stage 1: Calibration

Calibration for wanted signal power level is the same as in subclause 9.2.

Calibration for the CLTA path is described in subclasue 8.8.

##### 13.3.2.2.2	Stage 2: BS measurement

1)	Place AAS BS and CLTA as specified in subclause 4.15, 3GPP TS 37.145-2 [24].

2)	Place Range antenna in boresight direction (reference direction) at far-field distance, aligned in both polarizations with the AAS BS.

3)	Connect Range antenna and CLTA to the measurement equipment.

4)	OTA co-location spurious emissions are measured at the CLTA conducted output(s).

5)	The measurement device (signal analyser) characteristics shall be:

-	Detection mode: True RMS.

6)	Set the AAS BS to transmit:

-	For AAS BS declared to be capable of single carrier operation only, set the AAS BS to transmit full beam power (rated beam EIRP), both polarizations, either simultaneously or sequentially, in boresight direction (reference direction), at manufacturer's declared rated output power, Prated,t,TRP.

-	For AAS BS declared to be capable of multi-carrier and/or CA operation, set the AAS BS to transmit full beam power (rated beam EIRP), both polarizations, in boresight direction (reference direction) on all carriers configured using the applicable test configuration and corresponding power setting.

-	Check that specified beam power (EIRP) is obtained at Range antenna RF output(s) (conducted side) for each polarization. Re-align if the specified beam power is not achieved.

7)	Measure the mean power spectral density from all CLTA conducted output(s) with each wanted signal polarization under test.

Additional factor to be considered in the measurement is that the test requirement is very close to thermal noise floor, and the measurement setup needs to be able to tolerate both very high and low signal levels. This will impact measurement uncertainty.

#### 13.3.2.3	MU value derivation, FR1

Table 13.3.2.3-1: General OTA chamber MU value derivation for co-location emissions level measurement

| General Directional Chamber |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| UID | Uncertainty source | Uncertainty value |  |  | Distribution of the probability | Divisor based on distribution shape | ci | Standard uncertainty ui [dB] |  |  |
|  |  | f≤3 GHz | 3<f≤4.2 GHz | 4.2<f≤6 GHz |  |  |  | f≤3 GHz | 3<f≤4.2 GHz | 4.2<f≤6 GHz |
| Stage 2: DUT measurement |  |  |  |  |  |  |  |  |  |  |
| C2-9 | Uncertainty related to the selection of the CLTA (Note) | 1.5 | 1.5 | 1.5 | Rectangular | 1.73 | 1 | 0.87 | 0.87 | 0.87 |
| C2-10 | Uncertainty related to the placement of the CLTA (Note) | 1.7 | 1.7 | 1.7 | Rectangular | 1.73 | 1 | 0.98 | 0.98 | 0.98 |
| C2-11 | Uncertainty related to measuring close to noise floor - Emissions | 0.68 | 0.68 | 0.68 | Gaussian | 1.00 | 1 | 0.68 | 0.68 | 0.68 |
| C2-13 | Impedance mismatch between feeder cable and CLTA | 0.14 | 0.23 | 0.25 | U-Shaped | 1.41 | 1 | 0.10 | 0.16 | 0.18 |
| C2-14 | Gain variations in LNA | 0.1 | 0.1 | 0.1 | Gaussian | 1.00 | 1 | 0.10 | 0.10 | 0.10 |
| A5-8 | Random uncertainty | 0.1 | 0.1 | 0.1 | Rectangular | 1.73 | 1 | 0.06 | 0.06 | 0.06 |
| C1-5 | Measurement Receiver (Co-location) | 0.41 | 0.74 | 0.8 | Gaussian | 1.00 | 1 | 0.41 | 0.74 | 0.80 |
| A5-20 | Reflections in anechoic chamber | 0.01 | 0.01 | 0.01 | Gaussian | 1.00 | 1 | 0.01 | 0.01 | 0.01 |
| C2-15 | Gain variations in measurement amplifier | 0.1 | 0.1 | 0.1 | Gaussian | 1.00 | 1 | 0.10 | 0.10 | 0.10 |
| Stage 1: Calibration measurement |  |  |  |  |  |  |  |  |  |  |
| C1-6 | Noise figure measurement accuracy | 0.2 | 0.2 | 0.2 | Gaussian | 1 | 1 | 0.2 | 0.2 | 0.2 |
| Combined standard uncertainty (1σ) [dB] |  |  |  |  |  |  |  | 1.55 | 1.68 | 1.71 |
| Expanded uncertainty (1.96σ - confidence interval of 95 %) [dB] |  |  |  |  |  |  |  | 3.05 | 3.29 | 3.34 |

UID are referenced to annex A, B or C as appropriate.

### 13.3.3	Maximum accepted test system uncertainty

Maximum test system uncertainties derivation methodology was described in subclause 5.1. The maximum accepted test system uncertainty values was derived based on test system specific values in subclasue 13.3.2.

Table 13.3.3-1: Common maximum accepted test system uncertainty value derivation for co-location emissions level measurement

|  | Expanded uncertainty [dB] |  |  |
| --- | --- | --- | --- |
|  | f≤3 GHz | 3<f≤4.2 GHz | 4.2<f≤6 GHz |
| General Directional Chamber | 3.05 | 3.29 | 3.34 |
| Common maximum accepted test system uncertainty | 3.2 | 3.4 | 3.4 |

Fulfilling the criteria for CLTA selection and placement in is deemed sufficient for the test purposes. When these criteria are met, the measurement uncertainty related to the selection of the co-location test antenna and its alignment as shall be used for evaluating the test system uncertainty.

### 13.3.4	Test Tolerance co-location spurious emissions

Considering the methodology described in subclause 5.1, Test Tolerance values for co-location emissions were derived based on values captured in subclause 13.3.2.

The TT was decided to be the same as the MU for TX OFF in FR1.

Table 13.3.4-1: Test Tolerance values for the co-location emissions in Normal test conditions

|  | f≤3 GHz | 3<f≤4.2 GHz | 4.2<f≤6 GHz |
| --- | --- | --- | --- |
| Test Tolerance (dB) | 3.4 | 3.7 | 3.7 |

An overview of the TT values for all the requirements is captured in clause 17.

--- Next change ---

## 13.4	OTA transmitter intermodulation

### 13.4.1	General

This subclause captures MU and TT values derivation for the OTA transmitter intermodulation co-location requirement in Normal test conditions.

The transmitter intermodulation requirement is that the spurious emissions requirements (OTA transmitter spurious emissions, OTA operating band unwanted emissions/SEM and OTA ACLR) are met whilst an interferer is injected into the antenna from a co-located bases station.

The measurement method and uncertainty for those measurements is the same as described in subclauses 12.2, 11.3 and 11.4

There is an additional uncertainty however associated with the accuracy of the interfering signal injected from the CLTA. It is that uncertainty which is investigated in this clause

The interferer is injected into the CLTA and the accuracy of this is not dependent on the chamber type used. The measurement uncertainty calculation is therefore done using a general chamber type

### 13.4.2	General OTA chamber

#### 13.4.2.1	Measurement system description

This method measures the OTA TX IMD in a general OTA chamber.

Measurement system description is captured in subclause 7.8.

#### 13.4.2.2 	Test procedure

##### 13.4.2.2.1	Stage 1: Calibration

Calibration for the emissions measurement is the same as in subclause 11.3, 11.5 and 12.2.

Calibration for the CLTA path is described in subclasue 8.8.

##### 13.4.2.2.2	Stage 2: BS measurement

The testing procedure consists of the following steps:

1)	Uninstall the reference antenna and install the BS with the manufacturer declared coordinate system reference point in the same place as the phase centre of the reference antenna. The manufacturer declared coordinate system orientation of the BS is set to be aligned with the testing system.

2)	Set the CLTA as specified in in TS 37.145-2 [4] subclause 4.15 for AAS BS, and in TS 38.141-2 [6] subclause 4.12 for NR BS, at the declared worst case side.

3)	Set the signal generator to generate the interfering signal to achieve the required level at the input port of CLTA

##### 4)	Perform required ACLR, OBUE (for E-UTRA/NR), SEM (for UTRA), occupied bandwidth, spurious emission tests according to the corresponding testing procedures.

#### 13.4.2.3	MU value derivation, FR1

Table 13.4.4.3-1 MU for OTA transmitter intermodulation interferer signal

| General Chamber |  |  |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| UID | Uncertainty source | Uncertainty value |  |  | Distribution of the probability | Divisor based on distribution shape | ci |  | Standard uncertainty ui [dB] |  |  |  |  |  |
|  |  | f≤3 GHz | 3<f≤4.2 GHz | 4.2<f≤6 GHz |  |  |  |  | f≤3 GHz |  | 3<f≤4.2 GHz |  | 4.2<f≤6 GHz |  |
| Stage 2: DUT measurement |  |  |  |  |  |  |  |  |  |  |  |  |  |
| C2-9 | Uncertainty related to the selection of the CLTA (Note) | 1,50 | 1,50 | 1,50 | Rectangular | 1,73 | 1 |  | 0,87 |  | 0,87 |  | 0,87 |  |
| C2-10 | Uncertainty related to the placement of the CLTA (Note) | 1,70 | 1,70 | 1,70 | Rectangular | 1,73 | 1 |  | 0,98 |  | 0,98 |  | 0,98 |  |
| C2-13 | Impedance mismatch between feeder cable and CLTA | 0,14 | 0,23 | 0,25 | U-Shaped | 1,41 | 1 |  | 0,10 |  | 0,16 |  | 0,18 |  |
| A5-8 | Random uncertainty | 0,10 | 0,10 | 0,10 | Rectangular | 1,73 | 1 |  | 0,06 |  | 0,06 |  | 0,06 |  |
| A5-20 | Reflections in anechoic chamber | 0,01 | 0,01 | 0,01 | Gaussian | 1,00 | 1 |  | 0,01 |  | 0,01 |  | 0,01 |  |
| Stage 1: Calibration measurement |  |  |  |  |  |  |  |  |  |  |  |  |  |
| C2-7 | TX IMD - conducted measurement uncertainty | 1,00 | 1,10 | 1,20 | Gaussian | 1,00 | 1 |  | 1,00 |  | 1,10 |  | 1,20 |  |
| Combined standard uncertainty (1σ) [dB] |  |  |  |  |  |  |  | 1,65 |  | 1,72 |  | 1,79 |  |
| Expanded uncertainty (1.96σ - confidence interval of 95 %) [dB] |  |  |  |  |  |  |  | 3,24 |  | 3,37 |  | 3,50 |  |

UID are referenced to annex A, B or C as appropriate.

### 13.4.3	Maximum accepted test system uncertainty

Maximum test system uncertainties derivation methodology was described in subclause 5.1. The maximum accepted test system uncertainty values was derived based on test system specific values in subclasue 13.4.4.

Table 13.4.3-1: Common maximum accepted test system uncertainty value derivation for co-location TX IMD interferer signal level

|  | Expanded uncertainty [dB] |  |  |
| --- | --- | --- | --- |
|  | f≤3 GHz | 3<f≤4.2 GHz | 4.2<f≤6 GHz |
| General Chamber | 3,24 | 3,37 | 3,51 |
| Common maximum accepted test system uncertainty | 3,20 | 3,40 | 3,50 |

Fulfilling the criteria for CLTA selection and placement in is deemed sufficient for the test purposes. When these criteria are met, the measurement uncertainty related to the selection of the co-location test antenna and its alignment as shall be used for evaluating the test system uncertainty.

### 13.4.4	Test Tolerance co-location spurious emissions

The test tolerance for the emissions measurements (OTA transmitter spurious emissions, OTA operating band unwanted emissions/SEM and OTA ACLR) is as specified for each specific requirement.

The test tolerance for the interferer level is zero.

--- Next change ---

## 13.5	OTA co-location blocking

### 13.5.1	General

This subclause 13.5 captures MU and TT values derivation for the OTA co-location blocking requirement in Normal test conditions.

The OTA co-location blocking measurement uncertainty is calculated as a combination of the wanted signal MU and the interferer signal MU in the same way as the normal out of band blocking requirement, where:

MUcolocation blocking=MUwanted signal2+MUinterferer2+Noiseeffect

The Noiseeffect from the signal generator is 0.1dB and the MUwantedsignal value is MUEIS from subclause 10.2.

The measurement uncertainty associated with the interferer is based on the accuracy of the interfering signal injected from the CLTA. It is that uncertainty which is investigated in this clause

The interferer is injected into the CLTA and the accuracy of this is not dependent on the chamber type used. The measurement uncertainty calculation is therefore done using a general chamber type.

Any chamber capable of performing the EIS measurement to the required accuracy is suitable for the test however.

### 13.5.2	General OTA chamber

#### 13.5.2.1	Measurement system description

This method measures the OTA co-location blocking characteristics in a general OTA chamber.

Measurement system description is captured in subclause 7.8.

#### 13.5.2.2 	Test procedure

##### 13.5.2.2.1	Stage 1: Calibration

Calibration for wanted signal power level is the same as in subclause 10.2.

##### Calibration for the CLTA path is described in subclasue 8.8.

##### 13.5.2.2.2	Stage 2: BS measurement

1)	Place NR BS and CLTA as specified in TS 37.145-2 [4] subclause 4.15 for AAS BS, and in TS 38.141-2 [6] subclause 4.12 for NR BS, at the declared worst case side. Several CLTA are required to cover the whole co-location blocking frequency ranges. The CLTA shall be selected according to subclause 4.12.2.2.

2)	Align the NR BS and test antenna(s) according to the directions to be tested.

3)	Connect test antenna and CLTA to the measurement equipment as depicted in subclause 7.8

5)	The NR BS receives the wanted signal in all supported polarizations, in the receiver target reference direction from the test antenna.

6)	The OTA co-location blocking interferer is injected via the CLTA. The CLTA is fed with the specified co-location blocking interferer power per supported polarization.

7)	Generate the wanted signal in receiver target reference direction, all supported polarizations, from the test antenna, according to the applicable test configuration using applicable reference measurement channel to the RIB.

8)	Configure the beam peak direction for the transmitter units associated with the RIB under test according to the declared reference beam direction pair for the appropriate beam identifier with the carrier set-up and power allocation according to the applicable test configuration(s) (see subclause 4.8). The transmitter may be turned OFF for the out-of-band blocker tests when the frequency of the blocker is such that no IM2 or IM3 products fall inside the bandwidth of the wanted signal.

9)	Adjust the signal generators to the type of interfering signals, levels and the frequency offsets as specified for general test requirements and, when applicable, for co-location test requirements.

10)	The CW interfering signal shall be swept with a step size of 1 MHz within the frequency range corresponding to downlink operating bands releated to co-located systems.

11)	Measure the performance of the wanted signal at the receiver unit associated with the RIB

#### 13.5.2.3	MU value derivation, FR1

The MU for the interferer signal is as follows:

Table 13.5.2.3-1: MU for co-location blocking interferer signal

| General Chamber |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| UID | Uncertainty source | Uncertainty value |  |  | Distribution of the probability | Divisor based on distribution shape | ci |  | Standard uncertainty ui [dB] |  |  |  |  |  |  |  |
|  |  | f≤3 GHz | 3<f≤4.2 GHz | 4.2<f≤6 GHz |  |  |  |  | f≤3 GHz |  | 3<f≤4.2 GHz |  |  | 4.2<f≤6 GHz |  |  |
| Stage 2: DUT measurement |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| C2-9 | Uncertainty related to the selection of the CLTA (Note) | 1,50 | 1,50 | 1,50 | Rectangular | 1,73 | 1 |  | 0,87 |  | 0,87 |  |  | 0,87 |  |  |
| C2-10 | Uncertainty related to the placement of the CLTA (Note) | 1,70 | 1,70 | 1,70 | Rectangular | 1,73 | 1 |  | 0,98 |  | 0,98 |  |  | 0,98 |  |  |
| C2-13 | Impedance mismatch between feeder cable and CLTA | 0,14 | 0,23 | 0,25 | U-Shaped | 1,41 | 1 |  | 0,10 |  | 0,16 |  |  | 0,18 |  |  |
| A5-8 | Random uncertainty | 0,10 | 0,10 | 0,10 | Rectangular | 1,73 | 1 |  | 0,06 |  | 0,06 |  |  | 0,06 |  |  |
| A5-3 | Quality of quiet zone | 0,10 | 0,10 | 0,10 | Gaussian | 1,00 | 1 |  | 0,10 |  | 0,10 |  |  | 0,10 |  |  |
| Stage 1: Calibration measurement |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| C3-9 | Colocation blocking - conducted measurement uncertainty | 1,00 | 1,10 | 1,20 | Gaussian | 1,00 | 1 |  | 1,00 |  | 1,10 |  |  | 1,20 |  |  |
| Combined standard uncertainty (1σ) [dB] |  |  |  |  |  |  |  | 1,65 |  | 1,72 |  |  | 1,79 |  |  |
| Expanded uncertainty (1.96σ - confidence interval of 95 %) [dB] |  |  |  |  |  |  |  | 3,24 |  | 3,37 |  |  | 3,51 |  |  |

UID are referenced to annex A, B or C as appropriate.

### 13.5.3	Maximum accepted test system uncertainty

The final MU is calculated as follows:

The Noiseeffect from the signal generator is 0.1dB and the MUwantedsignal value is MUEIS from subclause 10.2.7

.Where MUinterferer is derived in subclause 13.2.4 and is given in table 13.4.5-1

Table 13.5.3-1: Common maximum accepted test system uncertainty value derivation for co-location blocking interferer signal level

|  | Expanded uncertainty [dB] |  |  |
| --- | --- | --- | --- |
|  | f≤3 GHz | 3<f≤4.2 GHz | 4.2<f≤6 GHz |
| General Chamber | 3,24 | 3,37 | 3,51 |
| Common maximum accepted test system uncertainty | 3,20 | 3,40 | 3,50 |

Fulfilling the criteria for CLTA selection and placement in is deemed sufficient for the test purposes. When these criteria are met, the measurement uncertainty related to the selection of the co-location test antenna and its alignment as shall be used for evaluating the test system uncertainty.

The final values are given in table 10.7.3-1

Table 13.5.3-2: MU values for out of co-location band blocking

| Co-location blocking MU (dB) |  | Wanted signal operating band |  |  |
| --- | --- | --- | --- | --- |
|  |  | 30MHz<f≤3 GHz | 3GHz<f ≤4.2GHz | 4.2<f≤6 GHz |
| Interferer frequency | 30MHz<f≤3 GHz | 3,4 | 3,5 | 3,6 |
|  | 3GHz<f ≤4.2GHz | 3,5 | 3,6 | 3,8 |
|  | 4.2<f≤6 GHz | 3,7 | 3,7 | 3,8 |

### 13.5.4	Test Tolerance for OTA co-location blocking

The test tolerance for co-location out of band blocking is zero.

--- End of changes ---