3GPP TSG-RAN WG4 Meeting #94-e	R4-2001712

Electronic meeting, 24 February-6 March 2020

Source:	Ericsson

Title:	Introduction of standalone NR-U combinations in Rel-16

Agenda item:	8.1.2

Document for:	Approval

# Introduction

The revised WID for Rel-16 NR-U work includes a number of new band combinations related to standalone NR-U operations. They are listed below as quoted from the revised WID [1]:

| NR-U standalone configuration | Uplink NR-U standalone configuration | supported next level fallback modes (in DL and UL) |
| --- | --- | --- |
| CA_n46E | CA_n46A | (new) DL_n46D_UL_n46A (new) DL_n46C_UL_n46A (new) DL_n46A_UL_n46A |
| CA_n46I | CA_n46A | (new) DL_n46H_UL_n46A (new) DL_n46G_UL_n46A (new) DL_n46B_UL_n46A (new) DL_n46A_UL_n46A |

In this contribution, we describe some of the relevant requirements and also present a related TP in [2].

# Discussion

For the above-mentioned combinations, we present some of our thinking regarding the requirements.

## Channel bandwidths per operating band for CA

# The channel bandwidths for the above above-mentioned stand-alone combinations need to follow the intra-band NR CA BW classes. We place it here for reference:

Source: TS 38.101-1; v16.2.0; Table 5.3A.5-1: NR CA bandwidth classes

| NR CA bandwidth class | Aggregated channel bandwidth | Number of contiguous CC | Fallback group |
| --- | --- | --- | --- |
| A | BWChannel ≤ BWChannel,max | 1 | 1, 2 |
| B | 20 MHz ≤ BWChannel_CA ≤ 100 MHz | 2 | 2 |
| C | 100 MHz < BWChannel_CA ≤ 2 x BWChannel,max | 2 | 1 |
| D | 200 MHz < BWChannel_CA ≤ 3 x BWChannel,max | 3 |  |
| E | 300 MHz < BWChannel_CA ≤ 4 x BWChannel,max | 4 |  |
| F |  | 2 | 2 |
| G | 100 MHz < BWChannel_CA ≤ 150 MHz | 3 |  |
| H | 150 MHz < BWChannel_CA ≤ 200 MHz | 4 |  |
| I | 200 MHz < BWChannel_CA ≤ 250 MHz | 5 |  |
| J | 250 MHz < BWChannel_CA ≤ 300 MHz | 6 |  |
| K | 300 MHz < BWChannel_CA ≤ 350 MHz | 7 |  |
| L | 350 MHz < BWChannel_CA ≤ 400 MHz | 8 |  |
| NOTE 1: BWChannel, max is maximum channel bandwidth supported among all bands in a release NOTE 2: It is mandatory for a UE to be able to fallback to lower order NR CA bandwidth class configuration within a fallback group. It is not mandatory for a UE to be able to fallback to lower order NR CA bandwidth class configuration that belong to a different fallback group |  |  |  |

# We follow the definition of intra-band CA classes and provide the channel bandwidth combinations in the table below for NR-U standalone combinations.

| NR CA configuration / Bandwidth combination set |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| NR CA configuration | Uplink CA configurations | Channel bandwidths for carrier [MHz] | Channel bandwidths for carrier [MHz] | Channel bandwidths for carrier [MHz] | Channel bandwidths for carrier [MHz] | Channel bandwidths for carrier [MHz] | Maximum aggregated bandwidth [MHz] | Bandwidth combination set |
| CA_n46A | CA_n46A | 20, 40, 60, 80, 100 |  |  |  |  | 100 |  |
| CA_n46B | CA_n46A | 20, 40, 60 | 20, 40 |  |  |  | 100 |  |
| CA_n46C | CA_n46A | 60, 80, 100 | 60, 80, 100 |  |  |  | 200 |  |
| CA_n46D | CA_n46A | 60, 80, 100 | 80, 100 | 80, 100 |  |  | 300 |  |
| CA_n46E | CA_n46A | 80, 100 | 80, 100 | 80, 100 | 80, 100 |  | 400 |  |
| CA_n46G | CA_n46A | 40, 60 | 40, 60 | 40 |  |  | 150 |  |
| CA_n46H | CA_n46A | 40, 80 | 40 | 40 | 40 |  | 200 |  |
| CA_n46I | CA_n46A | 40, 60 | 40 | 40 | 40 | 40 | 250 |  |

Please note that, the above combinations do not cover some of the aggregation combinations, such as 5 CCs of 20MHz each to aggregate 100MHz in one go. We have a corresponding contribution in [3] which proposes new intra-band CA bandwidth classes to provide such aggregation opportunities.

## Co-existence studies

There are no co-existence issues for this combination.

## REFSENS

In Rel-13 LAA, only CA was allowed involving band 46, thus we did not have any REFSENS values introduced in the REFSENS table in Section 7 of the spec.

In Rel-16, we will also have standalone NR-U operations. Following LAA specification, REFSENS for 15kHz SCS with 20MHz CBW can be reused as -90dBm. However, the other REFENS numbers need to be investigated.

|  | Operating band / SCS / Channel bandwidth / Duplex-mode |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Operating Band |  | SCS kHz | 5 MHz (dBm) | 10 MHz (dBm) | 15 MHz (dBm) | 20 MHz (dBm) | 25 MHz (dBm) | 30 MHz (dBm) | 40 MHz (dBm) | 50 MHz (dBm) | 60 MHz (dBm) | 70 MHz (dBm) | 80 MHz (dBm) | 90 MHz (dBm) | 100 MHz (dBm) | Duplex Mode |
| n46 |  | 15 |  |  |  | -90.0 |  |  | TBD |  | TBD |  | TBD |  | TBD | TDD |
|  |  | 30 |  |  |  | TBD |  |  | TBD |  | TBD |  | TBD |  | TBD |  |
|  |  | 60 |  |  |  | TBD |  |  | TBD |  | TBD |  | TBD |  | TBD |  |

## A-MPR studies

Since there will be wideband UL operation (UL CBW larger than 20MHz), A-MPR studies are required too. In future,

# Proposal

Corresponding to above requirements, we also present a corresponding TP to TR38.716-01-01 as submitted in [2].

In this contribution, we showed some example cases of NR-U standalone CA cases. After further discussions in RAN4, we can agree to add these combinations in the spec.

# References

- RP-192926, “Revised WID on NR-based Access to Unlicensed Spectrum”, RAN#86, Qualcomm Inc.

- R4-2001712, “TP for TR 38.716-01-01: Introduction of standalone NR-U combinations in Rel-16”, Ericsson

- R4-2001318, “CA bandwidth classes and CA carrier spacing for NR-U”, Ericsson