3GPP TSG-RAN WG4 Meeting #94-e	R4-2001718

Electronic Meeting, 24 February – 6 March 2020

Source:	Nokia, Nokia Shanghai Bell

Title:	Discussion on aligning simulation results of PRACH preamble restricted set type B for Rel-16 LTE HST

Agenda item:	7.13.4.2

Document for:	Discussion

# 1	Introduction

During the RAN4#93 meeting, a CR to TS 36.104 capturing PRACH performance requirements for maximum Doppler frequency of 1944 Hz [1] was agreed. However, the SNR entries are either TBD or in “[]”, which is due to the large difference in the simulation results provided by different companies [2].

This document examines our simulation results [3] for PRACH preamble restricted set type B that were presented at the last meeting to uncover errors or discrepancies in the simulation; and, then followed by our conclusions.

# 2	Discussion

From [1], the PRACH performance requirements for frequency offset at 1944 Hz are shown in Table 1.

Table 1: PRACH missed detection requirements for high speed mode restricted set type B (from [1])

| Number of TX antennas | Number of RX antennas | Propagation conditions and correlation matrix (Annex B) | Frequency offset | SNR [dB] |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  | Burst format 0 | Burst format 1 | Burst format 2 | Burst format 3 |
| 1 | 2 | AWGN | 0 | -14.5 | -14.1 | -16.7 | -16.8 |
|  |  | AWGN | 625 | -12.0 | -11.7 | -13.9 | -13.9 |
|  |  | ETU 70 Low | 270 Hz | -7.3 | -6.9 | -9.1 | -9.2 |
|  |  | AWGN | 1875 Hz | -11.8 | -11.4 | -13.8 | -14.0 |
|  |  | AWGN | 1944 Hz | TBD | [-11.6] | [-14.1] | [-14.3] |
|  | 4 | AWGN | 0 | -17.1 | -16.6 | -19.1 | -19.1 |
|  |  | AWGN | 625 | -14.4 | -14.1 | -16.1 | -16.2 |
|  |  | ETU 70 Low | 270 Hz | -11.8 | -11.3 | -13.5 | -13.4 |
|  |  | AWGN | 1875 Hz | -14.2 | -13.8 | -15.9 | -16.3 |
|  |  | AWGN | 1944 Hz | TBD | [-14.0] | [-16.5] | [-16.6] |
|  | 8 | AWGN | 0 | -19.6 | -19.1 | -21.2 | -21.2 |
|  |  | AWGN | 625 | -16.4 | -16.3 | -18.1 | -18.2 |
|  |  | ETU 70 Low | 270 Hz | -15.3 | -15.1 | -17.1 | -17.5 |
|  |  | AWGN | 1875 Hz | -16.3 | -16.0 | -18.0 | -18.4 |
|  |  | AWGN | 1944 Hz | TBD | [-16.2] | [-18.5] | [-18.7] |

Observation 1: The SNR values for 1944 Hz for PRACH format 1, 2 and 3 are less than those for 1875 Hz, which seem to be odd.

Observation 2: As the frequency offset for 1944 Hz is slightly larger than 1875 Hz, the minimum SNR for achieving the target PRACH detection probability of 99% should be comparable to but not be less than the SNR values for 1875 Hz.

Observation 3: SNR values for the 1875 Hz frequency offset case can be used as a baseline for computing the final SNR based on simulation results of different companies.

Table 2 shows our simulation results [3] when impairments are included.

Table 2: Minimum SNR for achieving the detection probability = 99% (with impairment) (from [3])

| Number of TX antennas | Number of RX antennas | SNR |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  |  | Burst Format 0 | Burst Format 1 | Burst Format 2 | Burst Format 3 |
| 1 | 2 | -11.8 dB | -11.4 dB | -13.8 dB | -14.1 dB |
|  | 4 | -14.2 dB | -13.8 dB | -16.0 dB | -16.4 dB |
|  | 8 | -16.1 dB | -15.9 dB | -18.0 dB | -18.4 dB |

Observation 4: The SNR values in Table 2 are aligned with Observation 2.

Observation 5: The difference between our SNR values in Table 2 and the corresponding SNR for 1875 Hz in Table 1 does not exceed 0.2 dB.

Based on our analysis and the above observations, our simulation results do not show any discrepancies with the SNR values for frequency offset 1875 Hz.

To finalize the PRACH performance requirements for 1944 Hz, SNR values that show discrepancies with the corresponding SNR values for 1875 Hz should be excluded from the averaging computation. This implies that those SNR values that are smaller than the corresponding SNR for 1875 Hz should be excluded. Similarly, SNR values that are far greater than the corresponding SNR for 1875 Hz should also be excluded.

Proposal 1: SNR values that are smaller or far greater than the corresponding SNR for 1875 Hz should be excluded from the averaging computation.

# 3	Conclusions

This document has examined our contribution to the simulation results for PRACH preamble restricted set type B [2] that were presented at the last meeting in an attempt to uncover errors or discrepancies in the simulation results. Based on our analysis, a number of observations can be made concerning the computed SNR values:

Observation 1: The SNR values for 1944 Hz for PRACH format 1, 2 and 3 are less than those for 1875 Hz, which seem to be odd.

Observation 2: As the frequency offset for 1944 Hz is slightly larger than 1875 Hz, the minimum SNR for achieving the target PRACH detection probability of 99% should be comparable to but not be less than the SNR values for 1875 Hz.

Observation 3: SNR values for the 1875 Hz frequency offset case can be used as a baseline for computing the final SNR based on simulation results of different companies.

We have analysed our simulation results, which are aligned with the above observations and do not show any discrepancies.

To finalize the PRACH performance requirements for 1944 Hz, SNR values that show discrepancies with the corresponding SNR values for 1875 Hz should be excluded from the averaging computation. This implies that those SNR values that are smaller than the corresponding SNR for 1875 Hz should be excluded. Similarly, SNR values that are far greater than the corresponding SNR for 1875 Hz should also be excluded.

Proposal 1: SNR values that are smaller or far greater than the corresponding SNR for 1875 Hz should be excluded from the averaging computation.

# References

- R4-1915883, CR to TS 36.104 Introduction of PRACH performance requirements for enhanced HST scenario, NTT DoCoMo, Nokia, Nokia Shanghai Bell

- R4-1913369, Summary of ideal and impairment results for LTE Rel-16 HST BS demodulation requirement, Samsung

- R4-1915210, BS demodulation simulation results of PRACH preamble restricted set type B for Rel-16 LTE HST, Nokia, Nokia Shanghai Bell