3GPP TSG-RAN WG4 Meeting#94-e                	R4-2001721

Electronic Meeting, 24 February – 6 March 2020

Source:	Nokia, Nokia Shanghai Bell

Title:	L1-RSRP measurement accuracy and delay for Rel-16 high speed train

Agenda item:	8.17.1.4

Document for:	Discussion

Introduction

During the RAN4#93 meeting, a way forward on RRM for NR HST [1] was approved. One of the open issues addressed in this paper is L1-RSRP measurement accuracy based on SSB and CSI-RS.

We provide comprehensive link simulation results for L1-RSRP measurement accuracy and conclude with our views.

Discussion

From [1], the following is agreed for beam management:

- L1-RSRP based SSB

  - FFS on whether Rel-15 requirements (measurement delay, measurement accuracy) can be reused for NR HST

- L1-RSRP based on CSI-RS

  - FFS on whether Rel-15 requirements (measurement delay, measurement accuracy) can be reused for NR HST

To evaluate L1-RSRP measurement accuracy and delay based on SSB and CSI-RS, link simulations were carried out using the approved assumptions in [2], which are provided in Appendix A.

SSB based L1-RSRP measurement accuracy

The simulation results for SSB based L1-RSRP measurements at the 5th, 50th and 95th percentiles of the CDF are shown in Tables 1-2 and Tables 3-4 for the synchronous and asynchronous cases, respectively.

Table 1: L1-RSRP measurement accuracy for SINR = -6 dB, AWGN with 2048 Hz offset and synchronous

| SCS | SSB burst periodicity | Number of Samples | 5th percentile | 50th percentile | 95th percentile |
| --- | --- | --- | --- | --- | --- |
| 15 kHz | 5 ms | 1 | -1.1 | 0.91 | 2.43 |
|  |  | 3 | -1.04 | 0.63 | 1.87 |
|  |  | 5 | -0.85 | 0.53 | 1.57 |
|  | 10 ms | 1 | -1.3 | 0.94 | 2.6 |
|  |  | 3 | -0.9 | 0.67 | 1.84 |
|  |  | 5 | -0.74 | 0.6 | 1.53 |
|  | 20 ms | 1 | -1.15 | 0.95 | 2.65 |
|  |  | 3 | -0.75 | 0.68 | 1.85 |
|  |  | 5 | -0.6 | 0.51 | 1.42 |
| 30 kHz | 5 ms | 1 | -1.14 | 0.95 | 2.72 |
|  |  | 3 | -0.57 | 0.52 | 1.52 |
|  |  | 5 | -0.39 | 0.44 | 1.22 |
|  | 10 ms | 1 | -1.09 | 0.96 | 2.71 |
|  |  | 3 | -0.52 | 0.58 | 1.6 |
|  |  | 5 | -0.39 | 0.47 | 1.29 |
|  | 20 ms | 1 | -1.13 | 1.04 | 2.67 |
|  |  | 3 | -0.39 | 0.65 | 1.69 |
|  |  | 5 | -0.25 | 0.5 | 1.37 |

Table 2: L1-RSRP measurement accuracy for SINR = -6 dB, AWGN with 3334 Hz offset and synchronous

| SCS | SSB burst Periodicity | Number of Samples | 5th percentile | 50th percentile | 95th percentile |
| --- | --- | --- | --- | --- | --- |
| 15 kHz | 5 ms | 1 | -1.57 | 0.64 | 2.2 |
|  |  | 3 | -0.92 | 0.25 | 1.31 |
|  |  | 5 | -0.71 | 0.07 | 1.04 |
|  | 10 ms | 1 | -1.45 | 0.67 | 2.18 |
|  |  | 3 | -0.95 | 0.22 | 1.29 |
|  |  | 5 | -0.83 | 0.08 | 0.99 |
|  | 20 ms | 1 | -1.67 | 0.72 | 2.23 |
|  |  | 3 | -0.99 | 0.23 | 1.31 |
|  |  | 5 | -0.82 | 0.12 | 1.01 |
| 30 kHz | 5 ms | 1 | -1.47 | 0.68 | 2.2 |
|  |  | 3 | -0.96 | 0.22 | 1.34 |
|  |  | 5 | -0.81 | 0.15 | 1 |
|  | 10 ms | 1 | -1.43 | 0.51 | 2.07 |
|  |  | 3 | -1.23 | 0.13 | 1.21 |
|  |  | 5 | -1.08 | 0 | 0.9 |
|  | 20 ms | 1 | -1.32 | 0.62 | 2.12 |
|  |  | 3 | -0.9 | 0.26 | 1.17 |
|  |  | 5 | -0.92 | 0.14 | 0.94 |

Table 3: L1-RSRP measurement accuracy for SINR = -6 dB, AWGN with 2048 Hz offset and asynchronous

| SCS | SSB burst Periodicity | Number of Samples | 5th percentile | 50th percentile | 95th percentile |
| --- | --- | --- | --- | --- | --- |
| 15 kHz | 5 ms | 1 | -1.31 | 0.79 | 2.34 |
|  |  | 3 | -0.7 | 0.43 | 1.51 |
|  |  | 5 | -0.51 | 0.29 | 1.14 |
|  | 10 ms | 1 | -1.26 | 0.86 | 2.51 |
|  |  | 3 | -0.63 | 0.49 | 1.47 |
|  |  | 5 | -0.46 | 0.38 | 1.09 |
|  | 20 ms | 1 | -1.1 | 0.84 | 2.47 |
|  |  | 3 | -0.54 | 0.48 | 1.49 |
|  |  | 5 | -0.4 | 0.35 | 1.23 |
| 30 kHz | 5 ms | 1 | -1.03 | 0.82 | 2.22 |
|  |  | 3 | -0.62 | 0.46 | 1.39 |
|  |  | 5 | -0.47 | 0.43 | 1.14 |
|  | 10 ms | 1 | -1.01 | 0.89 | 2.48 |
|  |  | 3 | -0.67 | 0.54 | 1.43 |
|  |  | 5 | -0.52 | 0.44 | 1.16 |
|  | 20 ms | 1 | -1 | 0.93 | 2.51 |
|  |  | 3 | -0.46 | 0.53 | 1.55 |
|  |  | 5 | -0.39 | 0.41 | 1.26 |

Table 4: L1-RSRP measurement accuracy for SINR = -6 dB, AWGN with 3334 Hz offset and asynchronous

| SCS | SSB burst Periodicity | Number of Samples | 5th percentile | 50th percentile | 95th percentile |
| --- | --- | --- | --- | --- | --- |
| 15 kHz | 5 ms | 1 | -0.97 | 0.92 | 2.36 |
|  |  | 3 | -0.55 | 0.53 | 1.58 |
|  |  | 5 | -0.41 | 0.41 | 1.19 |
|  | 10 ms | 1 | -1.27 | 0.78 | 2.42 |
|  |  | 3 | -0.82 | 0.45 | 1.46 |
|  |  | 5 | -0.65 | 0.3 | 1.09 |
|  | 20 ms | 1 | -1.64 | 0.95 | 2.34 |
|  |  | 3 | -0.84 | 0.44 | 1.51 |
|  |  | 5 | -0.58 | 0.34 | 1.17 |
| 30 kHz | 5 ms | 1 | -0.97 | 0.82 | 2.26 |
|  |  | 3 | -0.51 | 0.49 | 1.49 |
|  |  | 5 | -0.49 | 0.39 | 1.13 |
|  | 10 ms | 1 | -1.18 | 0.85 | 2.22 |
|  |  | 3 | -0.81 | 0.47 | 1.42 |
|  |  | 5 | -0.76 | 0.36 | 1.16 |
|  | 20 ms | 1 | -0.96 | 0.92 | 2.34 |
|  |  | 3 | -0.54 | 0.56 | 1.56 |
|  |  | 5 | -0.45 | 0.45 | 1.31 |

SSB burst periodicity and SCS have little or no impact on the L1-RSRP measurement accuracy.

The difference in L1-RSRP measurement accuracy between the 2 frequency offsets is small, where a larger frequency offset improves the measurement accuracy as expected.

No major difference in the measurement accuracy between the synchronous and asynchronous cases

Measurement accuracy for 5 measurement samples is the highest. However, the accuracy is slightly degraded by approximately 0.3 dB for 3 measurement samples as compared with 5 measurement samples.

For HST, 3 measurement samples are recommended because the time-of-outage rate per call is reduced, which is demonstrated through our system simulations.

Based on the above observations, we propose the following:

SSB-based L1-RSRP measurement accuracy for Rel-15 NR can be reused for NR HST and the number of measurement samples is 3.

CSI-RS based L1-RSRP measurement accuracy

The simulation results for CSI-RS based L1-RSRP measurements at the 5th, 50th and 95th percentiles of the CDF are shown in Tables 4-5 and Tables 6-7 for the synchronous and asynchronous cases, respectively.

The CSI-RS density was set to 3 and the number of PRBs was set to 25.

Table 5: L1-RSRP measurement accuracy for SINR = -6 dB, AWGN with 2048 Hz offset and synchronous

| SCS | CSI-RS periodicity | Number of Samples | 5th percentile | 50th percentile | 95th percentile |
| --- | --- | --- | --- | --- | --- |
| 15 kHz | 5 ms | 1 | -1.58 | 1.11 | 3.1 |
|  |  | 3 | -0.98 | 0.69 | 1.96 |
|  |  | 5 | -0.78 | 0.45 | 1.54 |
|  | 10 ms | 1 | -1.56 | 1.04 | 2.94 |
|  |  | 3 | -0.82 | 0.58 | 1.78 |
|  |  | 5 | -0.61 | 0.42 | 1.48 |
|  | 20 ms | 1 | -1.63 | 1.05 | 3.04 |
|  |  | 3 | -0.94 | 0.64 | 1.96 |
|  |  | 5 | -0.78 | 0.51 | 1.63 |
| 30 kHz | 5 ms | 1 | -1.54 | 1.02 | 2.93 |
|  |  | 3 | -0.89 | 0.62 | 1.8 |
|  |  | 5 | -0.7 | 0.47 | 1.59 |
|  | 10 ms | 1 | -1.59 | 1.05 | 3.09 |
|  |  | 3 | -0.88 | 0.58 | 1.97 |
|  |  | 5 | -0.73 | 0.5 | 1.58 |
|  | 20 ms | 1 | -1.44 | 1.2 | 2.93 |
|  |  | 3 | -0.89 | 0.7 | 1.96 |
|  |  | 5 | -0.72 | 0.55 | 1.58 |

Table 6: L1-RSRP measurement accuracy for SINR = -6 dB, AWGN with 3334 Hz offset and synchronous

| SCS | CSI-RS Periodicity | Number of Samples | 5th percentile | 50th percentile | 95th percentile |
| --- | --- | --- | --- | --- | --- |
| 15 kHz | 5 ms | 1 | 1.48 | 1.16 | 3.08 |
|  |  | 3 | -0.92 | 0.68 | 1.96 |
|  |  | 5 | -0.82 | 0.52 | 1.63 |
|  | 10 ms | 1 | -1.6 | 0.96 | 2.93 |
|  |  | 3 | -0.86 | 0.51 | 1.87 |
|  |  | 5 | -0.74 | 0.46 | 1.47 |
|  | 20 ms | 1 | -1.67 | 1.04 | 3.09 |
|  |  | 3 | -0.99 | 0.58 | 2.03 |
|  |  | 5 | -0.79 | 0.44 | 1.56 |
| 30 kHz | 5 ms | 1 | -1.61 | 1.05 | 2.87 |
|  |  | 3 | -0.68 | 0.63 | 1.89 |
|  |  | 5 | -0.54 | 0.48 | 1.51 |
|  | 10 ms | 1 | -1.52 | 1.04 | 2.92 |
|  |  | 3 | -1.02 | 0.59 | 1.87 |
|  |  | 5 | -0.95 | 0.48 | 1.53 |
|  | 20 ms | 1 | -1.52 | 1.17 | 2.89 |
|  |  | 3 | -0.87 | 0.72 | 1.94 |
|  |  | 5 | -0.59 | 0.55 | 1.62 |

Table 7: L1-RSRP measurement accuracy for SINR = -6 dB, AWGN with 2048 Hz offset and asynchronous

| SCS | CSI-RS Periodicity | Number of Samples | 5th percentile | 50th percentile | 95th percentile |
| --- | --- | --- | --- | --- | --- |
| 15 kHz | 5 ms | 1 | -1.27 | 1.17 | 2.99 |
|  |  | 3 | -0.84 | 0.64 | 1.85 |
|  |  | 5 | -0.55 | 0.49 | 1.46 |
|  | 10 ms | 1 | -1.49 | 1.05 | 2.86 |
|  |  | 3 | -0.81 | 0.54 | 1.8 |
|  |  | 5 | -0.75 | 0.4 | 1.36 |
|  | 20 ms | 1 | -1.84 | 0.92 | 3.06 |
|  |  | 3 | -0.92 | 0.53 | 1.92 |
|  |  | 5 | -0.83 | 0.43 | 1.47 |
| 30 kHz | 5 ms | 1 | -1.36 | 1.04 | 2.87 |
|  |  | 3 | -0.95 | 0.6 | 1.85 |
|  |  | 5 | -0.76 | 0.42 | 1.39 |
|  | 10 ms | 1 | -1.63 | 1.1 | 2.97 |
|  |  | 3 | -0.93 | 0.59 | 1.93 |
|  |  | 5 | -0.69 | 0.47 | 1.54 |
|  | 20 ms | 1 | -1.53 | 1.13 | 3.01 |
|  |  | 3 | -0.85 | 0.67 | 1.97 |
|  |  | 5 | -0.7 | 0.54 | 1.57 |

Table 8: L1-RSRP measurement accuracy for SINR = -6 dB, AWGN with 3334 Hz offset and asynchronous

| SCS | CSI-RS Periodicity | Number of Samples | 5th percentile | 50th percentile | 95th percentile |
| --- | --- | --- | --- | --- | --- |
| 15 kHz | 5 ms | 1 | -1.48 | 1.14 | 3 |
|  |  | 3 | -0.93 | 0.7 | 1.87 |
|  |  | 5 | -0.61 | 0.49 | 1.48 |
|  | 10 ms | 1 | -1.54 | 1.04 | 2.96 |
|  |  | 3 | -0.96 | 0.6 | 1.79 |
|  |  | 5 | -0.77 | 0.45 | 1.38 |
|  | 20 ms | 1 | -1.49 | 1.16 | 3.01 |
|  |  | 3 | -0.86 | 0.63 | 1.94 |
|  |  | 5 | -0.69 | 0.55 | 1.54 |
| 30 kHz | 5 ms | 1 | -1.56 | 1.06 | 2.81 |
|  |  | 3 | -1.05 | 0.58 | 1.81 |
|  |  | 5 | -0.8 | 0.49 | 1.33 |
|  | 10 ms | 1 | -1.64 | 1.12 | 3.08 |
|  |  | 3 | -0.99 | 0.68 | 2.03 |
|  |  | 5 | -0.74 | 0.53 | 1.6 |
|  | 20 ms | 1 | -1.61 | 1.16 | 3.06 |
|  |  | 3 | -0.91 | 0.72 | 1.84 |
|  |  | 5 | -0.72 | 0.51 | 1.51 |

The observations are similar to the SSB case.

CSI-RS based L1-RSRP measurement accuracy for Rel-15 NR can be reused for NR HST and the number of measurement samples is 3.

SSB based L1-RSRP measurement delay

From TS 38.133, the SSB based measurement delay TL1-RSRP_Measurement_Period_SSB for FR1 is shown in Table 8.

Table 8: Measurement period TL1-RSRP_Measurement_Period_SSB for FR1

| Configuration | TL1-RSRP_Measurement_Period_SSB (ms) |
| --- | --- |
| non-DRX | max(TReport, ceil(M*P)*TSSB) |
| DRX cycle ≤ 320ms | max(TReport, ceil(1.5*M*P)*max(TDRX,TSSB)) |
| DRX cycle > 320ms | ceil(M*P)*TDRX |
| Note: TSSB = ssb-periodicityServingCell is the periodicity of the SSB-Index configured for L1-RSRP measurement. TDRX is the DRX cycle length. TReport is configured periodicity for reporting. |  |

The simulation results for SSB based measurement delay are based on 90% successful detection rate, which are expressed as a number of SSB period TSSB, are shown in Table 9.

As frequent reporting is required for NR HST, M = 1 in Table 8.

Table 9: L1-RSRP measurement delay for SINR = - 6 dB

| SCS | Case | AWGN with 2408 Hz | AWGN with 3334 Hz |
| --- | --- | --- | --- |
|  |  | SINR = -6 dB | SINR = -6 dB |
| 15 kHz | Synchronous | 1×TSSB | 1×TSSB |
|  | Asynchronous | 1×TSSB | 1×TSSB |
| 30 kHz | Synchronous | 1×TSSB | 1×TSSB |
|  | Asynchronous | 1×TSSB | 1×TSSB |

SSB based L1-RSRP measurement delay is not affected by different SCS and frequency offset values.

In both synchronous and asynchronous cases, 1 SSB period TSSB is sufficient.

SSB-based L1-RSRP measurement delay TL1-RSRP_Measurement_Period_SSB for Rel-15 NR can be reused for NR HST.

CSI-RS based L1-RSRP measurement delay

From TS 38.133, the CSI-RS based L1-RSRP measurement delay TL1-RSRP_Measurement_Period_CSI-RS for FR1 is shown in Table 10.

Table 10: Measurement period TL1-RSRP_Measurement_Period_CSI-RS for FR1

| Configuration | TL1-RSRP_Measurement_Period_CSI-RS (ms) |
| --- | --- |
| non-DRX | max(TReport, ceil(M*P)*TCSI-RS) |
| DRX cycle ≤ 320ms | max(TReport, ceil(1.5*M*P)*max(TDRX,TCSI-RS)) |
| DRX cycle > 320ms | ceil(M*P)*TDRX |
| Note 1: TCSI-RS is the periodicity of CSI-RS configured for L1-RSRP measurement. TDRX is the DRX cycle length. TReport is configured periodicity for reporting. Note 2: the requirements are applicable provided that the CSI-RS resource configured for L1-RSRP measurement is transmitted with Density = 3. |  |

Similar to SSB based L1-RSRP measurement delay, the simulation results are based on 90% successful detection rate, which are expressed as a number of CSI-RS period TCSI-RS, are shown in Table 11.

Table 11: L1-RSRP measurement delay for SINR = - 6 dB

| SCS | Case | AWGN with 2408 Hz | AWGN with 3334 Hz |
| --- | --- | --- | --- |
|  |  | SINR = -6 dB | SINR = -6 dB |
| 15 kHz | Synchronous | 1×TCSI-RS | 1×TCSI-RS |
|  | Asynchronous | 1×TCSI-RS | 1×TCSI-RS |
| 30 kHz | Synchronous | 1×TCSI-RS | 1×TCSI-RS |
|  | Asynchronous | 1×TCSI-RS | 1×TCSI-RS |

The observations are similar to the SSB case.

CSI-RS based L1-RSRP measurement delay TL1-RSRP_Measurement_Period_CSI-RS for Rel-15 NR can be reused for NR HST.

Conclusion

The document has presented link simulation results for SSB and CSI-RS based L1-RSRP measurement accuracy.

From the simulation results and observations, the following proposals are made:

- SSB-based L1-RSRP measurement accuracy for Rel-15 NR can be reused for NR HST and the number of measurement samples is 3.

- CSI-RS based L1-RSRP measurement accuracy for Rel-15 NR can be reused for NR HST and the number of measurement samples is 3.

- SSB-based L1-RSRP measurement delay TL1-RSRP_Measurement_Period_SSB for Rel-15 NR can be reused for NR HST.

- CSI-RS based L1-RSRP measurement delay TL1-RSRP_Measurement_Period_CSI-RS for Rel-15 NR can be reused for NR HST.

References

[1]   R4-1915887, WF on RRM for NR HST, CMCC

[2]    R4-1912842, Link level simulation assumption on measurement accuracy in NR HST, Huawei, HiSilicon

[3]

# A

Table 1: General parameters

| Simulation parameters | Comments/values |
| --- | --- |
| Carrier frequency for Cell 1 and Cell 2 | 3.6 GHz |
| DRX | No |
| BS transmit antennas for SS blocks | 1 tx or single layer transmissions |
| UE receive antennas | 2 rx |
| Data and control channel subcarrier spacing | The same as SS block subcarrier spacing |
| Measurement period (in number of measurement samples) | 5 (other values should be considered such as 1 and 3) |
| - Subcarrier spacing | 15 kHz; 30 kHz |
| - Number of SS blocks per SS burst set, K | 1 up to 8 is possible (a subset can be used) |
| - SS burst periodicity | 5 ms and other values, e.g., 10ms, 20 ms |

Table 2: Cell-specific parameters

| Parameter | Unit | Cell 1 | Cell 2 |
| --- | --- | --- | --- |
| E-UTRA RF Channel number | - | Channel 1 | Channel 1 |
| NR-PSS and NR-SSS sequences | - | To be indicated by companies | To be indicated by companies |
| PBCH and DMRS power offset with respect to NR-PSS and NR-SSS | dB | 0 | 0 |
| Data and control PSD relative to NR-PSS and NR-SSS | dB | 0 | 0 |
| RB Utilization | % | 100 | 100 |
| Data Modulation | - | QPSK | QPSK |
| Slot length | - | 14 symbols | 14 symbols |
| CP Length | - | Normal | Normal |
| Frequency Offset relative to UE frequency reference | Hz | 0 | 0 |
| 1)Relative Delay of 1st Path (synchronous) | µs | 0 | CP/2 |
| 2) Relative Delay of 1st Path (asynchronous): Fixed delay | Ms | 0 | 3 ms |
| Es/IoT | dB | 6.4 | 1.25; FSS: -0.75, 0.25; (SINR: -6dB; FFS: -8dB, -7dB) |
| Propagation conditions | - | AWGN with 2408Hz offset between desired and interfering cells; AWGN with 3334Hz offset between desired and interfering cells |  |