3GPP TSG-RAN WG4 Meeting #94-e                                                R4-2001728

Electronic Meeting, February 24-March 06, 2020

Source:	Ericsson

Title:	NR-U BS RX Simulation Results

Agenda item:	8.1.3.2

Document for:	Discussion

# 1.	Introduction

During RAN4#93, interested companies defined the new NR-U BS RX simulation assumptions needed for input towards new FRC(s) to be defined.  The FRC parameters will differ for NR-U compared to NR due to interleaving aspects for NR-U compared to that of NR.

# 2.	Discussion

As captured in WF [1] the starting assumptions for alignment on results between companies on the SNR for BS RF receiver requirements: dynamic range, REFSENS, and ICS.  In this contribution, the initial results are presented for discussion.

Table 1: REFSENS and ICS assumptions

| Parameter | Value |
| --- | --- |
| Propagation | AWGN |
| Modulation | QPSK |
| Coding rate | 1/3 |
| Symbol type | CP-OFDM |
| HARQ | None |
| # antenna | 1 |
| Channel estimation | Practical |
| Criteria | SNR at 95% throughput |

Table 2: Dynamic Range

| Parameter | Value |
| --- | --- |
| Propagation | AWGN |
| Modulation | 16 QAM |
| Coding rate | 2/3 |
| Symbol type | CP-OFDM |
| HARQ | None |
| # antenna | 1 |
| Channel estimation | Practical |
| Criteria | SNR at 95% throughput |

Observation: Different simulation results given by companies, will need to be treated to determine the resultant value for input to TS 38.104

The values presented thus far based upon simulation results are not so far from each other and there leaves two options:

- Take average between all numbers submitted

- Take the highest SNR level as not to exclude any implementations currently being considered

The results below only consider FRCs and their associated parameters based upon revised interlace parameters considered in companion paper [2]

Table 3: Results summary for REFSENS and ICS

|  | NR-U BS FRC | SNR [dB] |
| --- | --- | --- |
| REFSENS and ICS | G-FR1-A1-10 | -0.45 |
|  | G-FR1-A1-11 | -0.45 |
|  | G-FR1-A1-13 | -0.4 |
|  | G-FR1-A1-14 | -0.45 |
|  | G-FR1-A1-16 | -0.65 |
|  | G-FR1-A1-17 | -0.4 |
|  | G-FR1-A1-19 | -0.65 |
|  | G-FR1-A1-21 | -0.45 |

Table 4: Results summary for Dynamic range

|  | NR-U BS FRC | SNR [dB] |
| --- | --- | --- |
| Dynamic range | G-FR1-A2-7 | 9.15 |
|  | G-FR1-A2-8 | 9.15 |
|  | G-FR1-A2-10 | 9.15 |
|  | G-FR1-A2-11 | 9.15 |
|  | G-FR1-A2-13 | 9 |
|  | G-FR1-A2-14 | 9.15 |
|  | G-FR1-A2-16 | 9 |
|  | G-FR1-A2-18 | 9 |

# 3.	Conclusion

In this contribution, initial results based upon simulation parameters highlighted in [1] but with further updates in [2] were presented.

# 4.	References

[1]	R4-1916162, “WF on NR-U BS RF RX FRC”, ZTE

[2]	R4-2001727, “Update to NR-U FRC definition”, Ericsson, Nokia, Nokia Shanghai Bell