3GPP TSG-RAN WG4 Meeting #94-e	R4-2001752

Electronic Meeting, February 24 – March 06, 2020

Agenda Item:	7.11.3.3

Source:	Ericsson

Title:	Remaining discussions on non-anchor carrier RRM measurements

Document for:	Discussion

# 1	Introduction

RAN4 discussed the non-anchor carrier RRM measurements with limited progress at RAN4#93 meeting. The open issues and the options were summarized as follows [1]:

- UE is allowed to combine the non-anchor NRSRP samples with anchor NRSRP samples - The anchor NRSRP can be obtained by applying the corresponding power offset signaled by the network to the non-anchor NRSRP. - FFS for conditions when UE can combine the non-anchor and anchor NRSRP samples - Any filtered results, either from anchor or non-anchor carrier, can trigger neighbor cell measurement and/or relaxed monitoring abortion. - FFS the feasibility of NRSRP measurements in non-sparse PO case (nB<T/2) - Option 1: disallow RRM measurements in non-anchor carrier. - Option 2: NRSRP measurements are allowed but with no requirements - Option 3: NRSRP measurements are allowed under some cases/conditions which are FFS. - Other options are not excluded.

In this contribution we discuss and provide our view on these open issues.

# 2	Discussion

NRSRP measurements in non-sparse PO

RAN4 has earlier agreed that it is feasible to perform the serving cell NRSRP measurement on the non-anchor carrier under following conditions:

- If configured by the network, serving cell non-anchor carrier measurements are feasible provided that - the relaxed cell monitoring criteria defined in TS 36.306 are met, and - transmit power difference of the signals/channels between anchor- and non-anchor carriers is known to the UE, and - UE is not configured with any positioning measurements and there are sufficient number of NRS subframes available for measurements as being discussed in RAN1

As noted, one of the conditions is that there are sufficient number of NRS subframes available for measurements. According to RAN1 agreements, there are two cases considered for NRS availability when there is no paging, namely sparse and non-sparse presence of POs.

Case 1: nB < T/2: Paging is sparse.

For this case, RAN4 reached an agreement that non-anchor carrier measurements are feasible [2].

Case 2: nB >=T/2: Paging density is high.

RAN1 has decided to have a decimation pattern for this case. At previous meeting, RAN1 reached following agreements for parameters M and N which constitutes the decimation pattern as follows:

Agreement The values of M/N for nB>=T/2 are nB; M; N. 4T; 1; 0. 2T; 2; 0. T; 5; 0. T/2; 10; 0.

- N= number of NRS subframes starting from the subframe containing the PO, i.e., within the NPDCCH search space

- M= number of NRS subframe before the subframe containing the PO.

It is recalled that NRSRP measurements are typically performed using sample of 1 subframe in normal coverage and at least 2 consecutive subframes which are non-coherently averaged over time in enhanced coverage. It is observed that the values of M and N for the different cases of nB may work well for UEs operating in normal coverage. However, non-anchor carrier measurements in enhanced coverage may be problematic for the case when nB=4T since it contains only 1 NRS subframe per radio frame, which may be enough to carry out measurements and fulfil the existing accuracy requirements. Thus, it is observed that the non-anchor carrier NRSRP measurement in enhanced coverage when nB=4T is not feasible and option 3 under the condition that the RRM measurements are allowed only in normal coverage.

- Proposal #1: RRM measurements on non-anchor carrier in non-sparse case is allowed only for measurements in normal coverage, thus option 3 is supported.

Measurement filtering

The second open issue is related to filtering of measurement samples across carriers. Generally, any combining of samples across carriers should be avoided in the same measurement because measured channels may vary a lot and therefore also the measured values. However, in this particular use case, one of the criteria for allowing the UE to stay and perform RRM measurement is relaxed cell monitoring. This implies that UE is expected to be stationary or of low mobility. The propagation channels in such scenarios are typically slowly varying (low doppler), but it does not exclude any fast fading. This means the measurement samples may still vary quite a lot and combining these may not always be a good choice. Taking into account these aspects, the filtering of samples across carriers can be allowed (e.g. using the power offset information signaled by the network) provided that the difference between first and the second samples from anchor and non-anchor carrier respectively that are filtered is less than a threshold X, which can be further discussed.

- Proposal #2: The UE is allowed to combine the non-anchor NRSRP samples with anchor NRSRP samples using the power offset information signaled by the network provided that the difference between the samples is less than X, where X is TBD.

# Summary

In this contribution we have discussed the feasibility of NRSRP measurement when the paging density is high, and we have discussed filtering of measurement samples across anchor and non-anchor carriers for NRSRP measurement. Based on the discussions, we have made following observation and proposal:

- Proposal #1: RRM measurements on non-anchor carrier in non-sparse case is allowed only for measurements in normal coverage, thus option 3 is supported.

- Proposal #2: The UE is allowed to combine the non-anchor NRSRP samples with anchor NRSRP samples using the power offset information signaled by the network provided that the difference between the samples is less than X, where X is TBD.

# References

- R4-1915890, “Way forward on RRM requirements of R16 enhancement for NB-IoT”, Huawei, HiSilicon, November 2019

- R4-1912783 , “Way forward on RRM requirements of R16 enhancement for NB-IoT”, Huawei, HiSilicon, October 2019