3GPP TSG-RAN4 WG4 Meeting # 94-e	R4-2001759

# Electronic meeting, 24 Feb- 06 Mar, 2020

| CR-Form-v12.0 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.101-1 | CR | 0262 | rev | - | Current version: | 16.2.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | CR on FR1 UL CA MPR requirement |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Huawei, HiSilicon |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R4 |  |  |  |  |  |  |  |  |  |
| Work item code: | NR_RF_FR1 |  |  |  |  |  | Date: |  |  | 2019-02-14 |
| Category: | B |  |  |  |  |  | Release: |  |  | Rel-16 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) Rel-12 (Release 12) Rel-13 (Release 13) Rel-14 (Release 14) Rel-15 (Release 15) Rel-16 (Release 16) |  |
| Reason for change: |  | Intra-band contiguous UL CA MPR requirement is added, the MPR values will be updated after the simulation/measurement results among companies is aligned. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Intra-band contiguous UL CA MPR requirement is added. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | RF requirement for Intra-band contiguous UL CA is missing. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 6.2A.2 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | x | Other core specifications |  |  |  | TS/TR … CR … |  |  |
| affected: |  | x |  | Test specifications |  |  |  | TS 38.521-1 |  |  |
| (show related CRs) |  |  | x | O&M Specifications |  |  |  | TS/TR … CR … |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |

| This CR’s revision history: |  |
| --- | --- |

<Start of change>

6.2A.2	UE maximum output power reduction for CA

6.2A.2.1	Void

6.2A.2.2	Void

6.2A.2.3	UE maximum output power reduction for CA

For inter-band carrier aggregation with uplink assigned to two NR bands, the requirements in clause 6.2.2 apply for each uplink component carrier.

For intra-band contiguous carrier aggregation the allowed Maximum Power Reduction (MPR) for the maximum output power in Table 6.2A.1.3-2 with contiguous RB allocation is specified in Table 6.2A.2.3-1 for UE power class 3 CA bandwidth classes B and C. In case the modulation format is different on different component carriers then the MPR is determined by the rules applied to higher order of those modulations.

Table 6.2A.2.3-1: Contiguous RB allocation for Power Class 3

| Modulation |  | MPR for bandwidth class B |  | MPR for bandwidth class C |  |
| --- | --- | --- | --- | --- | --- |
|  |  |  | inner | outer |  | inner | outer |
| DFT-s-OFDM | Pi/2 BPSK |  | TBD | TBD |  | TBD | TBD |
|  | QPSK |  | TBD | TBD |  | TBD | TBD |
|  | 16QAM |  | TBD | TBD |  | TBD | TBD |
|  | 64QAM |  | TBD | TBD |  | TBD | TBD |
|  | 256QAM |  | TBD | TBD |  | TBD | TBD |
| CP-OFDM | QPSK |  | TBD | TBD |  | TBD | TBD |
|  | 16QAM |  | TBD | TBD |  | TBD | TBD |
|  | 64QAM |  | TBD | TBD |  | TBD | TBD |
|  | 256QAM |  | TBD | TBD |  | TBD | TBD |

For CA bandwidth class B, the following parameters are defined to specify valid RB allocation ranges for Outer and Inner RB allocations:

NRB,agg is the maximum number of RBs for a given aggregated channel bandwidth assuming same numerology is the used for UL CCs. NRB_alloc is the total number of simultaneously transmitted RBs in the Aggregated Channel Bandwidth.

RBstart,low=max(1,floor(NRB_alloc/2)),

RBStart,High = NRB,agg – RBStart,Low –NRB_alloc

The RB allocation is an Inner RB allocation if the following conditions are met

RBStart,Low ≤ RBStart ≤ RBStart,High, and NRB_alloc≤Ceil[(1/2NRB,agg) ]

When mixed numerology is used for UL CCs,

NRB_alloc=LCRB1 + LCRB2*(SCS2/SCS1)

NRB,agg=NRB1+NRB2*(SCS2/SCS1)

where SCS2 > SCS1, SCS1 and SCS2 are the SCS for CC1 and CC2 respectively

For CA bandwidth class C, the following parameters are defined to specify valid RB allocation ranges for Outer and Inner RB allocations:

NRB,agg is the maximum number of RBs for a given aggregated channel bandwidth assuming same numerology is the used for UL CCs. NRB_alloc is the total number of simultaneously transmitted RBs in the Aggregated Channel Bandwidth.

RBstart,low=max(1,NRB_alloc),

RBStart,High = NRB,agg – RBStart,Low –NRB_alloc

The RB allocation is an Inner RB allocation if the following conditions are met

RBStart,Low ≤ RBStart ≤ RBStart,High, and NRB_alloc≤Floor[(1/3NRB,agg) ]

When mixed numerology is used for UL CCs,

NRB_alloc=LCRB1 + LCRB2*(SCS2/SCS1)

NRB,agg=NRB1+NRB2*(SCS2/SCS1)

where SCS2 > SCS1, SCS1 and SCS2 are the SCS for CC1 and CC2 respectively

The RB allocation is an Outer RB allocation for all other allocations which are not an Inner RB allocation.

For UE power class 3 intra-band contiguous carrier aggregation bandwidth class C with non-contiguous resource allocation, the allowed Maximum Power Reduction (MPR) for the maximum output power in Table 6.2A.1.3-3 with contiguous RB allocation is specified in Table 6.2A.2.3-2 for non-contiguous RB allocation for UE power class 3 CA bandwidth classes B and C.

Table 6.2A.2.3-2: non-contiguous RB allocation for Power Class 3

| Modulation |  | MPR for bandwidth class B |  | MPR for bandwidth class C |  |
| --- | --- | --- | --- | --- | --- |
|  |  | inner | outer | inner | outer |
| DFT-s-OFDM | Pi/2 BPSK | TBD | TBD | TBD | TBD |
|  | QPSK | TBD | TBD | TBD | TBD |
|  | 16QAM | TBD | TBD | TBD | TBD |
|  | 64QAM | TBD | TBD | TBD | TBD |
|  | 256QAM | TBD | TBD | TBD | TBD |
| CP-OFDM | QPSK | TBD | TBD | TBD | TBD |
|  | 16QAM | TBD | TBD | TBD | TBD |
|  | 64QAM | TBD | TBD | TBD | TBD |
|  | 256QAM | TBD | TBD | TBD | TBD |

For CA bandwidth classes B and C with non-contiguous RB allocation, the following parameters are defined to specify valid RB allocation ranges for Outer and Inner RB allocations:

NRB,agg is the maximum number of RBs for a given aggregated channel bandwidth assuming same numerology is the used for UL CCs.

NRB_alloc is the total number of simultaneously transmitted RBs in the Aggregated Channel Bandwidth and the gap between UL CCs.

RBstart,low=max(1,NRB_alloc),

RBStart,High = NRB,agg – RBStart,Low –NRB_alloc

The RB allocation is an Inner RB allocation if the following conditions are met

RBStart,Low ≤ RBStart ≤ RBStart,High, and NRB_alloc≤Floor[(1/3NRB,agg) ]

When mixed numerology is used for UL CCs,

NRB_alloc=LCRB1 + LCRB2*(SCS2/SCS1)+ ∆fc,gap/SCS1

NRB,agg=NRB1+NRB2*(SCS2/SCS1)

where SCS2 > SCS1, SCS1 and SCS2 are the SCS for CC1 and CC2 respectively, ∆fc,gap is the frequency gap between the RB allocations on UL CCs.

The RB allocation is an Outer RB allocation for all other allocations which are not an Inner RB allocation.

<End of change>