3GPP TSG RAN WG4 Meeting #94-e	R4-2001859

Electronic Meeting, February 24-March 06, 2020

Agenda Item:	8.8.2.1.4

Source: 	Ericsson

Title:	UE Rx-Tx Measurement Report Mapping in NR

Document for:	Discussion

- Introduction

In RAN4#93 meeting the WF on RRM requirements for positioning was approved [1]. There was no agreement on UE Rx-Tx measurement report mapping in NR. But RAN4 received LS from RAN1 regarding RAN1 agreements on the granularity of timing measurements for positioning including the UE Rx-Tx measurement. According to the agreements [2]:

- The reporting granularity for the UE/gNB timing measurements (DL RSTD, the UE Rx-Tx time difference, UL RTOA, gNB Rx-Tx time difference) is defined as , where k is a configuration parameter with a minimum value of at most 0.

  - Note: RAN4 can determine if -1 can be a minimum value

- RAN1 assumes that the details of the reporting granularity and ranges for the UE/gNB timing measurements (DL RTSD, the UE Rx-Tx time difference, UL RTOA, gNB Rx-Tx time difference) will be determined by RAN4, including the potential relation of the parameter k to DL PRS bandwidth.

In this paper we provide analysis various aspects of the UE Rx-Tx measurement report mapping in NR for different cases.

# General principles: report mapping for UE Rx-Tx measurement

The UE can be configured to performe the UE Rx-Tx time difference measurement on PRS and SRS transmitted in the serving cell of the UE e.g. on special cell (SpCell) such as PCell or PSCell.

The UE Rx-Tx time measurement mapping depends on the configured value (k) of the reporting granularity and the configured value of :

Table 1: values in NR [TS 38.133]

| Frequency range and band of cell used for uplink transmission | (Unit: TC) |
| --- | --- |
| FR1 FDD band without LTE-NR coexistence case or FR1 TDD band without LTE-NR coexistence case | 0 |
| FR1 FDD band with LTE-NR coexistence case | 25600 |
| FR1 TDD band with LTE-NR coexistence case | 39936 |
| FR2 | 13792 |

The smallest reportable value of the UE Rx-Tx measurement cannot be smaller than the . Therefore, the report mapping needs to be defined separately for each . In FR2 there is only one. But in FR1 there are three possible values. Therefore, in FR1 is configured by gNB. However, the UE Rx-Tx measurement can be configured by gNB or by the LMF via LPP.

In order to interpret the received UE Rx-Tx measurement value, the positioning node needs to be aware of the NTA offset used by the UE for deriving the reported UE Rx-Tx measurement value. Therefore, the UE should also signal, the used by the UE for deriving the reported value, to the positioning node via LPP.

Furthermore, the UE may have to signal the UE Rx-Tx measurement value to gNB. Since mapping also depends on (k) therefore the UE should also signal the value of k used by the UE for deriving the results to gNB.

The UE can be configured to perform UE Rx-Tx measurement on the serving cell or on serving cell and neighbour cell(s) (multi-RTT positioning). The mapping needs to be specified for both cases.

# Report mapping for single cell UE Rx-Tx measurement

## UE Rx-Tx measurement report mapping in FR1

In FR1 firstly we propose that only larger values of k e.g. 3 or larger are applicable since the sampling rate in FR1 is much lower than in FR2. The largest reported value can be the same as defined for LTE (i.e. 20472*64 Tc for NTA offset =0). However, smaller values of the reported value need to be adjusted based on the values of k. One basic principle is that for k =5 the resolution is the same as in LTE i.e. 2 Ts (128 Tc) for lower part of the range and 8 Ts (512 Tc) for upper part of the range. However, for k < 5 the resolution decreases proportionally. For example for k=4 the resolution is 1 Ts (64 Tc) for lower part of the range and 4 Ts (256 Tc) for upper part of the range. Based on this argument the Rx-Tx mapping for different values of NTA offset and k=3 are provided in the following tables. The mapping for other values of k are provided in annex A.

Table 1 shows report mapping when UE Rx-Tx measurement is configured only in the serving cell in FR1 and for= 0.

Table 1: UE Rx-Tx measurement report mapping when= 0 and k = 3.

| Reported value | Measured quantity value | Unit |
| --- | --- | --- |
| RX-TX_TIME_DIFFERENCE_0000 | TUE Rx-Tx 32 | Tc |
| RX-TX_TIME_DIFFERENCE_0001 | 32 TUE Rx-Tx < 64 | Tc |
| RX-TX_TIME_DIFFERENCE_0002 | 64 TUE Rx-Tx < 96 | Tc |
| … | … | … |
| RX-TX_TIME_DIFFERENCE_8190 | 262080 TUE Rx-Tx < 262112 | Tc |
| RX-TX_TIME_DIFFERENCE_8191 | 262112 TUE Rx-Tx < 262144 | Tc |
| RX-TX_TIME_DIFFERENCE_8192 | 262144 TUE Rx-Tx < 262400 | Tc |
| RX-TX_TIME_DIFFERENCE_8193 | 262400 TUE Rx-Tx < 262528 | Tc |
| … | … | … |
| RX-TX_TIME_DIFFERENCE_16279 | 1309952 TUE Rx-Tx < 1310080 | Tc |
| RX-TX_TIME_DIFFERENCE_16380 | 1310080 TUE Rx-Tx < 1310208 | Tc |
| RX-TX_TIME_DIFFERENCE_16381 | 1310208 TUE Rx-Tx | Tc |

Table 2 shows report mapping when UE Rx-Tx measurement is configured only in the serving cell in FR1 and for= 25600.

Table 2: UE Rx-Tx measurement report mapping when=25600 and k = 3.

| Reported value | Measured quantity value | Unit |
| --- | --- | --- |
| RX-TX_TIME_DIFFERENCE_0000 | TUE Rx-Tx 25632 | Tc |
| RX-TX_TIME_DIFFERENCE_0001 | 25632 TUE Rx-Tx < 25664 | Tc |
| RX-TX_TIME_DIFFERENCE_0002 | 25664 TUE Rx-Tx < 25696 | Tc |
| … | … | … |
| RX-TX_TIME_DIFFERENCE_8190 | 287680 TUE Rx-Tx < 287712 | Tc |
| RX-TX_TIME_DIFFERENCE_8191 | 287712 TUE Rx-Tx < 287744 | Tc |
| RX-TX_TIME_DIFFERENCE_8192 | 287744 TUE Rx-Tx < 287872 | Tc |
| RX-TX_TIME_DIFFERENCE_8193 | 287872 TUE Rx-Tx < 288000 | Tc |
| … | … | … |
| RX-TX_TIME_DIFFERENCE_16279 | 1335552 TUE Rx-Tx < 1335680 | Tc |
| RX-TX_TIME_DIFFERENCE_16380 | 1335680 TUE Rx-Tx < 1335808 | Tc |
| RX-TX_TIME_DIFFERENCE_16381 | 1335808 TUE Rx-Tx | Tc |

Table 3 shows report mapping when UE Rx-Tx measurement is configured only in the serving cell in FR1 and for NTA offset = 39936.

Table 3: UE Rx-Tx measurement report mapping when= 39936 and k = 3.

| Reported value | Measured quantity value | Unit |
| --- | --- | --- |
| RX-TX_TIME_DIFFERENCE_0000 | TUE Rx-Tx 39968 | Tc |
| RX-TX_TIME_DIFFERENCE_0001 | 39968 TUE Rx-Tx < 40000 | Tc |
| RX-TX_TIME_DIFFERENCE_0002 | 40000 TUE Rx-Tx < 40032 | Tc |
| … | … | … |
| RX-TX_TIME_DIFFERENCE_8190 | 302016 TUE Rx-Tx < 302048 | Tc |
| RX-TX_TIME_DIFFERENCE_8191 | 302048 TUE Rx-Tx < 302080 | Tc |
| RX-TX_TIME_DIFFERENCE_8192 | 302080 TUE Rx-Tx < 302336 | Tc |
| RX-TX_TIME_DIFFERENCE_8193 | 302336 TUE Rx-Tx < 302464 | Tc |
| … | … | … |
| RX-TX_TIME_DIFFERENCE_16279 | 1349888 TUE Rx-Tx < 1350016 | Tc |
| RX-TX_TIME_DIFFERENCE_16380 | 1350016 TUE Rx-Tx < 1350144 | Tc |
| RX-TX_TIME_DIFFERENCE_16381 | 1350144 TUE Rx-Tx | Tc |

## Report mapping in FR2

In FR2 the report mappings are defined for all values of k = 0, 1, 2, 3, 4 and 5. The maximum number of reportable values shall be for k=0.

Table 4 shows report mapping when UE Rx-Tx measurement is configured only in the serving cell in FR2 and for= 13792, and for k=0. The mapping for other values of k are provided in annex A.

Table 4: UE Rx-Tx measurement report mapping when= 13792 and k = 0.

| Reported value | Measured quantity value | Unit |
| --- | --- | --- |
| RX-TX_TIME_DIFFERENCE_0000 | TUE Rx-Tx 13796 | Tc |
| RX-TX_TIME_DIFFERENCE_0001 | 13796 TUE Rx-Tx < 13800 | Tc |
| RX-TX_TIME_DIFFERENCE_0002 | 13800 TUE Rx-Tx < 13804 | Tc |
| … | … | … |
| RX-TX_TIME_DIFFERENCE_65534 | 275928 TUE Rx-Tx < 275932 | Tc |
| RX-TX_TIME_DIFFERENCE_65535 | 275932 TUE Rx-Tx < 275952 | Tc |
| RX-TX_TIME_DIFFERENCE_65536 | 275952 TUE Rx-Tx < 275968 | Tc |
| RX-TX_TIME_DIFFERENCE_65537 | 275968 TUE Rx-Tx < 275984 | Tc |
| … | … | … |
| RX-TX_TIME_DIFFERENCE_131038 | 1323968 TUE Rx-Tx < 1323984 | Tc |
| RX-TX_TIME_DIFFERENCE_131039 | 1323984 TUE Rx-Tx < 1324000 | Tc |
| RX-TX_TIME_DIFFERENCE_131040 | 1324000 TUE Rx-Tx | Tc |

# Report mapping for multi-cell UE Rx-Tx measurement

When the UE is configured with multi-RTT positioning by LMF then the UE is required to measure and report the UE Rx-Tx time difference measurement from multiple cells. The measured value of the UE Rx-Tx time difference measurement for the neighbour cell can be larger than the maximum value in the report mapping defined for the serving cell only case (i.e. in section 2). We propose to extend the UE Rx-Tx measurement report mapping for the neigbor cell to the same level as max reportable RSTD value. This means extending the current maximum UE Rx-Tx measured value (e.g. 1310208 for =0) by around 10310*64 Tc or more specifically by 10308*64 Tc to obtain rounded figures in the mapping.

The UE Rx-Tx measurement report mapping when the UE is configured with multi-RTT positioning also depends on the configured value (k) of the reporting granularity and the configured value of .

## UE Rx-Tx measurement report mapping in FR1

The Rx-Tx mapping for different values of k=3 and NTA offset are provided in the following tables. The mapping for other values of k are provided in annex B.

Table 5 shows report mapping when UE Rx-Tx measurement is configured for serving cell and one or more neighbour cells in FR1 and for= 0.

Table 5: UE Rx-Tx measurement report mapping when= 0 and k = 3.

| Reported value | Measured quantity value | Unit |
| --- | --- | --- |
| RX-TX_TIME_DIFFERENCE_0000 | TUE Rx-Tx 32 | Tc |
| RX-TX_TIME_DIFFERENCE_0001 | 32 TUE Rx-Tx < 64 | Tc |
| RX-TX_TIME_DIFFERENCE_0002 | 64 TUE Rx-Tx < 96 | Tc |
| … | … | … |
| RX-TX_TIME_DIFFERENCE_8190 | 262080 TUE Rx-Tx < 262112 | Tc |
| RX-TX_TIME_DIFFERENCE_8191 | 262112 TUE Rx-Tx < 262144 | Tc |
| RX-TX_TIME_DIFFERENCE_8192 | 262144 TUE Rx-Tx < 262400 | Tc |
| RX-TX_TIME_DIFFERENCE_8193 | 262400 TUE Rx-Tx < 262528 | Tc |
| … | … | … |
| RX-TX_TIME_DIFFERENCE_21532 | 1969664 TUE Rx-Tx < 1969792 | Tc |
| RX-TX_TIME_DIFFERENCE_21533 | 1969792 TUE Rx-Tx < 1969920 | Tc |
| RX-TX_TIME_DIFFERENCE_21534 | 1969920 TUE Rx-Tx | Tc |

Table 6 shows report mapping when UE Rx-Tx measurement is configured for serving cell and one or more neighbour cells in FR1 and for= 25600.

Table 6: UE Rx-Tx measurement report mapping when= 25600 and k = 3.

| Reported value | Measured quantity value | Unit |
| --- | --- | --- |
| RX-TX_TIME_DIFFERENCE_0000 | TUE Rx-Tx 25632 | Tc |
| RX-TX_TIME_DIFFERENCE_0001 | 25632 TUE Rx-Tx < 25664 | Tc |
| RX-TX_TIME_DIFFERENCE_0002 | 25664 TUE Rx-Tx < 25696 | Tc |
| … | … | … |
| RX-TX_TIME_DIFFERENCE_8190 | 287680 TUE Rx-Tx < 287712 | Tc |
| RX-TX_TIME_DIFFERENCE_8191 | 287712 TUE Rx-Tx < 287744 | Tc |
| RX-TX_TIME_DIFFERENCE_8192 | 287744 TUE Rx-Tx < 287872 | Tc |
| RX-TX_TIME_DIFFERENCE_8193 | 287872 TUE Rx-Tx < 288000 | Tc |
| … | … | … |
| RX-TX_TIME_DIFFERENCE_21532 | 1995264 TUE Rx-Tx < 1995392 | Tc |
| RX-TX_TIME_DIFFERENCE_21533 | 1995392 TUE Rx-Tx < 1995520 | Tc |
| RX-TX_TIME_DIFFERENCE_21534 | 1995520 TUE Rx-Tx | Tc |

Table 7 shows report mapping when UE Rx-Tx measurement is configured for serving cell and one or more neighbour cells and for= 39936.

Table 7: UE Rx-Tx measurement report mapping when= 39936 and k = 3.

| Reported value | Measured quantity value | Unit |
| --- | --- | --- |
| RX-TX_TIME_DIFFERENCE_0000 | TUE Rx-Tx 39968 | Tc |
| RX-TX_TIME_DIFFERENCE_0001 | 39968 TUE Rx-Tx < 40000 | Tc |
| RX-TX_TIME_DIFFERENCE_0002 | 40000 TUE Rx-Tx < 40032 | Tc |
| … | … | … |
| RX-TX_TIME_DIFFERENCE_8190 | 302016 TUE Rx-Tx < 302048 | Tc |
| RX-TX_TIME_DIFFERENCE_8191 | 302048 TUE Rx-Tx < 302080 | Tc |
| RX-TX_TIME_DIFFERENCE_8192 | 302080 TUE Rx-Tx < 302336 | Tc |
| RX-TX_TIME_DIFFERENCE_8193 | 302336 TUE Rx-Tx < 302464 | Tc |
| … | … | … |
| RX-TX_TIME_DIFFERENCE_21532 | 2009600 TUE Rx-Tx < 2009728 | Tc |
| RX-TX_TIME_DIFFERENCE_21533 | 2009728 TUE Rx-Tx < 2009856 | Tc |
| RX-TX_TIME_DIFFERENCE_21534 | 2009856 TUE Rx-Tx | Tc |

## Report mapping in FR2

Table 8 shows report mapping when UE Rx-Tx measurement is configured only in the serving cell and one or more neighbour cells in FR2 and for= 13792. The UE Rx-Tx mapping for other values of k are provided in the following tables.

Table 8: UE Rx-Tx measurement report mapping when= 13792 and k = 0.

| Reported value | Measured quantity value | Unit |
| --- | --- | --- |
| RX-TX_TIME_DIFFERENCE_0000 | TUE Rx-Tx 13796 | Tc |
| RX-TX_TIME_DIFFERENCE_0001 | 13796 TUE Rx-Tx < 13800 | Tc |
| RX-TX_TIME_DIFFERENCE_0002 | 13800 TUE Rx-Tx < 13804 | Tc |
| … | … | … |
| RX-TX_TIME_DIFFERENCE_65534 | 275928 TUE Rx-Tx < 275932 | Tc |
| RX-TX_TIME_DIFFERENCE_65535 | 275932 TUE Rx-Tx < 275952 | Tc |
| RX-TX_TIME_DIFFERENCE_65536 | 275952 TUE Rx-Tx < 275968 | Tc |
| RX-TX_TIME_DIFFERENCE_65537 | 275968 TUE Rx-Tx < 275984 | Tc |
| … | … | … |
| RX-TX_TIME_DIFFERENCE_172270 | 1983680 TUE Rx-Tx < 1983696 | Tc |
| RX-TX_TIME_DIFFERENCE_172271 | 1983696 TUE Rx-Tx < 1983712 | Tc |
| RX-TX_TIME_DIFFERENCE_172272 | 1983712 TUE Rx-Tx | Tc |

# Summary

In this paper we have analysed and proposed UE Rx-Tx measurement report mappings in NR. Following are the main observation and proposals:

- Observation#1: In FR1 the serving gNB can configure one of the three possible values ofwhich impacts the UE Rx-Tx measurement report mapping.

- Observation#2: The positioning node needs to be aware of the NTA offset used by the UE for deriving the reported UE Rx-Tx measurement value.

- Observation#3: gNB needs to be aware of the granularity parameter (k) used by the UE for deriving the reported UE Rx-Tx measurement value when reporting the UE Rx-Tx measurement value to gNB.

- Proposal #1: In FR1 the UE shall signal the information about the NTA offset used for deriving the reported value when reporting UE Rx-Tx measurement value to the positioning node.

- Proposal #2: The UE shall signal the granularity parameter (k) used for deriving the reported value when reporting UE Rx-Tx measurement value to gNB.

- Observation#4: Smaller value of granularity parameter, k, leads to finer resolution compared to larger value of k. In FR1 the sampling rate is much lower than in FR2.

- Proposal #3: The basic principle is that the resolution shall not be larger than 128Tc and 1024Tc for lower and upper parts of the report mapping (i.e. at least as good as LTE UE Rx-Tx report mapping). The resolution proportionally increases with k as shown below:

| k | Resolution (Tc) of UE Rx-Tx measurement report mapping |  |
| --- | --- | --- |
|  | Lower part of mapping | Upper part of mapping |
| 0 | 4 | 16 |
| 1 | 8 | 32 |
| 2 | 16 | 64 |
| 3 | 32 | 128 |
| 4 | 64 | 256 |
| 5 | 128 | 512 |

- Proposal #4: In FR1 the UE Rx-Tx reporting mapping is defined for the granularity parameter, k ≥ 3.

- Proposal #5: In FR2 the UE Rx-Tx reporting mapping is defined for all values of the granularity parameter, k ≥ 0.

- Proposal #6: In FR1, for UE Rx-Tx measured only on the serving cell, the largest number of reportable values (for k =3) shall be 8189.

- Proposal #7: In FR2, for UE Rx-Tx measured only on the serving cell, the largest number of reportable values (for k =0) shall be 131041.

- Observation#5: UE configured with multi-RTT positioning is required to report the UE Rx-Tx measurement from multiple cells with measured value which can be larger than the maximum value in the UE Rx-Tx report mapping being defined for only serving cell measurement.

- Proposal #8: For multi-RTT positioning the UE Rx-Tx report mapping is obtained by extending the upper bound of the UE Rx-Tx report mapping defined for serving cell by 10308*64 Tc resulting in the maximum range corresponding to RSTD in LTE.

- Proposal #9: In FR1, for UE Rx-Tx measured on serving and neighbor cells (multi-RTT), the largest number of reportable values for (k =3) shall be 21535.

- Proposal #10: In FR2, for UE Rx-Tx measured on serving and neighbor cells (multi-RTT), the largest number of reportable values for (k =0) shall be 172273.

# References

- R4-1915854, Way forward on NR Positioning RRM, Ericsson.

- R1-1913522, RAN1 LS on agreements related to NR Positioning.

# Annex A: UE Rx-Tx mapping for measurement only on serving cell

Table A-1: UE Rx-Tx measurement report mapping when= 0 and k = 4.

| Reported value | Measured quantity value | Unit |
| --- | --- | --- |
| RX-TX_TIME_DIFFERENCE_0000 | TUE Rx-Tx 64 | Tc |
| RX-TX_TIME_DIFFERENCE_0001 | 64 TUE Rx-Tx < 128 | Tc |
| RX-TX_TIME_DIFFERENCE_0002 | 128 TUE Rx-Tx < 192 | Tc |
| … | … | … |
| RX-TX_TIME_DIFFERENCE_4094 | 262016 TUE Rx-Tx < 262080 | Tc |
| RX-TX_TIME_DIFFERENCE_4095 | 262080 TUE Rx-Tx < 262144 | Tc |
| RX-TX_TIME_DIFFERENCE_4096 | 262144 TUE Rx-Tx < 262400 | Tc |
| RX-TX_TIME_DIFFERENCE_4097 | 262400 TUE Rx-Tx < 262656 | Tc |
| … | … | … |
| RX-TX_TIME_DIFFERENCE_8188 | 1309696 TUE Rx-Tx < 13109952 | Tc |
| RX-TX_TIME_DIFFERENCE_8089 | 13109952 TUE Rx-Tx < 1310208 | Tc |
| RX-TX_TIME_DIFFERENCE_8190 | 1310208 TUE Rx-Tx | Tc |

Table A-2: UE Rx-Tx measurement report mapping when= 0 and k = 5.

| Reported value | Measured quantity value | Unit |
| --- | --- | --- |
| RX-TX_TIME_DIFFERENCE_0000 | TUE Rx-Tx 128 | Tc |
| RX-TX_TIME_DIFFERENCE_0001 | 128 TUE Rx-Tx < 256 | Tc |
| RX-TX_TIME_DIFFERENCE_0002 | 256 TUE Rx-Tx < 384 | Tc |
| … | … | … |
| RX-TX_TIME_DIFFERENCE_2044 | 261888 TUE Rx-Tx < 262016 | Tc |
| RX-TX_TIME_DIFFERENCE_2045 | 262016 TUE Rx-Tx < 262144 | Tc |
| RX-TX_TIME_DIFFERENCE_2046 | 262144 TUE Rx-Tx < 262656 | Tc |
| RX-TX_TIME_DIFFERENCE_2047 | 262656 TUE Rx-Tx < 263168 | Tc |
| … | … | … |
| RX-TX_TIME_DIFFERENCE_4093 | 1309184 TUE Rx-Tx < 1309696 | Tc |
| RX-TX_TIME_DIFFERENCE_4094 | 1309696 TUE Rx-Tx < 1310208 | Tc |
| RX-TX_TIME_DIFFERENCE_4095 | 1310208 TUE Rx-Tx | Tc |

Table A-3: UE Rx-Tx measurement report mapping when= 25600 and k = 4.

| Reported value | Measured quantity value | Unit |
| --- | --- | --- |
| RX-TX_TIME_DIFFERENCE_0000 | TUE Rx-Tx 25664 | Tc |
| RX-TX_TIME_DIFFERENCE_0001 | 25664 TUE Rx-Tx < 25728 | Tc |
| RX-TX_TIME_DIFFERENCE_0002 | 25728 TUE Rx-Tx < 25792 | Tc |
| … | … | … |
| RX-TX_TIME_DIFFERENCE_4094 | 287616 TUE Rx-Tx < 287680 | Tc |
| RX-TX_TIME_DIFFERENCE_4095 | 287680 TUE Rx-Tx < 287744 | Tc |
| RX-TX_TIME_DIFFERENCE_4096 | 287744 TUE Rx-Tx < 288000 | Tc |
| RX-TX_TIME_DIFFERENCE_4097 | 288000 TUE Rx-Tx < 288256 | Tc |
| … | … | … |
| RX-TX_TIME_DIFFERENCE_8188 | 1335296 TUE Rx-Tx < 11335552 | Tc |
| RX-TX_TIME_DIFFERENCE_8089 | 1335552 TUE Rx-Tx < 1335808 | Tc |
| RX-TX_TIME_DIFFERENCE_8190 | 1335808 TUE Rx-Tx | Tc |

Table A-4: UE Rx-Tx measurement report mapping when= 25600 and k = 5.

| Reported value | Measured quantity value | Unit |
| --- | --- | --- |
| RX-TX_TIME_DIFFERENCE_0000 | TUE Rx-Tx 25728 | Tc |
| RX-TX_TIME_DIFFERENCE_0001 | 25728 TUE Rx-Tx < 25856 | Tc |
| RX-TX_TIME_DIFFERENCE_0002 | 25856 TUE Rx-Tx < 25984 | Tc |
| … | … | … |
| RX-TX_TIME_DIFFERENCE_2044 | 287488 TUE Rx-Tx < 287616 | Tc |
| RX-TX_TIME_DIFFERENCE_2045 | 287616 TUE Rx-Tx < 287744 | Tc |
| RX-TX_TIME_DIFFERENCE_2046 | 287744 TUE Rx-Tx < 288256 | Tc |
| RX-TX_TIME_DIFFERENCE_2047 | 288256 TUE Rx-Tx < 288768 | Tc |
| … | … | … |
| RX-TX_TIME_DIFFERENCE_4093 | 1334784 TUE Rx-Tx < 1335296 | Tc |
| RX-TX_TIME_DIFFERENCE_4094 | 1335296 TUE Rx-Tx < 1335808 | Tc |
| RX-TX_TIME_DIFFERENCE_4095 | 1335808 TUE Rx-Tx | Tc |

Table A-5: UE Rx-Tx measurement report mapping when= 3996 and k = 4.

| Reported value | Measured quantity value | Unit |
| --- | --- | --- |
| RX-TX_TIME_DIFFERENCE_0000 | TUE Rx-Tx 40000 | Tc |
| RX-TX_TIME_DIFFERENCE_0001 | 40000 TUE Rx-Tx < 4064 | Tc |
| RX-TX_TIME_DIFFERENCE_0002 | 4064 TUE Rx-Tx < 4128 | Tc |
| … | … | … |
| RX-TX_TIME_DIFFERENCE_4094 | 302016 TUE Rx-Tx < 302080 | Tc |
| RX-TX_TIME_DIFFERENCE_4095 | 302080 TUE Rx-Tx < 302336 | Tc |
| RX-TX_TIME_DIFFERENCE_4096 | 302336 TUE Rx-Tx < 302592 | Tc |
| RX-TX_TIME_DIFFERENCE_4097 | 302592 TUE Rx-Tx < 302848 | Tc |
| … | … | … |
| RX-TX_TIME_DIFFERENCE_8187 | 1349632 TUE Rx-Tx < 1349888 | Tc |
| RX-TX_TIME_DIFFERENCE_8188 | 1349888 TUE Rx-Tx < 1350144 | Tc |
| RX-TX_TIME_DIFFERENCE_8189 | 1350144 TUE Rx-Tx | Tc |

Table A-6: UE Rx-Tx measurement report mapping when= 3996 and k = 5.

| Reported value | Measured quantity value | Unit |
| --- | --- | --- |
| RX-TX_TIME_DIFFERENCE_0000 | TUE Rx-Tx 40064 | Tc |
| RX-TX_TIME_DIFFERENCE_0001 | 40064 TUE Rx-Tx < 40192 | Tc |
| RX-TX_TIME_DIFFERENCE_0002 | 40192 TUE Rx-Tx < 40320 | Tc |
| … | … | … |
| RX-TX_TIME_DIFFERENCE_2044 | 301824 TUE Rx-Tx < 301952 | Tc |
| RX-TX_TIME_DIFFERENCE_2045 | 301952 TUE Rx-Tx < 302080 | Tc |
| RX-TX_TIME_DIFFERENCE_2046 | 302080 TUE Rx-Tx < 302592 | Tc |
| RX-TX_TIME_DIFFERENCE_2047 | 302592 TUE Rx-Tx < 303616 | Tc |
| … | … | … |
| RX-TX_TIME_DIFFERENCE_4093 | 1349120 TUE Rx-Tx < 1349632 | Tc |
| RX-TX_TIME_DIFFERENCE_4094 | 1349632 TUE Rx-Tx < 1350144 | Tc |
| RX-TX_TIME_DIFFERENCE_4095 | 1350144 TUE Rx-Tx | Tc |

Table A-7: UE Rx-Tx measurement report mapping when= 13792 and k = 1.

| Reported value | Measured quantity value | Unit |
| --- | --- | --- |
| RX-TX_TIME_DIFFERENCE_0000 | TUE Rx-Tx 13800 | Tc |
| RX-TX_TIME_DIFFERENCE_0001 | 13800 TUE Rx-Tx < 13808 | Tc |
| RX-TX_TIME_DIFFERENCE_0002 | 13808 TUE Rx-Tx < 13816 | Tc |
| … | … | … |
| RX-TX_TIME_DIFFERENCE_32766 | 275920 TUE Rx-Tx < 275928 | Tc |
| RX-TX_TIME_DIFFERENCE_32767 | 275928 TUE Rx-Tx < 275936 | Tc |
| RX-TX_TIME_DIFFERENCE_32768 | 275936 TUE Rx-Tx < 275968 | Tc |
| RX-TX_TIME_DIFFERENCE_32769 | 275968 TUE Rx-Tx < 276000 | Tc |
| … | … | … |
| RX-TX_TIME_DIFFERENCE_65518 | 1323936 TUE Rx-Tx < 1323968 | Tc |
| RX-TX_TIME_DIFFERENCE_65519 | 1323968 TUE Rx-Tx < 1324000 | Tc |
| RX-TX_TIME_DIFFERENCE_65520 | 1324000 TUE Rx-Tx | Tc |

Table A-8: UE Rx-Tx measurement report mapping when= 13792 and k = 2.

| Reported value | Measured quantity value | Unit |
| --- | --- | --- |
| RX-TX_TIME_DIFFERENCE_0000 | TUE Rx-Tx 13808 | Tc |
| RX-TX_TIME_DIFFERENCE_0001 | 13808 TUE Rx-Tx < 13824 | Tc |
| RX-TX_TIME_DIFFERENCE_0002 | 13824 TUE Rx-Tx < 13840 | Tc |
| … | … | … |
| RX-TX_TIME_DIFFERENCE_16382 | 275904 TUE Rx-Tx < 275920 | Tc |
| RX-TX_TIME_DIFFERENCE_16383 | 275920 TUE Rx-Tx < 275936 | Tc |
| RX-TX_TIME_DIFFERENCE_16384 | 275936 TUE Rx-Tx < 276000 | Tc |
| RX-TX_TIME_DIFFERENCE_16385 | 276064 TUE Rx-Tx < 276128 | Tc |
| … | … | … |
| RX-TX_TIME_DIFFERENCE_32758 | 1323936 TUE Rx-Tx < 1323968 | Tc |
| RX-TX_TIME_DIFFERENCE_32759 | 1323968 TUE Rx-Tx < 1324000 | Tc |
| RX-TX_TIME_DIFFERENCE_32760 | 1324000 TUE Rx-Tx | Tc |

# Annex B: UE Rx-Tx report mapping for multi-RTT positioning

Table B-1: UE Rx-Tx measurement report mapping when= 0 and k = 4.

| Reported value | Measured quantity value | Unit |
| --- | --- | --- |
| RX-TX_TIME_DIFFERENCE_0000 | TUE Rx-Tx 64 | Tc |
| RX-TX_TIME_DIFFERENCE_0001 | 64 TUE Rx-Tx < 128 | Tc |
| RX-TX_TIME_DIFFERENCE_0002 | 128 TUE Rx-Tx < 192 | Tc |
| … | … | … |
| RX-TX_TIME_DIFFERENCE_4094 | 262016 TUE Rx-Tx < 262080 | Tc |
| RX-TX_TIME_DIFFERENCE_4095 | 262080 TUE Rx-Tx < 262144 | Tc |
| RX-TX_TIME_DIFFERENCE_4096 | 262144 TUE Rx-Tx < 262400 | Tc |
| RX-TX_TIME_DIFFERENCE_4097 | 262400 TUE Rx-Tx < 262656 | Tc |
| … | … | … |
| RX-TX_TIME_DIFFERENCE_10765 | 1969408 TUE Rx-Tx < 1969664 | Tc |
| RX-TX_TIME_DIFFERENCE_10766 | 1969664 TUE Rx-Tx < 1969920 | Tc |
| RX-TX_TIME_DIFFERENCE_10767 | 1969920 TUE Rx-Tx | Tc |

Table B-2: UE Rx-Tx measurement report mapping when= 25600 and k = 4.

| Reported value | Measured quantity value | Unit |
| --- | --- | --- |
| RX-TX_TIME_DIFFERENCE_0000 | TUE Rx-Tx 25664 | Tc |
| RX-TX_TIME_DIFFERENCE_0001 | 25664 TUE Rx-Tx < 25728 | Tc |
| RX-TX_TIME_DIFFERENCE_0002 | 25728 TUE Rx-Tx < 25792 | Tc |
| … | … | … |
| RX-TX_TIME_DIFFERENCE_4094 | 287616 TUE Rx-Tx < 287680 | Tc |
| RX-TX_TIME_DIFFERENCE_4095 | 287680 TUE Rx-Tx < 287744 | Tc |
| RX-TX_TIME_DIFFERENCE_4096 | 287744 TUE Rx-Tx < 288000 | Tc |
| RX-TX_TIME_DIFFERENCE_4097 | 288000 TUE Rx-Tx < 288256 | Tc |
| … | … | … |
| RX-TX_TIME_DIFFERENCE_10765 | 1995008 TUE Rx-Tx < 1995264 | Tc |
| RX-TX_TIME_DIFFERENCE_10766 | 1995264 TUE Rx-Tx < 1995520 | Tc |
| RX-TX_TIME_DIFFERENCE_10767 | 1995520 TUE Rx-Tx | Tc |

Table B-3: UE Rx-Tx measurement report mapping when= 3996 and k = 4.

| Reported value | Measured quantity value | Unit |
| --- | --- | --- |
| RX-TX_TIME_DIFFERENCE_0000 | TUE Rx-Tx 40000 | Tc |
| RX-TX_TIME_DIFFERENCE_0001 | 40000 TUE Rx-Tx < 4064 | Tc |
| RX-TX_TIME_DIFFERENCE_0002 | 4064 TUE Rx-Tx < 4128 | Tc |
| … | … | … |
| RX-TX_TIME_DIFFERENCE_4094 | 302016 TUE Rx-Tx < 302080 | Tc |
| RX-TX_TIME_DIFFERENCE_4095 | 302080 TUE Rx-Tx < 302336 | Tc |
| RX-TX_TIME_DIFFERENCE_4096 | 302336 TUE Rx-Tx < 302592 | Tc |
| RX-TX_TIME_DIFFERENCE_4097 | 302592 TUE Rx-Tx < 302848 | Tc |
| … | … | … |
| RX-TX_TIME_DIFFERENCE_10765 | 2009344 TUE Rx-Tx < 2009600 | Tc |
| RX-TX_TIME_DIFFERENCE_10766 | 2009600 TUE Rx-Tx < 2009856 | Tc |
| RX-TX_TIME_DIFFERENCE_10767 | 2009856 TUE Rx-Tx | Tc |

Table B-4: UE Rx-Tx measurement report mapping when= 13792 and k = 1.

| Reported value | Measured quantity value | Unit |
| --- | --- | --- |
| RX-TX_TIME_DIFFERENCE_0000 | TUE Rx-Tx 13800 | Tc |
| RX-TX_TIME_DIFFERENCE_0001 | 13800 TUE Rx-Tx < 13808 | Tc |
| RX-TX_TIME_DIFFERENCE_0002 | 13808 TUE Rx-Tx < 13816 | Tc |
| … | … | … |
| RX-TX_TIME_DIFFERENCE_32766 | 275920 TUE Rx-Tx < 275928 | Tc |
| RX-TX_TIME_DIFFERENCE_32767 | 275928 TUE Rx-Tx < 275936 | Tc |
| RX-TX_TIME_DIFFERENCE_32768 | 275936 TUE Rx-Tx < 275968 | Tc |
| RX-TX_TIME_DIFFERENCE_32769 | 275968 TUE Rx-Tx < 276000 | Tc |
| … | … | … |
| RX-TX_TIME_DIFFERENCE_86134 | 1983648 TUE Rx-Tx < 1983680 | Tc |
| RX-TX_TIME_DIFFERENCE_86135 | 1983680 TUE Rx-Tx < 1983712 | Tc |
| RX-TX_TIME_DIFFERENCE_86136 | 1983712 TUE Rx-Tx | Tc |