3GPP TSG RAN WG4 Meeting #94-e	R4-2001889

Electronic Meeting, February 24-March 06, 2020

Agenda Item:	8.5.4.2.1

Source: 	Ericsson

Title:	TP to TS 38.174: IAB output power

Document for:	Approval

- Introduction

After RAN4#92bis the IAB TS [1] is email approved. Section 2 provides text proposal on 9 IAB output power for the IAB TS 38.174 (Integrated access and backhaul radio transmission and reception).

# Text Proposal

--------------------------------------------------Start of TP------------------------------------------------------

## 9.3	IAB output power

### 9.3.1 General

OTA IAB output power is declared separately for IAB MT and DU as the TRP radiated requirement, with the output power accuracy requirement defined at the RIB during the transmitter ON period. TRP does not change with beamforming settings as long as the beam peak direction is within the OTA peak directions set. Thus the TRP accuracy requirement must be met for any beamforming setting for which the beam peak direction is within the OTA peak directions set.

The IAB rated carrier TRP output power for IAB DU/IAB MT type 1-O shall be within limits as specified in table 9.3.1-1.

Table 9.3.1-1: IAB MT/DU rated carrier TRP output power limits for IAB MT/DU of IAB type 1-O

| IAB class | Prated,c,TRP |
| --- | --- |
| Wide Area IAB | (note) |
| Medium Range IAB | ≤ + 47 dBm |
| Local Area IAB | ≤ + 33 dBm |
| NOTE: There is no upper limit for the Prated,c,TRP of the Wide Area IAB. |  |

There is no upper limit for the rated carrier TRP output power of IAB DU/MT type 2-O.

Despite the general requirements for the IAB output power described in subclauses 9.3.2 – 9.3.3, additional regional requirements might be applicable.

NOTE:	In certain regions, power limits corresponding to IAB classes may apply for IAB DU/MT of IAB type 2-O.

### 9.3.2 Minimum requirement for IAB DU of IAB type 1-O

In normal conditions, the IAB DU of IAB type 1-O maximum carrier TRP output power, Pmax,c,TRP measured at the RIB shall remain within ±2 dB of the rated carrier TRP output power Prated,c,TRP, as declared by the manufacturer.

Normal conditions are defined in TS 38.141-1, annex B [6].

### 9.3.3 Minimum requirement for IAB DU of IAB type 2-O

In normal conditions, the IAB DU of IAB type 2-O maximum carrier TRP output power, Pmax,c,TRP measured at the RIB shall remain within ±3 dB of the rated carrier TRP output power Prated,c,TRP, as declared by the manufacturer.

Normal conditions are defined in TS 38.141-2, annex B [6].

### 9.3.4 Minimum requirement for IAB MT of IAB type 1-O

In normal conditions, the IAB MT type 1-O maximum carrier TRP output power, PIAB MT,max,c,TRP measured at the RIB shall remain within ±2 dB of the rated carrier TRP output power Prated,c,TRP, as declared by the manufacturer.

Normal conditions are defined in TS 38.141-1, annex B [6].

### 9.3.5 Minimum requirement for IAB MT of IAB type 2-O

In normal conditions, the IAB MT type 2-O maximum carrier TRP output power, PIAB MT,max,c,TRP measured at the RIB shall remain within ±3 dB of the rated carrier TRP output power Prated,c,TRP, as declared by the manufacturer.

Normal conditions are defined in TS 38.141-2, annex B [6].

--------------------------------------------------End of TP------------------------------------------------------

- References

- R4-1916000, “Updated IAB TS”, Qualcommn Inc.