3GPP TSG RAN WG4 Meeting #94-e	R4-2001890

Electronic Meeting, February 24-March 06, 2020

Agenda Item:	8.5.4.2.1

Source: 	Ericsson

Title:	TP to TS 38.174: Carrier leakage

Document for:	Approval

- Introduction

After RAN4#92bis the IAB TS [1] is email approved. Section 2 provides text proposal on 9 Carrier leakage for the IAB TS 38.174 (Integrated access and backhaul radio transmission and reception).

# Text Proposal

--------------------------------------------------Start of TP------------------------------------------------------

#### 9.6.2.3	Carrier leakage

##### 9.6.2.3.1	General

Carrier leakage is an additive sinusoid waveform. The carrier leakage requirement is defined for each component carrier. The measurement interval is one slot in the time domain. The relative carrier leakage power is a power ratio of the additive sinusoid waveform to the power in the modulated waveform.

The requirement is verified with the test metric of Carrier Leakage (Link=TX beam peak direction, Meas=Link angle).

##### 9.6.2.3.2	Carrier leakage for IAB MT of IAB type 2-O

When carrier leakage is contained inside the spectrum confined within the configured UL and DL CCs, the relative carrier leakage power shall not exceed the values specified in Table 9.6.2.3.2-1.

Table 9.6.2.3.2-1: Minimum requirements for relative carrier leakage power

| Parameters | Relative Limit (dBc) |
| --- | --- |
| PIAB MT,max,c,TRP | -25 |

--------------------------------------------------End of TP------------------------------------------------------

- References

- R4-1916000, “Updated IAB TS”, Qualcommn Inc.