3GPP TSG RAN WG4 Meeting #94-e	R4-2001891

Electronic Meeting, February 24-March 06, 2020

Agenda Item:	8.5.4.2.1

Source: 	Ericsson

Title:	TP to TS 38.174: EVM equalizer spectrum flatness

Document for:	Approval

- Introduction

After RAN4#92bis the IAB TS [1] is email approved. Section 2 provides text proposal on 9 EVM equalizer spectrum flatness for the IAB TS 38.174 (Integrated access and backhaul radio transmission and reception).

# Text Proposal

--------------------------------------------------Start of TP------------------------------------------------------

#### 9.6.2.4	EVM equalizer spectrum flatness

##### 9.6.2.4.1	Minimum Requirement for IAB MT of IAB type 2-O

The EVM measurement process (as described in Annex F) entails generation of a zero-forcing equalizer. The EVM equalizer spectrum flatness is defined in terms of the maximum peak-to-peak ripple of the equalizer coefficients (dB) across the allocated uplink block. The basic measurement interval is the same as for EVM.

The peak-to-peak variation of the EVM equalizer coefficients contained within the frequency range of the uplink allocation shall not exceed the maximum ripple specified in Table 9.6.2.4.1-1for normal conditions. For uplink allocations contained within both Range 1 and Range 2, the coefficients evaluated within each of these frequency ranges shall meet the corresponding ripple requirement and the following additional requirements: the relative difference between the maximum coefficient in Range 1 and the minimum coefficient in Range 2 (Table 9.6.2.4.1-1) must not be larger than 7 dB, and the relative difference between the maximum coefficient in Range 2 and the minimum coefficient in Range 1 must not be larger than 8 dB (see Figure 9.6.2.4.1-1).

The requirement is verified with the test metric of EVM SF (Link=TX beam peak direction, Meas=Link angle).

Table 9.6.2.4.1-1: Minimum requirements for EVM equalizer spectrum flatness (normal conditions)

| Frequency range | Maximum ripple (dB) |
| --- | --- |
| FUL_Meas – FUL_Low ≥ X MHz and FUL_High – FUL_Meas ≥ X MHz (Range 1) | 6 (p-p) |
| FUL_Meas – FUL_Low < X MHz or FUL_High – FUL_Meas < X MHz (Range 2) | 9 (p-p) |
| NOTE 1: FUL_Meas refers to the sub-carrier frequency for which the equalizer coefficient is evaluated NOTE 2: FUL_Low and FUL_High refer to channel edges NOTE 3: X, in MHz, is equal to 20% of the CC bandwidth |  |

Table 6.4.2.4-2: (Void)

Figure 9.6.2.4.1-1: The limits for EVM equalizer spectral flatness with the maximum allowed variation of the coefficients indicated under normal conditions

--------------------------------------------------End of TP------------------------------------------------------

- References

- R4-1916000, “Updated IAB TS”, Qualcommn Inc.