3GPP TSG-RAN WG4 Meeting #94-e                     	R4-2001919

# Electronic Meeting, 24 Feb. – 6 Mar., 2020

Source:	Nokia, Nokia Shanghai Bell

Title:	On gNB measurement requirements for NR positioning

Agenda item:	8.8.2.2

Document for:	Discussion and Agreement

Introduction

At RAN4 #93, the discussion on gNB measurement requirements for NR positioning [1] was continued and the next steps identified as depicted in the WF on NR Positioning RRM [2]:

Agreements gNB measurements - Option 1: Define gNB measurement accuracy requirements for gNB measurement types in the Perf part - Rx-Tx timing difference - UL SRS-RSRP measurements - Option 2: Define gNB measurement accuracy requirements for gNB measurement types in the Perf part - Rx-Tx timing difference - UL SRS-RSRP measurements - AoA / ZoA measurements - UL RTOA measurements - Option 3: Do not define gNB requirements

The above three options were discussed. It is noted that option 1 had the broadest support at RAN4 #93. This contribution discusses the above options and states our preference to go with option 1.

Discussion

The discussion in this section refers to the three options listed in section 1.

Necessity of requirements

NR positioning is a feature introduced into Rel-16. As for existing positioning techniques, network support is considered optional for this feature, and thus, gNB requirements for NR positioning are optional. The following proposal is made.

gNB requirements for NR positioning are optional.

Requirements types

RAN1 has defining following UL measurement types for NR positioning [3]:

- UL SRS-RSRP,

- AoA and ZoA,

- UL RTOA,

- gNB Rx time and gNB Tx time (for determination of gNB Rx-Tx time difference).

RAN1 has also defined the granularity for reporting of these measurements which were liaised to RAN4 in [4].

Table 1 summarizes these RAN1 agreements.

| UL measurement type | Reporting granularity | RAN4 tasks |
| --- | --- | --- |
| AoA and ZoA | 0.1 degree | details of the reporting granularity and reporting ranges |
| gNB Rx-Tx time difference | with k≥0 | study if k=-1 is feasible details of the reporting granularity and ranges |
| RTOA | with k≥0 | study if k=-1 is feasible details of the reporting granularity and ranges |
| SRS-RSRP | To be defined by RAN4 | range of values and mapping |

Table 1: RAN1 agreements on measurement reporting parameters for NR positioning [4].

AoA and ZoA

For AoA and ZoA, RAN1 has defined this measurement type under UL Angle of Arrival in TS 38.215 (see Annex), and the reporting granularity has been fixed to 0.1 degree for both. This allows accurate reporting for the received signal in azimuth and zenith angles.

Considering defined deployment scenarios in TR 38.855 [5], table 1 shows the horizontal uncertainty equivalent to the reporting granularity for AoA for a UE at cell edge and a comparison to regulatory requirements and commercial performance targets, respectively, as specified in [5].

| Deployment scenarios | Indoor office | UMi street canyon | UMa macro cell |
| --- | --- | --- | --- |
| ISD | 20 m | 200 m | 500 m |
| Cell range | 10 m | 115.5 m | 288.7 m |
| Horizontal uncertainty corresponding to 0,1 degree for AoA for UE at cell edge | 0.017 m | 0.2 m | 0.5 m |
| Regulatory requirements - horizontal positioning error for 80% of UEs | 50 m | 50 m | 50 m |
| Commercial performance targets - horizontal positioning error for 80% of UEs | 3 m | 10 m | 10 m |

Table 2: Equivalent horizontal uncertainty for UE at cell edge for AoA reporting granularity of 0.1 degree versus specified regulatory requirements and commercial performance targets [5].

It is observed that in all deployment scenarios, considered in [5], the equivalent horizontal uncertainty for UE at cell edge is much lower than the specified regulatory requirement or commercial performance target, respectively (e.g. 50 / 20 times lower for UMa, 250 / 50 times lower for UMi, and even 2940 / 176 times lower for indoor office).  Thus, the reporting granularity is much finer than what is required for the position estimate. Assuming combination of angular measurements from different TRPs and gNB sites for deriving the position estimate at the Location Server, and combination with other measurement types, performance requirements for AoA and ZoA cannot be defined on basis of the reporting granularity and moreover are difficult to assess, as seen in Table 2 above, due to the deployment impact.

Moreover, as already discussed in [6], AoA and ZoA measurement types may be differently processed, i.e. using RX beamforming with super-resolution per beam or best beam selection in order to identify the first path. No agreement on the processing method for AoA was reached in RAN1, however. Based on the above, performance requirements in terms of measurement accuracy for angular measurements cannot be determined straight forward, and thus, it is proposed to define merely the details of the reporting granularity and reporting ranges, as proposed by RAN1.

For instance, a variable reporting granularity should be investigated for AoA and ZoA (i.e. lower granularity around antenna main direction, higher granularity beyond). Reporting ranges for AoA and ZoA depend on deployment items (i.e. sectorization and antenna radiation patterns); related assumptions should be defined by RAN4. Thus, the following proposal is made.

No AoA and ZoA minimum accuracy requirements will be defined. RAN4 will investigate a variable reporting granularity and the reporting range for both measurement types.

gNB Rx-Tx time difference

For gNB Rx-Tx time difference TgNB-RX – TgNB-TX, the reporting granularity has been fixed to  by RAN1. This allows accurate reporting for this measurement type, based on the two measurements gNB Rx time TgNB-RX and gNB Tx time TgNB-TX as defined by RAN1 in TS 38.215 (see Annex):

TgNB-RX is the positioning node received timing of uplink subframe #i containing SRS associated with UE, defined by the first detected path in time.

TgNB-TX is the positioning node transmit timing of downlink subframe #j that is closest in time to the subframe #i received from the UE.

Multiple SRS resources for positioning can be used to determine the start of one subframe containing SRS.

It is noted that a reporting granularity of Tc corresponds to a time step of 0.509 ns and a radial uncertainty of 0.15 m. Thus, this reporting granularity exceeds by far what is required for NR positioning in Rel-16 (see Table 2). Hence it is anticipated that the minimum measurement accuracy will be relaxed versus this reporting uncertainty, both for UE and gNB. It is also noted that for achieving the target position accuracy, a combination of measurement types (and different positioning methods) is likely to happen at the Location Server.

The gNB Rx-Tx time difference, used for the RTT positioning method, is reported along UE Rx-Tx time difference to the Location Server. Interpretation at the LMF is only sensible, if both measurements have a similar accuracy, i.e. satisfy a given accuracy requirement. Therefore, the specification of RRM performance requirements in terms of minimum accuracy requirements for UE Rx-Tx time and gNB Rx-Tx time will be required. It is noted, that this measurement type could be used in multi-vendor networks. Given that accuracy requirements for gNB Rx-Tx time difference are aligned to those for UE Rx-Tx time difference, it is expected that there is limited impact to gNB implementation. In our view, performance requirements for gNB measurements should be limited to fixed antenna beams for simplification purposes. In addition, requirements should be derived from the serving cell scenario, i.e. serving cell Ec/No is to be configured, and no requirements for gNB Rx-Tx time difference are foreseen to be defined for neighbor cell UEs, as this would only add testing effort. In our view only semi-persistent SRS transmission mode needs to be considered.

In addition, RAN1 asks RAN4 to study if a lower reporting granularity of Tc/2 is possible (this corresponds to a time step of 0.254 ns and a radial uncertainty of 0.076 m), which may reduce the number of positioning nodes, used for the position estimate. Furthermore, as multiple SRS resources can be used and SRS bandwidth will vary like for LTE, RAN4 needs to investigate suitable SRS configurations for defining minimum accuracy requirements.

Thus, the following proposal is made:

a) gNB minimum accuracy requirements for gNB Rx-Tx time difference will be defined for the serving cell UE scenario and are aligned to those for UE Rx-TX time difference.

- b) Performance requirements for gNB measurements are specified for fixed antenna beams only.

- c) RAN4 should study if a higher reporting resolution of Tc/2 is possible for gNB Rx-Tx time difference.

- d) RAN4 needs to investigate suitable SRS configurations in terms of number of SRS resources and SRS bandwidth, assuming semi-persistent SRS transmission mode, for defining minimum accuracy requirements.

RTOA

For UL RTOA, the reporting granularity has been fixed to  by RAN1. This allows accurate reporting for this measurement type as defined by RAN1 in TS 38.215 (see Annex):

[The UL Relative Time of Arrival (TUL-RTOA) is the beginning of subframe i containing SRS received in positioning node j, relative to the configurable reference time.]

Multiple SRS resources for positioning can be used to determine the beginning of one subframe containing SRS received at a positioning node.

For UL RTOA, used for UL-TDOA positioning method, we observe that the relative time of arrival is defined in TS 38.215 in a similar way as the gNB Rx-Tx time difference, as both

- are based on the same reference signal (UL-SRS, one or multiple resources),

- use the same measurement reference point,

- can be made identical, if the reference time for UL RTOA corresponds to the positioning node transmit timing of downlink subframe #j.

For the RRM performance requirement definition, it is proposed that RTOA is derived in single gNB – single UE configuration with interferer and receiver noise being modelled by AWGN, as for LTE in TS 36.111. Dependency on number of SRS resources (SRS transmissions) and SRS bandwidth will need to be considered. In our view only semi-persistent SRS transmission mode needs to be considered.

Since the gNB Rx-Tx time difference and RTOA refer to the same timing measurements and can be made identical, as depicted above, the following proposal is made:

No separate RTOA minimum accuracy requirements will be defined. The same requirement as for gNB Rx-Tx time difference applies.

UL SRS-RSRP

For UL SRS-RSRP, RAN1 has defined this measurement type in TS 38.215 (see Annex) but has not proposed a reporting granularity. Thus, the reporting granularity should be investigated by RAN4, either 1 dB or a value lower should be investigated (1 dB was e.g. mentioned in [7]). In particular for LOS propagation conditions, a reduction below 1 dB is beneficial (1 dB lower RSRP corresponds to 12% larger distance).

Since all positioning methods may use this measurement type and the UL reference signal is the same as for RTOA and gNB Rx-Tx time difference measurements, specifying minimum accuracy requirements is expected to yield an improvement for all positioning methods. As for gNB Rx-Tx time difference measurement type, requirements should be derived for UL SRS-RSRP from the serving cell scenario, i.e. serving cell Ec/No is to be configured, and no requirements for UL SRS-RSRP are foreseen to be defined for neighbor cell UEs, as this would only add testing effort. In our view only semi-persistent SRS transmission mode needs to be considered.

Thus, the following proposal is made:

a) gNB minimum accuracy requirements for UL SRS-RSRP will be defined for the serving cell UE scenario.

- b) Performance requirements for gNB measurements are specified for fixed antenna beams only.

- c) RAN4 should study if a lower reporting granularity than 1 dB is feasible.

- d) RAN4 needs to investigate suitable (i.e. UE supported) SRS configurations in terms of number of SRS resources and SRS bandwidth, assuming semi-persistent SRS transmission mode, for defining minimum accuracy requirements for UL SRS-RSRP.

Applicability to frequency bands

Achievable performance depends on the measurement bandwidth, thus gNB measurement performance for FR1 and FR2 is expected to be different.

Different performance requirements for NR positioning for BS types operating in FR1 and FR2 are envisaged.

Conclusion

This contribution has discussed gNB measurement requirements for NR positioning. It is proposed to proceed along option 1 and agree the following proposals:

- gNB requirements for NR positioning are optional.

No AoA and ZoA minimum accuracy requirements will be specified. RAN4 will investigate a variable reporting granularity and the reporting range for both measurement types.

a) gNB minimum accuracy requirements for gNB Rx-Tx time difference will be defined for the serving cell UE scenario and are aligned to those for UE Rx-TX time difference.

- b) Performance requirements for gNB measurements are specified for fixed antenna beams only.

- c) RAN4 should study if a higher reporting resolution of Tc/2 is possible for gNB Rx-Tx time difference.

- d) RAN4 needs to investigate suitable SRS configurations in terms of number of SRS resources and SRS bandwidth, assuming semi-persistent SRS transmission mode, for defining minimum accuracy requirements.

No separate RTOA minimum accuracy requirements will be specified. The same requirement as for gNB Rx-Tx time difference applies.

a) gNB minimum accuracy requirements for UL SRS-RSRP will be defined for the serving cell UE scenario.

- b) Performance requirements for gNB measurements are specified for fixed antenna beams only.

- c) RAN4 should study if a lower reporting granularity than 1 dB is feasible.

- d) RAN4 needs to investigate suitable (i.e. UE supported) SRS configurations in terms of number of SRS resources and SRS bandwidth, assuming semi-persistent SRS transmission mode, for defining minimum accuracy requirements.

Different performance requirements for NR positioning for BS types operating in FR1 and FR2 are envisaged.

References

- RP-191156, "Revised WID: NR Positioning Support", source: Intel, Ericsson

- R4-1915854, "Way forward on NR Positioning RRM", source: Ericsson

- R1-1909796, "LS on DL/UL Reference Signals and Measurements for NR Positioning", source: RAN1

- R1-1913522, "LS on agreements related to NR Positioning", source: RAN1

- 3GPP TR 38.355, "Study on NR positioning support (Rel-16)"

- R4-1915171, "On gNB measurement requirements for NR positioning ", source: Nokia, Nokia Shanghai Bell

- R4-1913463, "Considerations on gNB measurement requirements for NR positioning", source: Intel Corporation

## Annex: Extract from 3GPP TS 38.215, V16.0.1

## 5.2	NG-RAN measurement abilities

The structure of the table defining a NG-RAN measurement quantity is shown below.

| Column field | Comment |
| --- | --- |
| Definition | Contains the definition of the measurement. |

### 5.2.1	SSS transmit power

| Definition | SSS transmit power is determined as the linear average over the power contributions (in [W]) of the resource elements that carry secondary synchronization signals within the secondary synchronization signal (SSS) bandwidth. For downlink reference signal transmit power determination the secondary synchronization signal according TS 38.211 [4] can be used. For frequency range 1, the reference point for the downlink reference signal power measurement shall be the transmit antenna connector. |
| --- | --- |

### 5.2.2	UL Relative Time of Arrival (TUL-RTOA)

| Definition | [The UL Relative Time of Arrival (TUL-RTOA) is the beginning of subframe i containing SRS received in positioning node j, relative to the configurable reference time.] Multiple SRS resources for positioning can be used to determine the beginning of one subframe containing SRS received at a positioning node. The reference point for TUL-RTOA shall be: - for type 1-C base station TS 38.104 [9]: the Rx antenna connector, - for type 1-O or 2-O base station TS 38.104 [9]: the Rx antenna, - for type 1-H base station TS 38.104 [9]: the Rx Transceiver Array Boundary connector. |
| --- | --- |

### 5.2.3	gNB Rx – Tx time difference

| Definition | The gNB Rx – Tx time difference is defined as TgNB-RX – TgNB-TX Where: TgNB-RX is the positioning node received timing of uplink subframe #i containing SRS associated with UE, defined by the first detected path in time. TgNB-TX is the positioning node transmit timing of downlink subframe #j that is closest in time to the subframe #i received from the UE. Multiple SRS resources for positioning can be used to determine the start of one subframe containing SRS. The reference point for TgNB-RX shall be: - for type 1-C base station TS 38.104 [9]: the Rx antenna connector, - for type 1-O or 2-O base station TS 38.104 [9]: the Rx antenna, - for type 1-H base station TS 38.104 [9]: the Rx Transceiver Array Boundary connector. The reference point for TgNB-TX shall be: - for type 1-C base station TS 38.104 [9]: the Tx antenna connector, - for type 1-O or 2-O base station TS 38.104 [9]: the Tx antenna, - for type 1-H base station TS 38.104 [9]: the Tx Transceiver Array Boundary connector. |
| --- | --- |

### 5.2.4	UL Angle of Arrival (UL AoA)

| Definition | UL Angle of Arrival (UL AoA) is defined as the estimated azimuth angle and vertical angle of a UE with respect to a reference direction, wherein the reference direction is defined: - In the global coordinate system (GCS), wherein estimated azimuth angle is measured relative to geographical North and is positive in a counter-clockwise direction and estimated vertical angle is measured relative to zenith and positive to horizontal direction - In the local coordinate system (LCS), wherein estimated azimuth angle is measured relative to x-axis of LCS and positive in a counter-clockwise direction and estimated vertical angle is measured relatize to z-axis of LCS and positive to x-y plane direction. The bearing, downtilt and slant angles of LCS are defined according to TS 38.901 [14]. The UL AoA is determined at the gNB antenna for an UL channel corresponding to this UE. |
| --- | --- |

### 5.2.5	UL SRS reference signal received power (UL SRS-RSRP)

| Definition | UL SRS reference signal received power (UL SRS-RSRP) is defined as linear average of the power contributions (in [W]) of the resource elements carrying sounding reference signals (SRS). UL SRS-RSRP shall be measured over the configured resource elements within the considered measurement frequency bandwidth in the configured measurement time occasions. For frequency range 1, the reference point for the UL SRS-RSRP shall be the antenna connector of the gNB. For frequency range 2, UL SRS-RSRP shall be measured based on the combined signal from antenna elements corresponding to a given receiver branch. For frequency range 1 and 2, if receiver diversity is in use by the gNB, the reported UL SRS-RSRP value shall not be lower than the corresponding UL SRS-RSRP of any of the individual receiver branches. |
| --- | --- |