3GPP TSG-RAN WG4 Meeting #94-e 	R4-2001934

# Electronic Meeting, February 24-March 06, 2020

| CR-Form-v12.0 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.133 | CR | 0566 | rev | - | Current version: | 16.2.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | X | Radio Access Network | X | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Introduction of RLM requirements for NR-U |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Ericsson |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R4 |  |  |  |  |  |  |  |  |  |
| Work item code: | NR_unlic-Core |  |  |  |  |  | Date: |  |  | 2020-02-14 |
| Category: | B |  |  |  |  |  | Release: |  |  | Rel-16 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) Rel-12 (Release 12) Rel-13 (Release 13) Rel-14 (Release 14) Rel-15 (Release 15) Rel-16 (Release 16) |  |
| Reason for change: |  | There are no RLM requirements for NR-U |  |  |  |  |  |  |  |  |
| Summary of change: |  | Adding RLM requirements for NR-U |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | No RLM requirements for NR-U |  |  |  |  |  |  |  |  |
| Clauses affected: |  | New sections added: 8.1A, 8.1A.1 8.1A.2, 8.1A.2.1, 8.1A.2.2, 8.1A.2.3 8.1A.3, 8.1A.3.1, 8.1A.3.2, 8.1A.3.3 8.1A.4, 8.1A.5, 8.1A.6 8.1A.7, 8.1A.7.1, 8.1A.7.1 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  |  | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  |  | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  |  | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

## 8.1A	Radio Link Monitoring with CCA on Target Frequency

### 8.1A.1	Introduction

The requirements in clause 8.1A apply for radio link monitoring on a carrier frequency with CCA for cells:

-	PCell in SA NR operation mode,

-	PSCell in EN-DC operation mode.

The UE shall monitor the downlink radio link quality based on the reference signal configured as RLM-RS resource(s) in order to detect the downlink radio link quality of the PCell and PSCell as specified in TS 38.213 [3]. The configured RLM-RS resources can be all SSBs, or all CSI-RSs, or a mix of SSBs and CSI-RSs. UE is not required to perform RLM outside the active DL BWP.

On each RLM-RS resource, the UE shall estimate the downlink radio link quality and compare it to the thresholds Qout,CCA and Qin,CCA for the purpose of monitoring downlink radio link quality of the cell.

The threshold Qout,CCA is defined as the level at which the downlink radio link cannot be reliably received and shall correspond to the out-of-sync block error rate (BLERout,CCA) as defined in Table 8.1A.1-1. For SSB based radio link monitoring, Qout_SSB,CCA is derived based on the hypothetical PDCCH transmission parameters listed in Table 8.1A.2.1-1. For CSI-RS based radio link monitoring, Qout_CSI-RS,CCA is derived based on the hypothetical PDCCH transmission parameters listed in Table 8.1A.3.1-1.

The threshold Qin,CCA is defined as the level at which the downlink radio link quality can be received with significantly higher reliability than at Qout,CCA and shall correspond to the in-sync block error rate (BLERin) as defined in Table 8.1A.1-1. For SSB based radio link monitoring, Qin_SSB,CCA is derived based on the hypothetical PDCCH transmission parameters listed in Table 8.1A.2.1-2. For CSI-RS based radio link monitoring, Qin_CSI-RS,CCA is derived based on the hypothetical PDCCH transmission parameters listed in Table 8.1A.3.1-2.

The out-of-sync block error rate (BLERout,CCA) and in-sync block error rate (BLERin,CCA) are determined from the network configuration via parameter rlmInSyncOutOfSyncThreshold signalled by higher layers. When UE is not configured with rlmInSyncOutOfSyncThreshold from the network, UE determines out-of-sync and in-sync block error rates from Configuration #0 in Table 8.1A.1-1 as default. All requirements in clause 8.1A are applicable for BLER Configuration #0 in Table 8.1A.1-1.

Table 8.1A.1-1: Out-of-sync and in-sync block error rates

| Configuration | BLERout,CCA | BLERin,CCA |
| --- | --- | --- |
| 0 | 10% | 2% |

UE shall be able to monitor up to NRLM RLM-RS resources of the same or different types in each corresponding carrier frequency range, depending on a maximum number Lmax of candidate SSBs per half frame according to TS 38.213 [3], where NRLM is specified in Table 8.1A.1-2, and meet the requirements as specified in clause 8.1A. UE is not required to meet the requirements in clause 8.1A if RLM-RS is not configured and no TCI state for PDCCH is activated.

Table 8.1A.1-2: Maximum number of RLM-RS resources NRLM

| Lmax | Maximum number of RLM-RS resources, NRLM |
| --- | --- |
| 8 | 4 |

### 8.1A.2	Requirements for SSB Based Radio Link Monitoring

#### 8.1A.2.1	Introduction

The requirements in this section apply for each SSB based RLM-RS resource configured for PCell or PSCell, provided that the SSB configured for RLM are actually configured to be transmitted within UE active DL BWP during the entire evaluation period specified in clause 8.1A.2.2 but occasionally may be not transmitted due to CCA operation.

Table 8.1A.2.1-1: PDCCH transmission parameters for out-of-sync evaluation

| Attribute | Value for BLER Configuration #0 |
| --- | --- |
| DCI format | TBD |
| Number of control OFDM symbols | TBD |
| Aggregation level (CCE) | TBD |
| Ratio of hypothetical PDCCH RE energy to average SSS RE energy | TBD |
| Ratio of hypothetical PDCCH DMRS energy to average SSS RE energy | TBD |
| Bandwidth (PRBs) | TBD |
| Sub-carrier spacing (kHz) | SCS of the active DL BWP |
| DMRS precoder granularity | TBD |
| REG bundle size | TBD |
| CP length | Normal |
| Mapping from REG to CCE | TBD |

Table 8.1A.2.1-2: PDCCH transmission parameters for in-sync evaluation

| Attribute | Value for BLER Configuration #0 |
| --- | --- |
| DCI payload size | TBD |
| Number of control OFDM symbols | TBD |
| Aggregation level (CCE) | TBD |
| Ratio of hypothetical PDCCH RE energy to average SSS RE energy | TBD |
| Ratio of hypothetical PDCCH DMRS energy to average SSS RE energy | TBD |
| Bandwidth (PRBs) | TBD |
| Sub-carrier spacing (kHz) | SCS of the active DL BWP |
| DMRS precoder granularity | TBD |
| REG bundle size | TBD |
| CP length | Normal |
| Mapping from REG to CCE | TBD |

#### 8.1A.2.2	Minimum Requirement

UE shall be able to evaluate whether the downlink radio link quality on the configured RLM-RS resource estimated over the last TEvaluate_out_SSB,CCA [ms] period becomes worse than the threshold Qout_SSB,CCA within TEvaluate_out_SSB,CCA [ms] evaluation period.

UE shall be able to evaluate whether the downlink radio link quality on the configured RLM-RS resource estimated over the last TEvaluate_in_SSB,CCA [ms] period becomes better than the threshold Qin_SSB,CCA within TEvaluate_in_SSB,CCA [ms] evaluation period.

TEvaluate_out_SSB,CCA and TEvaluate_in_SSB,CCA are defined in Table 8.1A.2.2-1, where

-	P=11-TSSBMRGP, when in the monitored cell there are measurement gaps configured for intra-frequency, inter-frequency or inter-RAT measurements, and these measurement gaps are overlapping with some but not all occasions of the SSB; and

-	P=1 when in the monitored cell there are no measurement gaps overlapping with any occasion of the SSB.

If the high layer in TS 38.331 [2] signaling of smtc2 is present, TSMTCperiod follows smtc2; Otherwise TSMTCperiod follows smtc1.

Longer evaluation period would be expected if the combination of RLM-RS, SMTC occasion, and measurement gap configurations does not meet previous conditions.

Table 8.1A.2.2-1: Evaluation period TEvaluate_out_SSB,CCA and TEvaluate_in_SSB,CCA

| Configuration | TEvaluate_out_SSB,CCA (ms) | TEvaluate_in_SSB,CCA (ms) |
| --- | --- | --- |
| no DRX | Max(200, Ceil((10+Lout)*P)*TSSB) | Max(100, Ceil((5+Lin)*P)*TSSB) |
| DRX cycle≤320 | Max(200, Ceil(1.5*(10+Lout)*P)*Max(TDRX,TSSB)) | Max(100, Ceil(1.5*(5+Lin)*P)*Max(TDRX,TSSB)) |
| DRX cycle>320 | Ceil((10+Lout)*P)*TDRX | Ceil((5+Lin)*P)*TDRX |
| NOTE 1: TSSB is the periodicity of the SSB configured for RLM. TDRX is the DRX cycle length. NOTE 2: Lin is the number of SSBs not available at the UE during TEvaluate_in_SSB,CCA, where Lin ≤ Lin,max. NOTE 3: Lin,max=7 for Max(TDRX,TSSB) ≤ 40 assuming TDRX=0 for non-DRX case, Lin,max=5 for 40<Max(TDRX,TSSB)≤320, Lin,max=3 for TDRX>320. NOTE 4: Lout is the number of SSBs not available at the UE during TEvaluate_out_SSB,CCA, where Lout ≤ Lout,max. NOTE 5: Lout,max=14 for Max(TDRX,TSSB) ≤ 40 assuming TDRX=0 for non-DRX case, Lout,max=10 for 40<Max(TDRX,TSSB)≤320, Lout,max=6 for TDRX>320. |  |  |

#### 8.1A.2.3	Measurement Restrictions for SSB based RLM

The UE is required to be capable of measuring SSB for RLM without measurement gaps. The UE is required to perform the SSB measurements with measurement restrictions as described in the following clauses.

For FR1, when the SSB for RLM is in the same OFDM symbol as CSI-RS for RLM, BFD, CBD or L1-RSRP measurement,

-	If SSB and CSI-RS have same SCS, UE shall be able to measure the SSB for RLM without any restriction;

-	If SSB and CSI-RS have different SCS,

-	If UE supports simultaneousRxDataSSB-DiffNumerology, UE shall be able to measure the SSB for RLM without any restriction;

-	If UE does not support simultaneousRxDataSSB-DiffNumerology, UE is required to measure one of but not both SSB for RLM and CSI-RS. Longer measurement period for SSB based RLM is expected, and no requirements are defined.

### 8.1A.3	Requirements for CSI-RS Based Radio Link Monitoring

#### 8.1A.3.1	Introduction

The requirements in this section apply for each CSI-RS based RLM-RS resource configured for PCell or PSCell, provided that the CSI-RS configured for RLM are actually configured to be transmitted within UE active DL BWP during the entire evaluation period specified in clause 8.1A.3.2 but occasionally may be not transmitted due to CCA operation. UE is not expected to perform radio link monitoring measurements on the CSI-RS configured as RLM-RS if the CSI-RS is not in the active TCI state of any CORESET configured in the UE active BWP.

Table 8.1A.3.1-1: PDCCH transmission parameters for out-of-sync evaluation

| Attribute | Value for BLER Configuration #0 |
| --- | --- |
| DCI format | TBD |
| Number of control OFDM symbols | TBD |
| Aggregation level (CCE) | TBD |
| Ratio of hypothetical PDCCH RE energy to average CSI-RS RE energy | TBD |
| Ratio of hypothetical PDCCH DMRS energy to average CSI-RS RE energy | TBD |
| Bandwidth (PRBs) | TBD |
| Sub-carrier spacing (kHz) | SCS of the active DL BWP |
| DMRS precoder granularity | TBD |
| REG bundle size | TBD |
| CP length | Normal |
| Mapping from REG to CCE | TBD |

Table 8.1A.3.1-2: PDCCH transmission parameters for in-sync evaluation

| Attribute | Value for BLER Configuration #0 |
| --- | --- |
| DCI payload size | TBD |
| Number of control OFDM symbols | TBD |
| Aggregation level (CCE) | TBD |
| Ratio of hypothetical PDCCH RE energy to average CSI-RS RE energy | TBD |
| Ratio of hypothetical PDCCH DMRS energy to average CSI-RS RE energy | TBD |
| Bandwidth (PRBs) | TBD |
| Sub-carrier spacing (kHz) | SCS of the active DL BWP |
| DMRS precoder granularity | TBD |
| REG bundle size | TBD |
| CP length | Normal |
| Mapping from REG to CCE | TBD |

#### 8.1A.3.2	Minimum requirement

Editor’s note: TBD

#### 8.1A.3.3	Measurement Restrictions for CSI-RS Based RLM

Editor’s note: TBD

### 8.1A.4	Minimum Requirement at Transitions

Editor’s note: TBD

### 8.1A.5	Minimum Requirement for UE Turning OFF the Transmitter

The transmitter power of the UE in the monitored cell shall be turned off within 40ms after expiry of T310 timer as specified in TS 38.331 [2]. The UE shall not perform CCA on any carrier frequency after the expiry of T310.

### 8.1A.6	Minimum Requirement for L1 Indication

Editor’s note: TBD

### 8.1A.7	Scheduling Availability of UE during Radio Link Monitoring

### 8.1A.7.1	Scheduling Availability of UE performing radio link monitoring with the same subcarrier spacing as for PDSCH/PDCCH on FR1

In this section, the same requirements apply as in Section 8.1.7.1.

### 8.1A.7.2	Scheduling Availability of UE performing radio link monitoring with a different subcarrier spacing than for PDSCH/PDCCH on FR1

In this section, the same requirements apply as in Section 8.1.7.2.