3GPP TSG-RAN WG4 Meeting #94-e 	R4-2001943

# Electronic Meeting, February 24-March 06, 2020

| CR-Form-v12.0 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.133 | CR | 0567 | rev | - | Current version: | 16.2.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | X | Radio Access Network | X | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Measurement report mapping for PRS RSTD |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Ericsson |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R4 |  |  |  |  |  |  |  |  |  |
| Work item code: | NR_POS-Core |  |  |  |  |  | Date: |  |  | 2020-02-14 |
| Category: | B |  |  |  |  |  | Release: |  |  | Rel-16 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) Rel-12 (Release 12) Rel-13 (Release 13) Rel-14 (Release 14) Rel-15 (Release 15) Rel-16 (Release 16) |  |
| Reason for change: |  | Measurement report mapping is missing for PRS RSTD |  |  |  |  |  |  |  |  |
| Summary of change: |  | Measurement report mapping is introduced for PRS RSTD |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Measurement report mapping is missing for PRS RSTD |  |  |  |  |  |  |  |  |
| Clauses affected: |  | New sections: 10.1.X1, 10.1.X2 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  |  | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  |  | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  |  | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

### 10.1.X1	DL RSTD measurement accuracy requirements

### 10.1.X2	DL RSTD measurement report mapping

To report an RSTD measurement, the UE shall report a reference quantity value based on Table 10.1.X2-1 and a relative quantity value  defined in Table 10.1.X2-2, so that the difference between the measured RSTD quantity and the lower bound of the corresponding range from Table 10.1.X2-1 is between  and + resolutionStep. The resolutionStep is 2k, where k shall be an applicable value satisfying kmin,≤k≤kmax and mod(RSTD,2k)==0, with

kmax=5,

kmin=4 for FR1 when RSTD_delta_i is reported together with a reference quantity from RSTD_2260 to RSTD_10451,

kmin=5 for FR1 when RSTD_delta_i is reported together with a reference quantity from RSTD_0000 to RSTD_2259 or from RSTD_10452 to RSTD_12711,

kmin=0 for FR2 when RSTD_delta_i is reported together with a reference quantity from RSTD_2260 to RSTD_10451,

kmin=4 for FR2 when RSTD_delta_i is reported together with a reference quantity from RSTD_0000 to RSTD_2259 or from RSTD_10452 to RSTD_12711.

If the value of k is configured by LMF but it is not an applicable value for the current RSTD measurement to be reported, the UE determines the resolutionStep for reporting the RSTD measurement based on the applicable k-value which is closest to the configured k, otherwise the UE uses the configured k to determine resolutionStep.

FFS when the value of k is not configured.

The mapping between the measured quantity and the reported reference quantity values is defined in Table 10.1.X2-1. The reporting range of the reference RSTD is defined from -15391Ts to 15391Ts with 1Ts resolution for absolute value of RSTD less or equal to 4096Ts and 5Ts for absolute value of RSTD greater than 4096Ts, where Ts is defined in TS 38.211 [6].

Table 10.1.X2-1: Reference quantity report mapping

| Reported Reference Quantity Value | Measured Quantity Value | Unit |
| --- | --- | --- |
| RSTD_0000 | -15391 > RSTD | Ts |
| RSTD_0001 | -15391 RSTD < -15386 | Ts |
| … |  | … |
| RSTD_2258 | -4106 RSTD < -4101 | Ts |
| RSTD_2259 | -4101 RSTD < -4096 | Ts |
| RSTD_2260 | -4096 RSTD < -4095 | Ts |
| RSTD_2261 | -4095 RSTD < -4094 | Ts |
|  |  | … |
| RSTD_6353 | -3 RSTD -2 | Ts |
| RSTD_6354 | -2 RSTD -1 | Ts |
| RSTD_6355 | -1 RSTD 0 | Ts |
| RSTD_6356 | 0 < RSTD 1 | Ts |
| RSTD_6357 | 1 < RSTD 2 | Ts |
| RSTD_6358 | 2 < RSTD 3 | Ts |
| … | … | … |
| RSTD_10450 | 4094 < RSTD 4095 | Ts |
| RSTD_10451 | 4095 < RSTD 4096 | Ts |
| RSTD_10452 | 4096 < RSTD 4101 | Ts |
| RSTD_10453 | 4101 < RSTD 4106 | Ts |
| … | … | … |
| RSTD_12709 | 15381 < RSTD 15386 | Ts |
| RSTD_12710 | 15386 < RSTD 15391 | Ts |
| RSTD_12711 | 15391 < RSTD | Ts |

The mapping between the measured relative quantity and the repoted relative quantity values is defined in Table 10.1.X2-2.

Table 10.1.X2-2: Relative quantity report mapping

| Reported Relative Quantity Value | Measured Relative Quantity Value, | Unit |
| --- | --- | --- |
| RSTD_delta_0 | 0 | Tc |
| RSTD_delta_1 | 1 | Tc |
| RSTD_delta_2 | 2 | Tc |
| … | … | … |
| RSTD_delta_63 | 63 | Tc |
| RSTD_delta_64 | 64 | Tc |
| RSTD_delta_65 | 80 | Tc |
| RSTD_delta_66 | 96 | Tc |
| RSTD_delta_67 | 112 | Tc |
| RSTD_delta_68 | 128 | Tc |
| RSTD_delta_69 | 144 | Tc |
| RSTD_delta_70 | 160 | Tc |
| RSTD_delta_71 | 176 | Tc |
| RSTD_delta_72 | 192 | Tc |
| RSTD_delta_73 | 208 | Tc |
| RSTD_delta_74 | 224 | Tc |
| RSTD_delta_75 | 240 | Tc |
| RSTD_delta_76 | 256 | Tc |
| RSTD_delta_77 | 272 | Tc |
| RSTD_delta_78 | 288 | Tc |
| RSTD_delta_79 | 304 | Tc |