3GPP RAN WG4 Meeting #94-e                                      R4-2001954

Online, 24th February – 6th March 2020

Agenda item:	8.5.5.2

Source: 	Huawei, HiSilicon

Title: 	Discussion on MT timing for IAB

Document for:	Discussion

- Introduction

In last meeting RAN4 93, the RRM requirements about IAB MT are discussed. In this paper, we present the analysis about the RRM requirements of MT timing.

- Discussion

In the last RAN4 meeting, it is agreed to introduce MT timing in terms of TA adjustment and transmit timing (Tp and Tq). The existing transmit timing requirement and TA adjustment are defined in TS 38.133 7.1 and 7.3. Based on our understanding, the MT timing related requirements as mentioned above can reuse the existing requirement defined in TS 38.133.

Proposal 1: The MT timing related requirements in terms of TA adjustment and transmit timing (Tp and Tq) can reuse the current requirements defined in TS 38.133.

We also present a parallel TDoc [3] to introduce the Timing requirements for TS 38.174. As we have mentioned in the email discussion, we prefer to write the spec in a completely self-contained manners. It may introduce longer sections when writing the spec at beginning, but it will be easy for updating and this is the main advantage to have an IAB specific spec.

Proposal 2: Write the TS38.174 in a completely self-contained manners.

In the email discussion about the spec structure of IAB, one issue raised that whether to introduce the timer accuracy requirement for IAB. From our view, there is no need to define timer requirement for IAB. Firstly, some company support that the timer may be used in other test cases. However, it could be observed that there is no corresponding test for timer in TS 38.133. Secondly, the accuracy of timer could be shown in the other test where the timer is used, so there no need to introduce extra requirement and test for IAB node.

Observation 1: There is no corresponding test for timer accuracy in TS 38.133.

Proposal 3: Not to define timer accuracy requirements for IAB.

- Conclusions

Proposal 1: The MT timing related requirements in terms of TA adjustment and transmit timing (Tp and Tq) can reuse the current requirements defined in TS 38.133.

Proposal 2: Write the TS38.174 in a completely self-contained manners.

Observation 1: There is no corresponding test for timer accuracy in TS 38.133.

Proposal 3: Not to define the timer accuracy requirements for IAB.

References

[1] R1-1903809 “LS on initial access SSB periodicity assumption for IAB MT”

[2] TS 38.133

[3] R4-2001955  TP to TS 38.174 MT Timing