3GPP TSG-RAN WG4 Meeting #94-e	R4-2001976

Online, 24th February – 6th March 2020

Source:	Ericsson, US Cellular, MediaTek

Title:	TP for TR 37.716-21-11: Including band combinations 5-66_n71

Agenda item:			9.4.2

Document for:	Approval

# 1. Introduction

A text proposal for TR 37.716-21-11 to add DC_5A-66A_n71A.

# 2. Text Proposal

---Start of changes---

## 5.1.X	DC_5-66_n71

### 5.1.X.1	Operating bands for EN-DC

Table 5.1.X.1-1: Band combinations EN-DC (three bands)

| EN-DC band | E-UTRA CA band | NR band | Single UL allowed |
| --- | --- | --- | --- |
| 5-66_n71 | CA_5-66 | n71 | No |

### 5.1.X.2	Configuration for DC

Table 5.1.X.2-1: Inter-band EN-DC configurations (three bands)

| EN-DC configuration | Uplink EN-DC configuration (NOTE 1) | E-UTRA CA configuration | NR band |
| --- | --- | --- | --- |
| DC_5A-66A_n71A | DC_5A_n71A DC_66A_n71A | CA_5A-66A | n71 |

### 5.1.X.3		∆TIB and ∆RIB values

For DC_5-66_n71, the TIB,c and RIB,c values for the 5_n71 part are reused from CA_4-5-13, and also checked with DC_5_n71 and DC_71_n5, and the values for 66 are checked from DC_5-7_n71 and DC_66_n71, and they are given in the tables below.

Table 5.1.X.3-1: ΔTIB,c

| Inter-band DC Configuration | E-UTRA and NR Band | ΔTIB,c [dB] |
| --- | --- | --- |
| DC_5-66_n71 | 5 | 0.5 |
|  | 66 | 0.3 |
|  | n71 | 0.5 |

Table 5.1.X.3-2: ΔRIB

| Inter-band DC Configuration | E-UTRA and NR Band | ΔRIB [dB] |
| --- | --- | --- |
| DC_5-66_n71 | 5 | 0 |
|  | 66 | 0 |
|  | n71 | 0 |

### 5.1.x.4	REFSENS requirements

Co-existence analysis from DC_5_n71 shows that there is IMD3 impact from DC_5_n71 UL to Band 66 DL. MSD value is 16.1 dB for 20_n3 UL affecting band n78 DL in DC_20_n3-n78. But test bench simulations have shown values closer to 10 dB. Therefor an MSD level of 13 dB is proposed for IMD3 in band 66 in table below

Co-existence analysis from DC_66_n71 shows that there is IMD5 impact from DC_66_n71 UL to Band 5 DL. MSD value is 3.3 dB when 1_n28 UL affecting band 8 DL in DC_1-8_n28. But since n71 TX frequency is lower than B28 that PA may have higher gain at lower frequency both wanted Tx and IMD spur level. Therefor an MSD level of 4.2 dB is proposed for IMD5 in band 5 in table below.

Table 7.3B.2.3.5.2-1: MSD test points for Scell due to dual uplink operation for EN-DC in NR FR1 (three bands)

| NR or E-UTRA Band / Channel bandwidth / NRB / MSD |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| EN-DC Configuration | EUTRA / NR band | UL Fc (MHz) | UL/DL BW (MHz) | UL LCRB | DL Fc (MHz) | MSD (dB) | IMD order |
| DC_5A-66A_n71A | 5 | 830 | 5 | 25 | 875 | N/A | N/A |
|  | 66 | 1761 | 5 | 25 | 2161 | 13 | IMD3 |
|  | n71 | 665.5 | 5 | 25 | 619.5 | N/A | N/A |
|  | 5 | 846.5 | 5 | 25 | 891.5 | 4.2 | IMD5 |
|  | 66 | 1770 | 5 | 25 | 2170 | N/A | N/A |
|  | n71 | 665.5 | 5 | 25 | 619.5 | N/A | N/A |

---End of changes---

# Reference

[1]		RP-192815, “Dual Connectivity (EN-DC) of 2 bands LTE inter-band CA (2DL/1UL) and 1 NR band (1DL/1UL)” Huawei, HiSilicon