3GPP TSG-RAN WG4 Meeting #94-e	R4-2001979

Online, 24th February – 6th March 2020

Source:	Ericsson, US Cellular

Title:	TP for TR 37.716-21-11: Including band combinations 48-66_n12

Agenda item:			9.4.2

Document for:	Approval

# 1. Introduction

A text proposal for TR 37.716-21-11 to add DC_48A-66A_n12A.

# 2. Text Proposal

---Start of changes---

## 5.1.X	DC_48-66_n12

### 5.1.X.1	Operating bands for EN-DC

Table 5.1.X.1-1: Band combinations EN-DC (three bands)

| EN-DC band | E-UTRA CA band | NR band | Single UL allowed |
| --- | --- | --- | --- |
| 48-66_n12 | CA_48-66 | n12 | No |

### 5.1.X.2	Configuration for DC

Table 5.1.X.2-1: Inter-band EN-DC configurations (three bands)

| EN-DC configuration | Uplink EN-DC configuration (NOTE 1) | E-UTRA CA configuration | NR band |
| --- | --- | --- | --- |
| DC_48A-66A_n12A | DC_48A_n12A DC_66A_n12A | CA_48A-66A | n12 |

### 5.1.X.3		∆TIB and ∆RIB values

For DC_48-66_n12, the TIB,c and RIB,c values are reused from DC_48-66_n5, CA_13-48-66 and CA_5-48-66, and are given in the tables below.

Table 5.1.X.3-1: ΔTIB,c

| Inter-band DC Configuration | E-UTRA and NR Band | ΔTIB,c [dB] |
| --- | --- | --- |
| DC_48-66_n12 | 48 | 0.8 |
|  | 66 | 0.6 |
|  | n12 | 0.3 |

Table 5.1.X.3-2: ΔRIB

| Inter-band DC Configuration | E-UTRA and NR Band | ΔRIB [dB] |
| --- | --- | --- |
| DC_48-66_n12 | 48 | 0.5 |
|  | 66 | 0.2 |
|  | n12 | 0 |

### 5.1.x.4	REFSENS requirements

Co-existence analysis from DC_48_n12 shows that there is IMD3 impact from DC_48_n12 UL to Band 66 DL.

Co-existence analysis from DC_66_n12 shows that there is IMD5 impact from DC_66_n12 UL to Band 48 DL.

MSD value for band 66 DL affected by DC_48_n12 UL are derived from 13-48 UL affecting band 66 DL in CA_13-48-66 and need to be defined in TS 38.101-3 like in table below:

Table 7.3B.2.3.5.2-1: MSD test points for Scell due to dual uplink operation for EN-DC in NR FR1 (three bands)

| NR or E-UTRA Band / Channel bandwidth / NRB / MSD |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| EN-DC Configuration | EUTRA / NR band | UL Fc (MHz) | UL/DL BW (MHz) | UL LCRB | DL Fc (MHz) | MSD (dB) | IMD order |
| DC_48A-66A_n12A | 48 | 3580 | 5 | 25 | 3580 | N/A | N/A |
|  | 66 | 1760 | 5 | 25 | 2160 | 17.1 | IMD3 |
|  | n12 | 710 | 5 | 25 | 740 | N/A | N/A |

---End of changes---

# Reference

[1]		RP-192815, “Dual Connectivity (EN-DC) of 2 bands LTE inter-band CA (2DL/1UL) and 1 NR band (1DL/1UL)” Huawei, HiSilicon