3GPP TSG-RAN WG4 Meeting #94-e	R4-2001980

Online, 24th February – 6th March 2020

Source:	Ericsson, US Cellular, MediaTek

Title:	TP for TR 37.716-21-11: Including band combinations 48-66_n71

Agenda item:			9.4.2

Document for:	Approval

# 1. Introduction

A text proposal for TR 37.716-21-11 to add DC_48A-66A_n71A.

# 2. Text Proposal

---Start of changes---

## 5.1.X	DC_48-66_n71

### 5.1.X.1	Operating bands for EN-DC

Table 5.1.X.1-1: Band combinations EN-DC (three bands)

| EN-DC band | E-UTRA CA band | NR band | Single UL allowed |
| --- | --- | --- | --- |
| 48-66_n71 | CA_48-66 | n71 | No |

### 5.1.X.2	Configuration for DC

Table 5.1.X.2-1: Inter-band EN-DC configurations (three bands)

| EN-DC configuration | Uplink EN-DC configuration (NOTE 1) | E-UTRA CA configuration | NR band |
| --- | --- | --- | --- |
| DC_48A-66A_n71A | DC_48A_n71A DC_66A_n71A | CA_48A-66A | n71 |

### 5.1.X.3		∆TIB and ∆RIB values

For DC_48-66_n71, the TIB,c and RIB,c values for the 48_n71 part are reused from CA_13-48-66 and CA_5-48-66 and are given in the tables below.

Table 5.1.X.3-1: ΔTIB,c

| Inter-band DC Configuration | E-UTRA and NR Band | ΔTIB,c [dB] |
| --- | --- | --- |
| DC_48-66_n71 | 48 | 0.8 |
|  | 66 | 0.6 |
|  | n71 | 0.3 |

Table 5.1.X.3-2: ΔRIB

| Inter-band DC Configuration | E-UTRA and NR Band | ΔRIB [dB] |
| --- | --- | --- |
| DC_48-66_n71 | 48 | 0.5 |
|  | 66 | 0.2 |
|  | n71 | 0 |

### 5.1.x.4	REFSENS requirements

Co-existence analysis from DC_48_n71 shows that there is IMD3 impact from DC_48_n71 UL to Band 66 DL. MSD value for band 66 DL affected by DC_48_n71 UL are derived from 28_n77 UL affecting band 1 DL in DC_1-28_n77 and need to be defined in TS 38.101-3 like in table below.

Co-existence analysis from DC_66_n71 shows that there is IMD4 impact from DC_66_n71 UL to Band 48 DL. MSD value for band 48 DL affected by DC_66_n71 UL are derived from 19-21 UL affecting band 42 DL in CA_19-21-42 and need to be defined in TS 38.101-3 like in table below.

Table 7.3B.2.3.5.2-1: MSD test points for Scell due to dual uplink operation for EN-DC in NR FR1 (three bands)

| NR or E-UTRA Band / Channel bandwidth / NRB / MSD |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| EN-DC Configuration | EUTRA / NR band | UL Fc (MHz) | UL/DL BW (MHz) | UL LCRB | DL Fc (MHz) | MSD (dB) | IMD order |
| DC_48A-66A_n71A | 48 | 3560 | 5 | 25 | 3560 | N/A | N/A |
|  | 66 | 1774 | 5 | 25 | 2174 | 15.8 | IMD3 |
|  | n71 | 693 | 5 | 25 | 647 | N/A | N/A |
|  | 48 | 3697.5 | 5 | 25 | 3697.5 | 13.0 | IMD4 |
|  | 66 | 1712.5 | 5 | 25 | 2112.5 | N/A | N/A |
|  | n71 | 665.5 | 5 | 25 | 619.5 | N/A | N/A |

---End of changes---

# Reference

[1]		RP-192815, “Dual Connectivity (EN-DC) of 2 bands LTE inter-band CA (2DL/1UL) and 1 NR band (1DL/1UL)” Huawei, HiSilicon